3GPP TSG-RAN WG4 Meeting #76	    R4-154448

Beijing, China, 24 – 28 Aug, 2015

Agenda Item:	9.4.2

Source: 	Samsung

Title: 	Cell search simulation results under AWGN channel with large frequency offset between cells

Document for:	Discussion

Introduction

In RAN4 #75 meeting, simulation assumptions for cell search in the existing scenarios for speeds up to 350km/h are proposed in [1]. Its main difference from existing assumption is: the propagation condition is AWGN with 1750Hz offset between desired and interfering cells.

In this contribution, we provide the simulation results of 90th percentile acquisition time of cell search delay based on PSS and SSS according to the agreed assumptions.

Smulation Results

The detailed simulation assumptions are listed in Table A.1~4 in Annex A.

Following Table 1 shows the simulation results of 90th percentile acquisition time of cell search delay under AWGN channel and AWGN with 1750Hz offset between desired and interfering cells.

The simulation results of 90th percentile acquisition time are also illustrated in Figure 1 ~ 4.

Table 1: 90th percentile acquisition time of cell search delay (ms)

| Channel Case | AWGN |  |  | AWGN+1750Hz |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | SINR= -4dB | SINR= -6dB | SINR= -8dB | SINR= -4dB | SINR= -6dB | SINR= -8dB |
| 1 | 10 | 40 | 190 | 20 | 60 | 420 |
| 2 | 30 | 40 | 120 | 60 | 100 | 270 |
| 3 | 10 | 20 | 40 | 20 | 30 | 40 |
| 4 | 10 | 40 | 250 | 20 | 60 | 530 |
| 5 | 40 | 70 | 180 | 30 | 70 | 170 |
| 6 | 40 | 80 | 190 | 30 | 70 | 160 |
| 7 | 20 | 30 | 50 | 20 | 30 | 40 |
| 8 | 40 | 80 | 190 | 40 | 70 | 170 |

Figure 1: 90th percentile cell detection time (AWGN, Synchronous)

Figure 2: 90th percentile cell detection time (AWGN, Asynchronous)

Figure 3: 90th percentile cell detection time (AWGN+1750Hz, Synchronous)

Figure 4: 90th percentile cell detection time (AWGN+1750Hz, Asynchronous)

Based on the above simulation results, it can be observed:

- Comparing the different channel condition, AWGN with 1750Hz offset between desired and interfering cells shows much worse performance for synchronous cases. However, the legacy cell search delay requirement, i.e. 600 ms, can still be satisfied for these cases with the side condition SINR = -6dB (cell 3 Ior/Noc = 1.25dB).

- For asynchronous cases, pure AWGN and AWGN with 1750Hz offset show almost same performance.

The above observations are summarized as:

Observation 1: High speed UE up to 350km/h can meet the legacy cell search requirements, i.e. 600ms.

Conclusion

In this contribution, we provide the simulation results of cell search performance for high speed UE up to 350km/h. Based on the simulation results, one observation is made.

Observation 1: High speed UE up to 350km/h can meet the legacy cell search requirements, i.e. 600ms.

Reference

- R4-153853, TP for TR 36.878: Cell search simulation assumptions, Ericsson, Huawei, HiSilicon

Annex A: Simulation Assumptions

Table A.1: Link Simulation Parameters for cell identification

| Parameter | Unit | Cell 1 | Cell 2 | Cell 3 |
| --- | --- | --- | --- | --- |
| E-UTRA RF Channel number | - | Channel 1 | Channel 1 | Channel 1 |
| Data and Control PSD relative to RS PSD | dB | 0 | 0 | 0 |
| P-SCH and S-SCH PSD relative to RS PSD | dB | 0 | 0 | 0 |
| Number of RB’s |  | 6 | 6 | 6 |
| RB Utilization | % | 100 | 100 | 100 |
| Data Modulation | - | QPSK | QPSK | QPSK |
| Frame Structure Type | - | 1 | 1 | 1 |
| CP Length | - | Normal | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | 0 | 0 | 0 |
| 1) Relative Delay of 1st Path (synchronous) | μs | 0 | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | μs | 0 | 1.5 ms | 3.0 ms |
| Ior/Noc | dB | 5.18 | 0.29 | Test 1: 1.25 Test 2: 0.25 Test 3: -0.75 |
| Number of Tx antennas | - | 1 | 1 | 1 |
| P-SCH Sequence ID | - | See Table 3, 4 | See Table 3, 4 | See Table 3, 4 |
| S-SCH Sequence ID [2] | - | See Table 3, 4 | See Table 3, 4 | See Table 3, 4 |
| Propagation Condition | - | AWGN with 0Hz offset between desired and interfering cells (for alignment), AWGN with 1750Hz offset between desired and interfering cells |  |  |
| Ioc Model | - | AWGN |  |  |

Table A.2: Other simulation assumption parameters for cell identification

| Simulation parameters | Comments/values |
| --- | --- |
| Prior knowledge of Cell 1 and Cell 2 by the UE | Yes |
| Cell 1, 2, 3 carrier frequency | Same |
| False detect threshold | Required as in a real UE implementation |
| UE having apriori knowledge of system being synchronous or synchronous (by signaling) | No |
| Duty cycle | 100% (to represent non-DRX case) |
| Performance criterion for comparison | 90th percentile acquisition time for “correct” cell detection of both P-SCH and S-SCH sequence id’s. |
| CP length detection | Both short or long CP may be present, hence UE needs to detect CP length |
| Receive antennas | 2 (uncorrelated) |

Table A.3: Cell ID Combinations to be simulated

| case # | Cell 3 (Desired Cell) |  | Cell 1 (Interferer 1) |  | Cell 2 (Interferer 2) |  | Scenario |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 2 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 3 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 4 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Synchronous |
| 5 | psc3 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 6 | psc1 | ssc3a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 7 | psc1 | ssc1a, ssc3b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |
| 8 | psc3 | ssc1a, ssc1b | psc1 | ssc1a, ssc1b | psc2 | ssc2a, ssc2b | Asynchronous |

Table A.4: PSC, SSC indices for simulations

| Label | Code index |
| --- | --- |
| psc1 | 29 |
| psc2 | 25 |
| psc3 | 34 |

| Label | Code index | Cell group index |
| --- | --- | --- |
| (ssc1a, ssc1b) | (6, 8) | 36 |
| (ssc2a, ssc2b) | (10, 12) | 40 |
| (ssc3a, ssc3b) | (7, 9) | 37 |
| (ssc1a, ssc3b) | (6, 9) | 65 |