3GPP TSG RAN WG4 Meeting #76                                      	      R4-155080

Beijing, China

August 24-28, 2015

Agenda Item:	4.2.1.2

Source:	Motorola Mobility

Title:	A-MPR for Band 42 with CA_NS_08 and Contiguous Resource Allocations

Document for:  Discussion

# Introduction

In this contribution, simulations are used to evaluate the A-MPR needed to meet the CA_NS_08 emissions co-existence requirements for Band 42 with contiguous resource allocations.  The simulations were performed using a 2 GHz PA model.  It should be noted that concerns were expressed in [1] over the use of these models at 3.5 GHz, as it was observed that the A-MPR could be underestimated for some RB allocations.

# Simulation Assumptions

The modulator impairments used in the simulations were the nominal impairments which apply above 1 GHz:

- Modulator IQ – image = -25 dB

- Modulator carrier leakage = -25 dBc

- Modulator C_IM3 = -60 dBc

Initially, the operating point of the PA was set such that the UTRAACLR1 requirement of 33 dB is just met with a 100 RB QPSK allocation, and the output power at which the requirement was just met was defined to be 22 dBm.

The emissions requirements used were

- E-UTRAACRL = 30dB

- UTRAACLR1 = 33dB

- UTRAACLR2 = 36dB

Also, the general E-UTRA CA spectrum emissions mask for Bandwidth Class C was applied as specified in TS 36.101 Table 6.6.2.1A-1.

When CA_NS_08 is signalled, the following additional requirements apply.

Table 1: CA_NS_08 Additional requirement.

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
| 3400 ≤ f ≤ 3800 | -23 (Note 1, Note 3) | 5 MHz |
|  | -40 (Note 2) | 1 MHz |
| Note 1: This requirement applies within an offset between 5 MHz and 25 MHz from the lower and from the upper edge of the channel bandwidth. Note 2: This requirement applies from 3400 MHz up to 25 MHz below the lower E-UTRA channel edge and from 25MHz above the upper E-UTRA channel edge up to 3800 MHz. Note 3: This emission limit can imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |

Simulations were performed for the following CA combinations:

- 39.8 MHz (100 RB’s + 100 RB’s)

- 34.85 MHz (75 RB’s + 100 RB’s)

- 29.9 MHz (50 RB’s + 100 RB’s)

- 24.95 MHz (25 RB’s + 100 RB’s).

All simulations were performed using 16QAM modulation.  The simulations results can be found in Figures 1-4 below

The simulation results in this contribution can be compared with the simulation results and measurements in [2-5].

Figure 1:  A-MPR required to meet to meet CA_NS_08 emissions requirements for an aggregated bandwidth of 39.8 MHz (100 RB’s + 100 RB’s).

Figure 2:  A-MPR required to meet to meet CA_NS_08 emissions requirements for an aggregated bandwidth of 34.85 MHz (75 RB’s + 100 RB’s).

Figure 3:  A-MPR required to meet to meet CA_NS_08 emissions requirements for an aggregated bandwidth of 29.9 MHz (50 RB’s + 100 RB’s).

Figure 4:  A-MPR required to meet to meet CA_NS_08 emissions requirements for an aggregated bandwidth of 24.95 MHz (25 RB’s + 100 RB’s).

# Summary

In this contribution, simulations are used to evaluate the A-MPR needed to meet the CA_NS_08 emissions co-existence requirements for Band 42 with contiguous resource allocations.  The simulation results in this contribution can be compared with the simulation results and measurements in [2-5].

# References

- R4-73AH-0088, On concerns about validity of B42&43 PA model in A-MPR simulations, Nokia, RAN4 #73 Ad Hoc.

- R4-150135, A-MPR for Band 42 CA and Band 43 UE-UE coexistence, Intel RAN4#74.

- R4-150224, Band 42/43 CA_NS_08 A-MPR, Qualcomm, RAN4 #74.

- R4-150639, B42 (43) 2CC A-MPR, Nokia, RAN4#74.

- R4-153801, B42C CA_NS_08 A-MPR, Qualcomm, RAN4#75.