3GPP TSG-RAN WG4 Meeting #78	R4-160010

St. Julian’s, Malta, 15 - 19 Feb, 2016

Source:	China Telecom

Title:	Summary of initial link results for BS IRC in asynchronous network

Agenda Item:	6.6.1

Document for:	Information

- Introduction

The contribution summarizes the BS IRC link results for asynchronous network based on the companies’ inputs at RAN4 #77 and RAN4 #78 meetings [1]-[10].

- Summary of initial link results

Table 0: List of simulation cases

| Case Num | PRB allocation/ Band width | MCS | Propagation condition (Serving, interferers) | Antenna configuration for serving and interferers | Scenario | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | (DIP1-1, DIP1-2) dB | (DIP1, DIP2) dB | (DIP1-1, DIP1-2) dB |
| 1 | 50 PRB/ 10MHz | 6 | (EVA70, ETU70) | 1x2 Low | HomNet | (-1.69, -0.50) | (-1.11, N/A) | (-1.11, -1.11) |
| 2 | 50 PRB/ 10MHz | 6 | (EPA5, ETU5) | 1x2 Low | HetNet | (-0.85, -0.12) | (-0.43, N/A) | (-0.43, -0.43) |
| 3 | 50 PRB/ 10MHz | 15 | (EVA70, ETU70) | 1x4 Low | HomNet | (-1.69, -0.50) | (-1.11, -10.91) | (-1.11, -1.11) |
| 4 | 50 PRB/ 10MHz | 15 | (EPA5, ETU5) | 1x4 Low | HetNet | (-0.85, -0.12) | (-0.43, -13.78) | (-0.43, -0.43) |
| 5 | 50 PRB/ 10MHz | 20 | (EVA70, ETU70) | 1x8 Low | HomNet | (-1.69, -0.50) | (-1.11, -10.91) | (-1.11, -1.11) |
| 6 | 50 PRB/ 10MHz | 20 | (EPA5, ETU5) | 1x8 Low | HetNet | (-0.85, -0.12) | (-0.43, -13.78) | (-0.43, -0.43) |

Table 1a: Link simulation results for option 1

| Case Num | MMSE async, SNR @ 70% TP (dB) |  |  |  |  | IRC async, SNR @ 70% TP (dB) |  |  |  |  |  | IRC sync, SNR @ 70% TP (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | SS | ZTE | CTC | E/// | HW | NN, ALU | ZTE |
| 1 | 6.78 | 5.31 | 5.59 | 5.90 | ~5.5 | 3.99 | 3.97 | 4.20 | 4.30 | 3.67 | ~3 | 2.81 | 2.37 | 2.52 | 2.70 | ~1 |
| 2 | >10 | 9.51 | 9.79 | 10.70 | ~7 | 6.73 | 6.10 | 6.22 | 9.30 | 6.62 | ~5 | 4.93 | 3.42 | 3.57 | 5.70 | ~2.5 |
| 3 | >10 | 9.25 | 9.53 | 9.50 | ~8.7 | 7.11 | 6.72 | 6.79 | 7.00 | 6.41 | ~6 | 5.77 | 5.26 | 5.06 | 5.00 | ~4.5 |
| 4 | >10 | 13.50 | 13.78 | 14.20 | >10 | 7.73 | 7.13 | 7.25 | 11.70 | 7.67 | ~7.2 | 5.80 | 4.56 | 4.83 | 6.40 | ~4.5 |
| 5 | >10 | 10.00 | 10.28 | 10.50 | ~9.2 | 6.99 | 7.32 | 7.35 | 7.70 | 6.50 | ~6.2 | 6.12 | 6.02 | 5.63 | 5.50 | ~5 |
| 6 | >10 | 14.60 | 14.88 | 15.70 |  | 7.12 | 6.55 | 6.59 | 11.60 | 7.60 |  | 5.53 | 4.82 | 4.85 | 6.20 |  |

Table 1b: Link simulation results analysis for option 1

| Case Num | SNR gap between IRC async and MMSE async |  |  |  |  | SNR gap between IRC async and IRC sync |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | ZTE |
| 1 | 2.79 | 1.34 | 1.39 | 1.60 | ~2.5 | 1.18 | 1.60 | 1.68 | 1.60 | ~2 |
| 2 | >3.27 | 3.41 | 3.57 | 1.40 | ~2 | 1.80 | 2.68 | 2.65 | 3.60 | ~2.5 |
| 3 | >2.89 | 2.53 | 2.74 | 2.50 | ~2.7 | 1.34 | 1.46 | 1.73 | 2.00 | ~1.5 |
| 4 | >2.27 | 6.37 | 6.53 | 2.50 | >2.8 | 1.93 | 2.57 | 2.42 | 5.30 | ~2.7 |
| 5 | >3.01 | 2.68 | 2.93 | 2.80 | ~3 | 0.87 | 1.30 | 1.72 | 2.20 | ~1.2 |
| 6 | >2.88 | 8.05 | 8.29 | 4.10 |  | 1.59 | 1.73 | 1.74 | 5.40 |  |

Table 2a: Link simulation results for option 2

| Case Num | MMSE async, SINR @ 70% TP (dB) |  |  |  |  | IRC async, SINR @ 70% TP (dB) |  |  |  |  |  | IRC sync, SINR @ 70% TP (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | SS | ZTE | CTC | E/// | HW | NN, ALU | ZTE |
| 1 | -1.38 | -1.76 | -1.78 | -1.90 |  | -4.28 | -3.85 | -4.23 | -4.00 | -4.34 |  | -4.53 | -3.94 | -4.51 | -4.20 |  |
| 2 | -2.11 | -2.39 | -1.36 | -2.20 | ~-3 | -7.34 | -6.87 | -5.99 | -6.10 | -6.64 | ~-7.6 | -7.48 | -7.29 | -6.26 | -6.50 | ~-7.8 |
| 3 | >1.6 | 2.03 | 2.63 | 1.80 |  | -1.39 | -1.25 | -1.71 | -1.90 | -1.55 |  | -1.51 | -1.38 | -2.01 | -2.20 |  |
| 4 |  | 1.01 | 2.18 | 1.20 | ~1 | -5.92 | -5.71 | -5.43 | -5.00 | -5.43 | ~-5.9 | -6.24 | -6.32 | -5.72 | -5.60 | ~-6.1 |
| 5 | >1.6 | 2.57 | 3.79 | 2.80 |  | -1.42 | -0.86 | -1.32 | -2.10 | -1.46 |  | -1.54 | -1.14 | -1.43 | -2.40 |  |
| 6 |  | 1.47 | 3.47 | 2.20 | ~1.8 | -6.17 | -6.20 | -5.76 | -5.50 | -5.16 | ~-6.8 | -6.52 | -6.76 | -5.88 | -6.20 | ~-7 |

Table 2b: Link simulation results analysis for option 2

| Case Num | SINR gap between IRC async and MMSE async |  |  |  |  | SINR gap between IRC async and IRC sync |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | ZTE |
| 1 | 2.89 | 2.09 | 2.45 | 2.10 |  | 0.25 | 0.09 | 0.28 | 0.20 |  |
| 2 | 5.23 | 4.48 | 4.63 | 3.90 | ~4.6 | 0.14 | 0.42 | 0.27 | 0.40 | ~0.2 |
| 3 | >2.99 | 3.28 | 4.34 | 3.70 |  | 0.12 | 0.13 | 0.30 | 0.30 |  |
| 4 |  | 6.72 | 7.61 | 6.20 | ~6.9 | 0.33 | 0.61 | 0.29 | 0.60 | ~0.2 |
| 5 | >3.02 | 3.43 | 5.11 | 4.90 |  | 0.11 | 0.28 | 0.11 | 0.30 |  |
| 6 |  | 7.67 | 9.23 | 7.70 | ~8.6 | 0.35 | 0.56 | 0.12 | 0.70 | ~0.2 |

Table 3a: Link simulation results for option 3

| Case Num | MMSE async, SINR @ 70% TP (dB) |  |  |  |  | IRC async, SINR @ 70% TP (dB) |  |  |  |  | IRC sync, SINR @ 70% TP (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | ZTE |
| 1 | -1.40 | -1.73 | -0.97 | 0.90 | ~-2.3 | -3.00 | -2.64 | -2.46 | -0.60 | ~-3.5 | -4.57 | -3.93 | -3.95 | -2.30 | ~-5 |
| 2 | -2.10 | -2.63 | -0.58 | 3.20 | ~-2.8 | -4.73 | -4.48 | -4.16 | 1.90 | ~-4.8 | -7.30 | -7.29 | -6.70 | -1.70 | ~-7.8 |
| 3 | 2.50 | 2.12 | 3.00 | 4.60 | >2 | 0.34 | 0.59 | 0.31 | 2.20 | ~-0.7 | -1.07 | -0.88 | -1.14 | 0.00 | ~-1.7 |
| 4 |  | 0.84 | 3.37 | 6.70 | ~1.7 | -3.32 | -3.36 | -3.06 | 4.20 | ~-3.1 | -5.34 | -5.59 | -5.56 | -1.10 | ~-5.5 |
| 5 | 3.01 | 2.70 | 3.77 | 5.60 | >2 | 0.36 | 1.54 | 0.81 | 2.80 | ~-0.7 | -0.63 | -0.02 | -0.49 | 0.60 | ~-1.4 |
| 6 |  | 1.45 | 4.48 | 8.20 |  | -3.78 | -3.85 | -3.83 | 4.00 | ~-3.7 | -4.97 | -5.32 | -5.54 | -1.30 | ~-5.2 |

Table 3b: Link simulation results analysis for option 3

| Case Num | SINR gap between IRC async and MMSE async |  |  |  |  | SINR gap between IRC async and IRC sync |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CTC | E/// | HW | NN, ALU | ZTE | CTC | E/// | HW | NN, ALU | ZTE |
| 1 | 1.61 | 0.91 | 1.49 | 1.50 | ~1.2 | 1.57 | 1.29 | 1.49 | 1.70 | ~1.5 |
| 2 | 2.63 | 1.85 | 3.58 | 1.30 | ~2 | 2.58 | 2.81 | 2.54 | 3.60 | ~3 |
| 3 | 2.16 | 1.53 | 2.69 | 2.40 | >2.7 | 1.41 | 1.47 | 1.45 | 2.20 | ~1 |
| 4 |  | 4.20 | 6.43 | 2.50 | ~4.8 | 2.02 | 2.23 | 2.50 | 5.30 | ~2.4 |
| 5 | 2.65 | 1.16 | 2.96 | 2.80 | >2.7 | 0.98 | 1.56 | 1.30 | 2.20 | ~0.7 |
| 6 |  | 5.30 | 8.31 | 4.20 |  | 1.18 | 1.47 | 1.71 | 5.30 | ~1.5 |

Observations from companies’ results:

- With option 2, IRC performance in asynchronous scenario is similar or slightly poorer compared to that in synchronous scenario.

- With option 1 and 3, IRC performance in asynchronous scenario is worse than that in synchronous scenario, and in asynchronous scenario IRC receiver can achieve obvious performance gain compared to MMSE receiver.

  - In such cases, different fast-fading channel seeds are used in two continuous TTIs of the interference, and the IRC receiver in asynchronous scenario needs to suppress the interference from two different spatial directions.

  - The separation in performance between the sync IRC and the async IRC cases are similar for option 1 and option 3.

- References

R4-157442, Link level analysis for BS IRC in asynchronous network, China Telecom, RAN4 #77, Nov 2015.

R4-157579, Asynchronous IRC simulation results, Ericsson, RAN4 #77, Nov 2015.

R4-160432, Asynchronous IRC simulation results, Ericsson, RAN4 #78, Feb 2016.

R4-157685, Evaluation of BS IRC performance under the asynchronous network, Huawei, RAN4 #77, Nov 2015.

R4-160759, BS-IRC demodulation performance requirements under asynchronous network, Huawei, HiSilicon, RAN4 #78, Feb 2016.

R4-157930, Discussion on UL IRC async cases, Nokia Networks, RAN4 #77, Nov 2015.

R4-161064, Discussion on BS-IRC performance under asynchronous networks, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN4 #78, Feb 2016.

R4-160122, Simulation results of asynchronous network for UL MMSE-IRC Evaluation, Samsung, RAN4 #78, Feb 2016.

R4-157496, Further discussion on the performance evaluation for the asynchronous network for BS IRC receiver, ZTE, RAN4 #77, Nov 2015.

R4-160990, Discussion on performance requirements for asynchronous network for BS IRC receiver, ZTE, RAN4 #78, Feb 2016.

Annex A: Detailed descriptions on the interference models

This section describes three options for asynchronous network interference model.

- Option 1: Modelling of time-varying interference in terms of interference power and fast fading

  - Configure two ON/OFF interfering signals (UEs) to model the interference from one dominant interfering cell, i.e., the dominant interfering cell schedule UE 1-1 in the even TTIs and schedule UE 1-2 in the odd TTIs. The interference power of UE 1-1 and UE 1-2 are different, and different channel seeds are used for the desired UE and interfering UEs.

  - As baseline, the transmission of the interference signal is delayed with respect to the desired signal by 0.33 ms.

Figure A-1: Modeling of time-varying interference: One explicit interfering cell

- Option 2: The only difference compare to the synchronous simulation setup is to model certain timing offsets

  - Model two simultaneous interfering UEs, and the transmissions from the first/second dominant interfering UE is delayed with respect to the desired UE by 0.33/0.67 ms.

Note: model only one interfering UE for 2Rx cases.

Figure A-2: Two simultaneous interfering UEs

- Option 3: Modeling of time-varying interference in terms of fast fading

  - The only difference compared to Option 1 is that the interference power of UE 1-1 and UE 1-2 are the same.