3GPP TSG-RAN WG4 RAN4 #78bis                                      R4-161556

San Jose del Cabo, Mexico, 11 – 15 April, 2016

Agenda item:			6.6.4

Source:			Samsung

Title:			Discussion on RI test for 4RX capable UE

Document for:			Discussion

1	Introduction

In RAN4#78, there were some discussion on RI test for 4RX UE [1] but no agreements were reached due to lack of enough online/offline discussion. In this contribution, we provide our view and analysis on test case design for RI test for 4RX capable UEs.

- RI Reporting Test

For legacy test setup for RI reporting test, the original test is design to make sure the reported rank indicator accurately represents the channel rank, while up to RI=2 is supported for legacy tests. As up to 4 layers can be supported by 4RX UEs and the SNR point (or range) for rank 1/2 switching will be lower than legacy 2RX UEs, we expect the new test case newly introduced can reflect both factors.

Firstly, the following test setup by modifying the one in 9.5.1.1 [2] into a 4RX version (4x4 channel is used) has been utilized to evaluate the performance for the scenario with fixed rank operation and the scheme following reported RI.

2.1 RI Tests in Low Correlation Condition

For 4x4 low spatial correlation environment, we provide the throughput performance over the whole SNR range in low spatial correlation environments in the Figure 1:

Figure 1: Throughput performance comparison for RI test with low correlation

Firstly, we consider the original two tests (Test 1 and Test 2 in 9.5.1.1) with the legacy requirement. In addition to the legacy requirement of γ1 and γ2, we consider the option of the requirement of γ3, i.e.,

- The ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 3 shall be ≥γ3.

The following Figure 2 provides our simulation result for three kinds of throughput ratio:

Figure 2: Throughput ratio comparison for RI test with low correlation

Specifically, the numerical results for test points at SNR=0dB and 20dB, and another new SNR point, i.e., SNR=-4dB, are provided in the following table:

Table 1: Throughput ratio at different SNR points for RI test with low correlation

| Tput Ratio | SNR=0dB (Test-1) | SNR=20dB (Test-2) | SNR=-4dB |
| --- | --- | --- | --- |
| Follow_RI/Fixed_Rank=1 | 1.115 | 2.189 | 0.998 |
| Follow_RI/Fixed_Rank=2 | 1.007 | 1.276 | 1.119 |
| Follow_RI/Fixed_Rank=3 | 1.088 | 0.985 | 2.068 |

It can be observed from the above figure and table that 4RX UE’s SNR range for rank 2 operation is lower than 2RX UE, which is also the phenomena observed in Qualcomm’s contribution [1]. Because of this, the original Test 1 (i.e., the ratio of the throughput at SNR=0dB obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 2 shall be no less than ≥γ2=1) is hard to pass, because the ratio is 1.007 based on our simulation results. Obviously, we have the following two options to solve this problem:

- Option-1 (suggested in [1]): lower the SNR test point for Test 1 to make sure γ2=1 can be satisfied. Based on our simulation, we found that -4dB is enough to make sure the test case can be passed (i.e., the ratio is 1.119).

- Option-2: still keep the SNR test point at 0dB, but use the requirement of γ3=1 instead, i.e., the ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 3 shall be ≥γ3=1. Based on our simulation, we found that the ratio is observed as 1.088 at SNR=0dB.

And the above two options are summarized in the following proposal:

Proposal 1: For RI Test 1, two methods can be utilized to modify the original test for 4RX UE:

- Option-1 (suggested in [1]): lower the SNR test point for Test 1 to make sure γ2=1 can be satisfied.

- Option-2: still keep the SNR test point at 0dB, but use the requirement of γ3=1 instead, i.e., the ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 3 shall be ≥γ3=1.

Obviously, Option-1 can be used to test RI reporting switching between rank=1 and rank=2, while Option-2 can be used to test RI reporting switching between rank=2 and rank=3, therefore both options are acceptable based on our analysis.

For Test 2 at SNR = 20dB, the original requirement of γ1=1.05 can be easily satisfied since Rank 3 is more likely to be reported, i.e., based on our simulation the ratio observed is 2.189. Considering the high rank switching behaviors should be tested, we can consider using the requirement of γ2 instead. Based on our observation, the RI reporting is switching to rank 3 from around SNR = 12dB, which means the original SNR working point 20dB is still usable to test the switching behavior. Based on the above observation, we have the following proposal:

Proposal 2: For RI Test 2, the original test can be modified to use the requirement γ2=1.05 for 4RX UE, while keeping the SNR test point as 20dB.

2.2 RI Tests in High Correlation Condition

For 4x4 high spatial correlation environment used in RI Test 3, we also provide the throughput performance over the whole SNR range in the Figure 3. The corresponding throughput ratios are illustrated in Figure 4.

Figure 3: Throughput performance comparison for RI test with high correlation

Figure 4: Throughput ratio comparison for RI test with high correlation

Based on our simulation results, we can observed that the throughput ratio at test point SNR = 20dB, as provided in the following table,

Table 2: Throughput ratio at different SNR points for RI test with high correlation

| Tput Ratio | SNR=20dB (Test-3) |
| --- | --- |
| Follow_RI/Fixed_Rank=1 | 1.021 |
| Follow_RI/Fixed_Rank=2 | 1.025 |

It can be observed from the above figure and table that fixed rank 1 and rank 2 transmission will achieve similar performance at SNR = 20dB, while fixed rank 3 transmission will have large performance degradation due to high spatial correlation environment.

Observation 1: In high correlation environment, fixed rank 1 and rank 2 transmission for 4RX UE will achieve similar performance at SNR = 20dB, while fixed rank 3 transmission will have large performance degradation.

From RI test perspective, 4RX UE can still pass the original Test 3, i.e., the ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 1 shall be ≥γ1=0.9. Therefore, we can still reuse the original test case to ensure UE’s RI reporting behaviour in high correlation environment. Based on our observation above, another obvious option is to use the requirement of γ2=0.9, and  4RX UE can also pass this test case. Since limited difference observed for fixed rank 1 and rank 2 transmission, both options can be accepted.

Proposal 3: For RI Test 3, two methods can be utilized to modify the original test for 4RX UE:

- Option-1: reuse the original requirement of γ1=0.9.

- Option-2: use the requirement of γ2=0.9 instead.

Both options can guarantee correct UE implementation of not choosing high layer operation (3 or 4 layers) in high correlation environment.

- Conclusion

In this contribution, we presented our view and analysis on test case design for RI test for 4RX capable UEs. Our observation and proposals can be summarized as:

Proposal 1: For RI Test 1, two methods can be utilized to modify the original test for 4RX UE:

- Option-1 (suggested in [1]): lower the SNR test point for Test 1 to make sure γ2=1 can be satisfied.

- Option-2: still keep the SNR test point at 0dB, but use the requirement of γ3=1 instead, i.e., the ratio of the throughput obtained when transmitting based on UE reported RI and that obtained when transmitting with fixed rank 3 shall be ≥γ3=1.

Proposal 2: For RI Test 2, the original test can be modified to use the requirement γ2=1.05 for 4RX UE, while keeping the SNR test point as 20dB.

Observation 1: In high correlation environment, fixed rank 1 and rank 2 transmission for 4RX UE will achieve similar performance at SNR = 20dB, while fixed rank 3 transmission will have large performance degradation.

Proposal 3: For RI Test 3, two methods can be utilized to modify the original test for 4RX UE:

- Option-1: reuse the original requirement of γ1=0.9.

- Option-2: use the requirement of γ2=0.9 instead.

Both options can guarantee correct UE implementation of not choosing high layer operation (3 or 4 layers) in high correlation environment.

- Reference

- R4-160037, “Further discussion on 4 Rx CSI requirements”, Qualcomm.

- 3GPP, TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 13)”.