3GPP TSG-RAN WG4 Meeting #78Bis 	R4-161569

San Jose Del Cabo, Mexico, 11 – 15 April 2016

Source:	DISH Network

Title:	Removal of bands from TS36.101 for future releases

Work item:	TEI12

Agenda item:	4.2.1

Document for:	Agreement

## Introduction

This is a discussion document proposing a common methodology to remove some of the requirements for operating bands which are not likely be implemented in future releases

## Background

Currently there are many bands in the UE specifications which fall into the following categories

- Bands which have been deployed and are now superseded

- Bands which never have been deployed and are superseded

- Bands which have been deployed and are planned to be superseded

For example a) the specification states” B6 is no longer applicable” as B19 is now a super set and therefore B6 is no longer implemented or deployed in the future.

TS36.101Table 5.5-1 E-U TRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive / UE transmit |  |  | Downlink (DL) operating band BS transmit / UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 61 | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| NOTE 1: Band 6 is not applicable |  |  |  |  |  |  |  |

In most cases, maintaining these bands in the specification is not a significant problem as the requirements are not applicable unless the superseded band is implemented. However, maintaining these superseded bands in the specifications can lead to the following issues:

- In many cases the requirement for the superseded bands are not updated and there requirements are not consistent as new bands are added in later releases. Taking the example of the UE to UE co-existence requirements in TS36.101 Table 6.6.3.2-1 for B6 compared to B19. Ideally in this case since B6 is defined as not applicable the list of protected bands should be removed similar to B35, B36 and B37 which are shown as blank

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 19 | E-UTRA Band 1, 11, 21, 28, 34, 42, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1839.9 | - | 1879.9 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  |  |  |  |  |  |

- In many cases the requirement for the superseded bands do not impact other bands and can be retained i.e. channel bandwidths/spacing/URFCN, UE power class / A-MPR, occupied bandwidth, out of band emissions, RFSENS, Out of band blocking, etc. For example

TS36.101 Table 6.2.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) |  | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 |  |  |  |  |  | 23 | ±2 |  |  |
| 19 |  |  |  |  |  | 23 | ±2 |  |  |

TS36.101 Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 6 |  |  | -100 | -97 |  |  | FDD |
| 19 |  |  | -100 | -97 | -95.2 |  | FDD |

TS36.101 Table 7.6.2.1-2: Out of band blocking

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 | Range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42 (NOTE 2), 43 (NOTE 2), 44, 45, 65, 66 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17 | FInterferer | MHz | - | - | - | FUL_low - FUL_high |
| NOTE 1: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 3: For the UE that supports both Band 4 and Band 66, the out-of-blocking frequency range for Band 4 is defined relative to FDL_low and FDL_high of Band 66. |  |  |  |  |  |  |

## Proposal

Therefore it is suggested we adopt a common methodology to improve the specification clarity and also remove some of these ambiguous requirements for bands which are superseded

Proposal 1 	Note 1 in the TS36.101 Table 5.5-1 (E-U TRA operating bands)   is expanded to include other bands which have been not been implemented or superseded (i.e. NOTE 1: Band 6, 23, 35, 36, 37 are not applicable)

Proposal 2	Requirements for superseded bands which do not impact other bands are retained (i.e. channel bandwidths/spacing/URFCN, UE power class / A-MPR, occupied bandwidth, out of band emissions, blocking, and etc. This will also reduce the amount of changes to the R4 and R5 specifications)

Proposal 3	UE co-existence requirements for;

- Superseded bands are removed (similar to B1 which does not need to protect B6 but only B19)

- Superseded band to protect other bands are removed (similar to B35, 36 and B37)