3GPP TSG-RAN WG4 Meeting#78 bis    						     							  R4-162217

# San Jose del Cabo, Mexico,  11-15 April 2016

Title:	Core requirments overview for Uplink Capacity Enhancements for LTE

Source:	CMCC

Agenda Item:	7.18.1

Document for:	Approval

- Introduction

In RAN#71, a new Work Item of uplink capacity enhancements for LTE was approved [1]. The work item will specify 256QAM support in UL for LTE. The support of 256QAM shall be specified in a generic manner for both licensed and unlicensed spectrum. Focus of the work should be specific to the support of small cell deployments or deployments with eNB with many receive antennas. The objectives are:

- Introduce new MCS table and signalling to support 256QAM in UL [RAN1]

- Define applicable RRC signalling, UE capability and potential new UE categories [RAN2, RAN1]

- Introduce RF requirements for single carrier and for carrier aggregation on both licensed and unlicensed spectrum [RAN4]

In addition the work item shall specify support UL PUSCH in special subframe. The objectives are:

- UL support of PUSCH transmission in special subframe [RAN1]

  - Specify mechanism for supporting PUSCH transmission in special subframe with DwPTS of 6 OFDM symbols, GP of 2 OFDM symbols. [RAN1, RAN2]

  - Introduce applicable RF requirements [RAN4]

  - Backward compatibility with legacy UEs is maintained

In this paper, we overview RAN4 work for core requirements.

- Discussion

  - Support of UL 256QAM

For supporting of UL 256QAM, following core requirements should be defined.

EVM: The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. A new row for 256QAM is required for Table 6.5.2.1.1-1 in TS 36.101. Link level simulation is needed to observe the impact on performance loss due to EVM requirements

MPR: Current MPR requirement in TS 36.101 is not applicable to 256QAM. Four kinds of MPR requirement listed in table 2.1-1 should be evaluated by simulation. Table format MPR requirements should be updated of adding new rows for 256QAM. Formula format MPR requirements should be checked whether it could be reused or updated. Following parameter assumptions used for 64QAM evaluation could be reused for 256QAM simulation [2].

-	PA operating point: UTRAACLR1 = 33 dBc @ Pout = 22 dBm for 100RB QPSK signal

-	Modulator IQ imbalance = 25 dBc

-	Modulator carrier leakage = 25 dBc

-	Modulator C_IM3 = 60 dBc

-	Phase noise=[33] dBc

-	Transceiver noise= [-29.5] dBc

Table 2.1-1 MPR requirements

| Case | Back off | Single carrier or CA | RB allocation | Format in current specification |
| --- | --- | --- | --- | --- |
| 1 | MPR | Single carrier | contiguous | table |
| 2 | MPR | Single carrier | non-contiguous | formula |
| 3 | MPR | Intra-band contiguous CA | contiguous | table |
| 4 | MPR | Intra-band contiguous CA | non-contiguous | formula |

A-MPR: there are three kinds of A-MPR requirements in the specification as in table 2.1-2. However, A-MPR requirements are band/band combination specific, it is almost not possible to update each requirement one by one. Therefore, it is proposed to follow the same method of UL 64QAM. For single carrier, total back-off is the sum of MPR and A-MPR, and A-MPR requirements for QPSK/16QAM/64QAM can be reused if MPR for 256QAM requirements are appropriately defined. For intra-band CA, total back off is the maximum value of MPR and A-MPR, and also A-MPR requirements for QPSK/16QAM/64QAM can be reused if intra-band MPR for 256QAM requirements are appropriately defined.

Table 2.1-2 MPR requirements

| Case | Back off | Single carrier or CA | RB allocation | Format in current specification |
| --- | --- | --- | --- | --- |
| 1 | MPR+A-MPR | Single carrier | contiguous | table |
| 2 | A-MPR | Intra-band contiguous CA | contiguous | table |
| 3 | A-MPR | Intra-band contiguous CA | non-contiguous | formula |

RMC: UL reference measurement channels in Annex A of TS 36.101 should be updated in for 256QAM base on RAN1’s output.

Observation#1: EVM, MPR, A-MPR and RMC requirements should be defined for UL 256QAM

Proposal#1: Reuse following assumptions for 256QAM MPR evaluation

PA operating point: UTRAACLR1 = 33 dBc @ Pout = 22 dBm for 100RB QPSK signal

-	Modulator IQ imbalance = 25 dBc

-	Modulator carrier leakage = 25 dBc

-	Modulator C_IM3 = 60 dBc

-	Phase noise=[33] dBc

-	Transceiver noise= [-29.5] dBc

Proposal#2: For UL 256QAM A-MPR requirement, follow the same method as for UL 64QAM

  - Support UL PUSCH in special subframe

- For supporting UL PUSCH in UpPTS, the only impact on RF core requirement may be ON/OFF time mask requirements in section 6.3.4 of TS 36.101. Companies have to check whether new requirement of ON/OFF time mask is needed.

Proposal #3: whether new requirements of ON/OFF time mask for UL PUSCH in UpPTS need to be checked.

- Conclusion

Observation#1: EVM, MPR, A-MPR and RMC requirements should be defined for UL 256QAM

Proposal#1: Reuse following assumptions for 256QAM MPR evaluation

PA operating point: UTRAACLR1 = 33 dBc @ Pout = 22 dBm for 100RB QPSK signal

-	Modulator IQ imbalance = 25 dBc

-	Modulator carrier leakage = 25 dBc

-	Modulator C_IM3 = 60 dBc

-	Phase noise=[33] dBc

-	Transceiver noise= [-29.5] dBc

Proposal#2: For UL 256QAM A-MPR requirement, follow the same method as for UL 64QAM

Proposal #3: whether new requirements of ON/OFF time mask for UL PUSCH in UpPTS need to be checked.

- Reference

[1] RP-160664, New Work Item on Uplink Capacity Enhancements for LTE, Ericsson, CMCC

[2] TR 36.883, Introducing Uplink (UL) 64QAM (64-state Quadrature Amplitude Modulation)