3GPP TSG RAN4 Meeting #78bis	R4-162201

San Jose del Cabo, Mexico, 11th – 15th Apr, 2016

Title: 	MSD for 3+41 2UL CA

Source: 	Huawei, HiSilicon

Agenda item:	7.6.1

Document for:	Discussion

# 1	Introduction

This contribution discusses MSD requirements for 3+41 UL CA.

# 2	Discussion

During the 1UL/2DL CA_3A_41A it was agreed to use a local diplexer as implementation architecture [1]. We use the same assumption in our respective UL CA analysis. Basically the architecture, found from [1] with added 2UL capabilities looks as below.

3+41 UL CA has IMD4 relation 2*B41 UL-2*B3UL=B3 DL. For instance the frequencies listed in table below result in IMD4 on top of B3 DL. The final UL frequencies are still to be agreed, but the IMD magnitude analysis can still be conducted.

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |
| CA_3A_41A | 3 | 1740 | 5 | 25 | 1835 | TBD | FDD-TDD |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A |  |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 |  |  |  |  |  |  |  |

We analyze the amount of IMD using the following component linearity assumptions:

Antenna switch IP4					56

Duplexer IP4							55

B3 forward mixing IP4				32

B41 forward mixing IP4				32

B3 reverse mixing IP4				33

B41 reverse mixing IP4				33

B3 LNA IIP4							-6

B41 LNA IIP4						-6

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation TX-RX				65

RFIC Pout B3/B41					0

B3 duplexer TX-RX isolation		50

B3 duplexer TX rejection at B41	31

B3 duplexer RX rejection at B41	42

B41 filter attenuation at B3 TX		26

B41 filter attenuation at B3 RX		22

Diplexer IP4							55

Diplexer isolation						15

Based on parameters above the IMD4 is 11.3dB excluding CF. The most dominating factors assuming these parameters are H-H diplexer, antenna switch, and L-H diplexer. However, diplexer and antenna switch linearity assumptions are fairly aged. We are reviewing these assumptions and will provide analysis and proposal based on reviewed assumptions in the next meeting.

# 3	Conclusion

MSD test points and initial IMD was provided. Concrete IMD proposal based on reviewed switch and diplexer linearity will be provided in the next meeting.

# 4	References

[1] R4-152493, "Way Forward on UE RF analysis for 3+41," China Unicom, CATR