3GPP TSG-RAN WG4 Meeting#78 bis    						     							  R4-162221

# San Jose del Cabo, Mexico,  11-15 April 2016

Title:	ACLR and ACS for standalone NB-IoT

Source:	CMCC

Agenda Item:	6.13.4

Document for:	Approval

- Introduction

In RAN4#78, standalone NB-IoT co-existence study was finished [1]. In this paper, we discuss how the BS ACLR and ACS requirement can be derived based on the co-existence conclusion.

- Discussion

  - ACLR

The BS ACLR results can be found in [1]

- GSM: BS with ACLR=40, outage degradation in [4.1 – 2.9] %

- UMTS: BS with ACLR=45, outage degradation in [3.27 – 0.34] %

  - Note: The ACLR value mentioned above corresponds to ACLR_3 or ACLR_4, not ACLR_1.

- LTE: BS With ACLR=45, loss in [12.3 – 0.74]%, BS with ACLR=50, loss in [6.7 – 0.38]%,

  - Note: The ACLR value mentioned above corresponds to ACLR_3 or ACLR_4, not ACLR_1.

Corresponding to the simulation assumption [2], NB-IoT ACLR and ACS are defined for its own bandwidth size. One problem is that NB-IoT ACLR_1 and ACLR_2 do not fall into transmission bandwidth of UMTS and LTE. However, if we define four ACLR requirements (from ACLR_1 to ACLR_4), it will be too redundant.  We believe ACLR_3 and ACLR_4 outperform ACLR_1 and ACLR_2. Therefore, we suggest that ACLR_1 with 40dB and ACLR_2 with 50dB is adopted for NB-IoT.

Proposal#1: Apply 40dB for ACLR_1 and 50dB forACLR_2 for standalone NB-IoT BS.

  - ACS

The ACLR results can be found in [1]

- GSM: BS with ACS=45, loss in [2.8 – 0.4] dB

- UMTS: BS with ACS=40, loss in [0.7 - 0] dB

- LTE: BS with ACS=45, loss in [1.3 – 0.1] dB

In Section 7.5 of TS 36.104, both narrowband blocking and ACS requirements are defined for LTE. To guarantee the co-existence performance, we propose to apply 45dB for NB-IoT ACS requirement and use both 200kHz GSM and 5MHz LTE as interferer for the test.

Proposal#2: Apply 45dB for NB-IoT BS ACS requirement and use both 200kHz GSM and 5MHz LTE as interferer for the test.

For E-UTRA narrowband blocking test, the wanted signal mean power is PREFSENS + 6dB; and for E-UTRA ACS test, the wanted signal mean power depends on the channel bandwidth. However, if we look at the MSR specification in TS 37.014, the wanted signal mean power of narrowband blocking test for GSM/EDGE is PREFSENS + 3dB. In addition, based on the simulation conclusion, NB-IoT loss is smaller than 3dB with appropriate ACS.

Proposal#3: the want signal mean power for NB-IoT BS ACS test should be PREFSENS + 3dB.

- Conclusion

Proposal#1: Apply 40dB for ACLR_1 and 50dB forACLR_2 for standalone NB-IoT BS.

Proposal#2: Apply 45dB for NB-IoT BS ACS requirement and use both 200kHz GSM and 5MHz LTE as interferer for the test.

Proposal#3: the want signal mean power for NB-IoT BS ACS test should be PREFSENS + 3dB.

- Reference

[1] R4-161295, WF on Coexistence Standalone, RAN4#78

[2] R4-161153, Way forward on how to apply ACLR and ACS on coexistence study for standalone case, Intel, Ericsson, Huawei, Nokia, Qualcomm, ZTE