3GPP TSG-RAN WG4 Meeting #78bis	                                         R4-162251

San Jose del Cabo, Mexico, 11 - 15 April, 2016

Source: 	Huawei, Hisilicon

Title: 	TP for Rel-14 2UL/2DL TR 36.714-00-02: operating bands, channel bandwidths for CA_3DL_2A-4A-29A_2UL_2A-4A, CA_3DL_2A-4A-5A_2UL_2A-4A, CA_4DL_2A-4A-5A-29A_2UL_2A-4A

Agenda Item:	7.7.1

Document for:	Approval

# Background

This contribution provides a TP for TR 36.714-00-02 to capture the operating bands and channel bandwidths for CA_3DL_2A-4A-29A_2UL_2A-4A, CA_3DL_2A-4A-5A_2UL_2A-4A and CA_4DL_2A-4A-5A-29A_2UL_2A-4A.

# References

[1] RP-16xxxx, “New WID: Basket WI for LTE Advanced Inter-band CA Rel-14 for xDL/2UL”, RAN #71

# Text Proposal

<Start of Text Proposal for TR 36.714-00-02>

## 6.x	CA_3DL_2A-4A-29A_2UL_2A-4A_BCS0

### 6.x.1 Operating bands for CA

Table 6.x.1-1: Inter-band CA operating bands for CA_2-4-29

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-4-29 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported E-UTRA bandwidths per CA configuration for CA_2-4-29

|  | CA operating / Channel bandwidth |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration |  | Uplink CA Configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-4A-29A |  | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 29 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

## 6.x	CA_3DL_2A-4A-5A_2UL_2A-4A_BCS0

### 6.x.1 Operating bands for CA

Table 6.x.1-1: Inter-band CA operating bands for CA_2-4-5

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-4-5 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported E-UTRA bandwidths per CA configuration for CA_2-4-5

|  | CA operating / Channel bandwidth |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration |  | Uplink CA Configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-4A-5A |  | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 5 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

## 6.x	CA_4DL_2A-4A-5A-29A_2UL_2A-4A_BCS0

### 6.x.1 Operating bands for CA

Table 6.x.1-1: Inter-band CA operating bands for CA_2-4-5-29

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-4-5-29 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 4 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported E-UTRA bandwidths per CA configuration for CA_2-4-5-29

|  | CA operating / Channel bandwidth |  |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration |  | Uplink CA Configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_2A-4A-5A-29A |  | CA_2A-4A | 2 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  |  | 29 |  |  | Yes | Yes |  |  |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.6.1-1 of TS 36.101 when operating in single carrier mode.

<End of Text Proposal>