3GPP TSG-RAN WG4 Meeting #78bis	R4-162633

# San Jose del Cabo, Mexico, 11 – 15 April, 2016

Agenda item:	6.3.3

Source: 	Qualcomm Incorporated

Title: 	Simulation results for reference receivers for Control Channel Interference Mitigation

Document for:	Discussion

# Introduction

In work item RP-151107 [1], it was proposed to use advanced receiver architectures for control channel interference mitigation. In RAN4 #77, RAN4 arrived at a consensus on reference receivers for synchronous networks. Simulation assumptions for down-selection of test cases were discussed over email. In this contribution, we provide simulation results for various control channels with different reference receivers.

# Simulation results

We provide simulation results with the following reference receivers

- MMSE-MRC (baseline receiver for Rel12)

- MMSE-IRC

- MMSE-IRC + CRS-IC

based on the simulation assumptions that were agreed over email. Since multiple options for interference modelling were still under consideration, we have provided the set of assumptions under which we have simulated the performance of reference receivers. Table 1 provides common simulation parameters.

Table 1: Common simulation parameters

| Parameters | Value |
| --- | --- |
| System bandwidth | 10 MHz for the serving and interfering cells |
| Duplexing mode | FDD |
| Cyclic prefix | Normal |
| Number of interference cells | 2 interfering cells |
| Interference power profile | High INR: I1/Noc = 13.91 dB, I2/Noc = 3.34 dB |
| Time-frequency offset modelling for synchronous network | Time offset: Interference cell #1 – 2us, Interference cell #2 – 3us Frequency offset: Interference cell #1 – 200Hz, Interference cell #2 – 300Hz |
| Cell ID | Serving cell: 0 Colliding CRS: Interferer cell #1 - 6, Interferer cell #2 - 1 Non-Colliding CRS: Interferer cell #1 - 1, Interferer cell #2 – 6 |
| CRS ports | Port 0 and 1 |
| Antenna configuration | 2x2, Low correlation |
| Tx EVM | 6% |
| Unused Serving cell RE-s and PRB-s | OCNG |

## PDCCH/PCFICH simulation results

Table 2 provides simulation parameters for PDCCH/PCFICH. Table 3 provides a list of test cases (from which down-selection of final test cases need to be done) for which results for PDCCH/PCFICH performance are provided.

Table 2. Simulation parameters for PDCCH/PCFICH

| Parameters | Value |
| --- | --- |
| Serving cell PDCCH | DCI Format 2 (43 bits – FDD, 46 bits – TDD) |
| CFI | Case 1: CFIS = 1, CFII = 1 (E-LMMSE-IRC + CRS-IC and LMMSE-IRC + CRS-IC) Case 2: CFIS = 3, CFII = 1 (LMMSE-IRC + CRS-IC only) Case 3: CFIS = 3, CFII = 3 (LMMSE-IRC + CRS-IC only) |
| PHICH Ng | Ng = 1/6 (i.e. number of PHICH groups = 2 for 10 MHz BW) |
| PHICH duration | Normal |
| Interference model | PDCCH/PHICH interference signals are emulated using random QPSK-modulated symbols with the SFBC-based precoding: - 50% interference loading for PDCCH/PHICH. - Guaranteed 50% interference loading on a subframe basis (50% of all available REGs/CCEs are chosen to be active) - Per-REG transmission granularity is allowed for simulation alignment - Unequal power boosting with uniform distribution from the [-6,6] dB range (uniformly random in log domain) PCFICH interference is explicitly modelled PDCCH/PHICH interference signals power normalization - Option 1 (baseline): No additional power normalization applied. PDSCH interference model: - CFIS ≤ CFII: PDSCH is emulated via OCNG with 100% loading. - CFIS > CFII: Per-PRB partial level model with 50% loading is used. Reuse Rel-11 Type A receiver TM4 interference model parameters (RI, modulation, precoding) |
| CRS assistance information | Provided |

Table 3. PDCCH/PCFICH test cases

| Test number | Network Type | PDCCH AL | Propagation Condition | Antenna configuration | CRS pattern | CFI |
| --- | --- | --- | --- | --- | --- | --- |
| PDCCH #1a | Synchronous | 2 CCE | EPA5 for all cells | 2 x 2 Low | Colliding | CFIS = 1, CFII = 1 |
| PDCCH #1b | Synchronous | 2 CCE | EPA5 for all cells | 2 x 2 Low | Colliding | CFIS = 3, CFII = 1 |
| PDCCH #1c | Synchronous | 2 CCE | EPA5 for all cells | 2 x 2 Low | Colliding | CFIS = 3, CFII = 3 |
| PDCCH #2a | Synchronous | 4 CCE | EPA5 for all cells | 2 x 2 Low | Non-colliding | CFIS = 1, CFII = 1 |
| PDCCH #2b | Synchronous | 4 CCE | EPA5 for all cells | 2 x 2 Low | Non-colliding | CFIS = 3, CFII = 1 |
| PDCCH #2c | Synchronous | 4 CCE | EPA5 for all cells | 2 x 2 Low | Non-colliding | CFIS = 3, CFII = 3 |

Results for PDCCH/PCFICH are provided below.

PDCCH #1a: Serving cell PDCCH AL 2 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 1, CSII = 1 Colliding CRS.

PDCCH #2a: Serving cell PDCCH AL 4 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 1, CSII = 1 Non-colliding CRS

PDCCH #1b: Serving cell PDCCH AL 2 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 3, CSII = 1 Colliding CRS.

PDCCH #2b: Serving cell PDCCH AL 4 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 3, CSII = 1 Non-colliding CRS

PDCCH #1c: Serving cell PDCCH AL 2 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 3, CSII = 3 Colliding CRS.

PDCCH #2c: Serving cell PDCCH AL 4 Interference PDCCH Model 50% loading + Non-Uniform power, High INR CFIS = 3, CSII = 3 Non-colliding CRS

## EPDCCH simulation results

For EPDCCH, simulation parameters are defined in Table 4 and scenarios for which simulation results are provided are given in Table 5.

Table 4. EPDCCH simulation parameters

| Parameters | Value |
| --- | --- |
| Serving cell EPDCCH | FRC #1: AL 2, Localized EPDCCH FRC #2: AL 4, Distributed EPDCCH DCI Format 2C (44 bits – FDD, 47 bits – TDD) |
| Number of PDCCH symbols and EPDCCH start | Distributed EPDCCH: CFI=2, EPDCCH starting symbols is derived from CFI Localized EPDCCH: The EPDCCH starting symbol is 2. CFI = 1. EPDCCH starting symbol is RRC configured. Aligned control regions and EPDCCH starting symbols in the serving and interference cells |
| EPDCCH parameters | Number of EPDCCH Sets Configured = 1 Distributed EPDCCH set PRBs {3, 17, 31, 45} Localized EPDCCH set PRBs {0, 7, 14, 21, 28, 35, 42, 49} EPDCCH is transmitted in all subframes EPDCCH precoding model is in accordance to TS 36.101 B.4.4. and B.4.5 |
| Interference model | Model #1: No PDSCH interference Model #2: Full PDSCH interference model. Reuse TM9 interference model from Rel-11 Type A receiver requirements (TS 36.101 8.3.1.1A, B.5.4). Model #3: Rel-11 Type A receiver asynchronous interference model (TS 36.101 B.5.2, 8.2.1.2.4) |
| Performance metrics | Pm-dsg vs SINR |
| Time-frequency offset modelling for asynchronous networks | 1/3 and 2/3 subframes as timing offset for the 2 NCs No frequency offsets |

Table 5. EPDCCH test cases

| Test number | FRC | Interference model | Propagation Condition | Antenna configuration | CRS pattern | NW type |
| --- | --- | --- | --- | --- | --- | --- |
| EPDCCH #1a | FRC #1 | Model #1 | EPA5 for all cells | 2 x 2 Low | Non-colliding | Sync |
| EPDCCH #1b | FRC #2 | Model #1 | EPA5 for all cells | 2 x 2 Low | Non-colliding | Sync |
| EPDCCH #2 | FRC #2 | Model #3 | EVA70 for all cells | 2 x 2 Low | NA | Async |
| EPDCCH #3a | FRC #1 | Model #2 | EPA5 for all cells | 2 x 2 Low | Colliding | Sync |
| EPDCCH #3b | FRC #2 | Model #2 | EPA5 for all cells | 2 x 2 Low | Colliding | Sync |

Results for EPDCCH are provided below:

EPDCCH #1a: EPDCCH AL 2, Localized, No PDSCH interference, High INR, Synchronous NW, Non-Colliding CRS pattern..

EPDCCH #1b: EPDCCH AL 4, Distributed, No PDSCH interference, High INR, Synchronous NW, Non-Colliding CRS pattern

< simulation results for test cases EPDCCH 2, 3a and 3b will be added later>

< simulation results for test cases PHICH 1 and 2 will be added later>

# References

- RP-151107, “New WI proposal: Interference mitigation for downlink control channels of LTE”, Intel Corporation, ZTE, RAN #68, June 2015