3GPP TSG-RAN WG4 Meeting #78	R4-162642

# San Jose del Cabo, Mexico, 11 – 15 April, 2016

Source:	Nokia

Title:	BS demodulation performance of TU600

Agenda item:	7.12.4

Document for:	Discussion

### 1	Introduction

The uplink performance impact under the Rel-14 WI [1] of performance enhancement for high speed scenarios will be new BS performance requirements under ETU600, following the TR 36.878 [2].

Based on TR 36.878 [2], there are 3 modulation schemes for ETU600, as listed in [3]:

Table 1    Options for ETU600 conditions

| Options | Propagation channel | Modulations and coding rates | Rx number |
| --- | --- | --- | --- |
| 1 | ETU600 | QPSK-1/3 | 2 |
| 2 | ETU600 | 16QAM-1/2 | 4 |
| 3 | ETU600 | 16QAM-1/2 | 2 |

The selection of modulation schemes will be decided in the WI phase, according to [2].

In the last RAN4 meeting, a WF on simulation assumption is agreed in [4].  Based on the WF, simulation results are provided in this contribution.

### 2	PUSCH performance for high speed scenario

The agreed simulation assumptions can be listed in Table 2.

Table 2    Simulation assumptions for PUSCH with ETU600

| Parameters | Value |
| --- | --- |
| Propagation conditions | ETU600 |
| # of TX antennas | 1 |
| Antenna correlation | Low |
| Noise model | AWGN |
| Channel bandwidth (MHz) | 10 |
| Number of PRBs | Full PRB allocation depending on BW |
| Cyclic perfix | Normal |
| Maximum number of HARQ transmissions | 4 |
| RV sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| Uplink-downlink allocation for TDD | Configuration 1 (2:2) |
| Channel estimation | Practical channel estimation |
| Simulation length | >= 10000 subframes |

The simulation results for QPSK rate 1/3 can be shown in Figure 1.  The FRC is based on A3-5 in Table A.3-1 of 36.104, with 50 PRB allocation (10MHz), as indicated in the simulation assumptions.

Figure 1    Throughput performance under ETU600

At the 70% throughput, the SNR=-1.3dB, based on Fig.1

### 3	Conclusion

Simulation results of QPSK rate 1/3 are provided for ETU600 in this contribution.

# Reference

[1]	RP-152263	“New WI proposal: Performance enhancements for high speed scenario”, NTT DOCOMO, Huawei, HiSilicon, Dec 2015

[2]	3GPP TR 36.878	“Study on performance enhancements for high speed scenario in LTE”, Rel 13

[3]	R4-161066	“Simulation assumptions for BS demodulation performance of TU600”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN4 #78

[4]	R4-161255	“Way forward on high speed BS demodulation assumptions”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN4 #78