3GPP TSG-RAN WG4 Meeting #78bis	   					       R4-162765

San Jose Del Cabo, Mexico, 11 - 15 Apr, 2016

Source:	China Telecom

Title:	TP on demodulation performance requirements in synchronous network

Agenda Item:	6.4.1.1

Document for:	Approval

Introduction

This contribution provides a text proposal for TR 36.884 to introduce the demodulation performance requirements in synchronous network in sub-clause 8.3.

Text Proposal

< Start of change: to be inserted after clause 8.2 >

## 8.3	Demodulation performance requirements

The BS enhanced performance requirements for MMSE-IRC receiver are derived by taking into account the ideal simulation results in clause 8.2 and the additional implementation margin. Based on interested companies’ inputs in R4-161523, the enhanced performance requirements are given in Table 8.3-1, and these requirements shall be introduced into TS 36.104.

Table 8.3-1: Enhanced performance requirement for MMSE-IRC receiver, SINR (dB) at 70% relative TP

| Case Num | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | [-4.8] | [-5.0] | [-5.1] | [-5.4] | [-5.5] | [-5.7] |
| 2 | [-4.1] | [-4.4] | [-4.1] | [-4.2] | [-4.0] | [-4.5] |
| 3 | [-4.7] | [-4.6] | [-4.7] | [-4.5] | [-4.5] | [-4.6] |
| 4 | [-2.0] | [-2.3] | [-2.5] | [-2.7] | [-2.7] | [-3.0] |
| 5 | [-0.1] | [-0.1] | [0.1] | [-0.1] | [0.0] | [-0.4] |
| 6 | [0.0] | [-0.1] | [-0.5] | [-0.2] | [-0.3] | [-0.1] |

< End of change >