3GPP TSG-RAN WG4 Meeting #78bis	R4-162983

San Jose Del Cabo, Mexico, 11 – 15 April, 2016

Agenda Item:	7.11.1

Source: 	Nokia, Alcatel-Lucent Shanghai Bell, Huawei

Title: 	TP for TR 37.871: Generic approach for Multi-Band Base Station testing with three or more bands

Document for:	Approval

# 1.	Introduction

The Multi-Band (MB) Base Station (BS) testing with three or more bands work item was approved at RAN#70 [1]. One objective of the work item is to conclude whether a generic testing approach can be applied to all the identified band combinations or separate testing approaches are necessary. In this contribution, we discuss this aspect, and provide a text proposal to include the findings into the TR [2] of this work item.

# 2.	Discussion

It was agreed at RAN4#78 that the following band combinations are considered feasible for implementation of multi-band BS capable of operation in three or more bands:

1.	Bands (8+20+28) in Europe.

2.	Bands (1+3+7).

3.	Bands (25+4+7).

Only these band combinations will be considered in this work item [3].

Now considering these band combinations, the 3rd order IMD products caused in the BS by transmitting of Band 1, Band 3 and Band 7 downlink (DL) carriers have been studied and it has been shown that the 3rd order IMD products will not fall into the BS receive band of Band 1, 3 or 7 [4].

On the other band, it has been shown that the 3rd order IMD products caused in the BS by transmitting of Band 2, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7 [4]. Since Band 25 is a superseding-band of Band 2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of Band 25, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Hence one may argue that a special testing approach is required for MB BS supporting Bands (25+4+7), e.g. the test configuration would need to ensure that the 3rd order IMD products of the BS transmitter(s) under test will fall into its receiver of Band 7. However, this issue is not specific to this band combination and would also happen for single band (SB) BS operating in certain frequency range, e.g. in Band 2 and Band 3, although the issue may become bigger with the wider frequency range covered by multi-band BS, and currently there is no specific test approach defined in the RAN4 specifications for such SB BS. Therefore, one may also argue that no special testing approach is required for MB BS supporting Bands (25+4+7).

In view of the above discussion, we consider that a generic testing approach can be applied to the three identified band combinations, while separate testing approaches may still be considered if find necessary for testing of some requirements.

# 3.	Conclusion

In this contribution, we have discussed and concluded that a generic testing approach can be applied to the three identified band combinations, while separate testing approaches may still be considered if found necessary for testing of some requirements. It is proposed to include the text proposal below into TR 37.871.

# 4.	Text proposal

<Start of change>

## 6	Necessary changes to the test requirements

Considering the identified band combinations, the 3rd order IMD products caused in the BS by transmitting of Band 1, Band 3 and Band 7 downlink (DL) carriers have been studied and it has been shown that the 3rd order IMD products will not fall into the BS receive band of Band 1, 3 or 7.

On the other band, it has been shown that the 3rd order IMD products caused in the BS by transmitting of Band 2, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Since Band 25 is a superseding-band of Band 2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of Band 25, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Hence one may argue that a special testing approach is required for MB BS supporting Bands (25+4+7), e.g. the test configuration would need to ensure that the 3rd order IMD products of the BS transmitter(s) under test will fall into its receiver of Band 7. However, this issue is not specific to this band combination and would also happen for single band (SB) BS operating in certain frequency range, e.g. in Band 2 and Band 3, although the issue may become bigger with the wider frequency range covered by multi-band BS, and currently there is no specific test approach defined in the RAN4 specifications for such SB BS. Therefore, one may also argue that no special testing approach is required for MB BS supporting Bands (25+4+7).

In view of the above discussion, it is agreed that a generic testing approach can be applied to the three identified band combinations, while separate testing approaches may still be considered if found necessary for testing of some requirements.

<End of change>

# References

[1]	RP-152205, “New Work Item proposal: Multi-Band Base Station testing with three or more bands”, Alcatel-Lucent.

[2]	R4-161598, “TR 37.871 V0.1.0”, Nokia

[3]	R4-161377, “TP for TR 37.871: Band combinations for feasible implementation of multi-band BS capable of operation in three or more bands”, Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, Ericsson, Huawei.

[4]	3GPP TR 36.853-13 v2.0.0, “LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)”.