3GPP TSG-RAN WG4 Meeting #79 	R4-164180

# Nanjing, China, 23 – 27 May, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.869 | CR | 0001 | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of BS coexistence studies of CA_66C, CA_66B and CA_66A-66A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Verizon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_AWS_EXT-Core |  |  |  |  |  |  | Date: |  | 2016-05-13 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | BS coexistence studies of CA_66C, CA_66B and CA_66A-66A are missing in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | BS coexistence studies of CA_66C, CA_66B and CA_66A-66A are put into the TR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No BS coexistence studies of CA_66C, CA_66B and CA_66A-66A in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 8.3.3, new clause 8.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 8.3.3	BS coexistence studies

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 66 contiguous DL carriers can be calculated as shown in Table 8.3.3-1 below:

Table 8.3.3-1: Band 66 contiguous DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2110 | 2200 |
| 2nd harmonics frequency limits (MHz) | 4220 | 4400 |
| 3rd harmonics frequency limits (MHz) | 6330 | 6600 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -90 | 90 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 2020 | 2290 |
| 3rd order IMD products (with maximum channel bandwidth for 66C) | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 2070 | 2240 |
| 3rd order IMD products (with maximum channel bandwidth for 66B) | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 2090 | 2220 |

It can be seen from Table 8.3.3-1 that the 2nd and 3rd harmonics as well as the 2nd IMD products of BS transmitting contiguous carriers in Band 66 will not fall into the BS receive band of any operating band, while the 3rd IMD products may fall into the BS receive band of Band 34. Note that the calculation in Table 8.3.3-1 (except the last two rows) assumes the BS is transmitting with the whole 90 MHz DL frequency of Band 66. If the BS is only transmitting up to 40 MHz (20 + 20 MHz) or 20 MHz (5 + 15 MHz) contiguous carriers in Band 66 as stated in the WIDS, the 3rd IMD products will not fall into the BS receive band of Band 34 as shown in the last two rows in Table 8.3.3-1. Moreover, Band 34 is not intended for use in the same geographical area as Band 66. Therefore, the 2nd and 3rd harmonics and IMD products caused by BS supporting CA_66C or CA_66B will not fall into the BS receive band of any other frequency band operating in the same geographical area.

<Next changed clause>

### 8.4.3	BS coexistence studies

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 66 non-contiguous DL carriers can be calculated as shown in Table 8.4.3-1 below:

Table 8.4.3-1: Band 66 non-contiguous DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2110 | 2200 |
| 2nd harmonics frequency limits (MHz) | 4220 | 4400 |
| 3rd harmonics frequency limits (MHz) | 6330 | 6600 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -90 | 90 |
| 3rd order IMD products | (2*f-low –f-high) | (2*f-high –f-low) |
| IMD frequency limits (MHz) | 2020 | 2290 |

It can be seen from Table 8.4.3-1 that the 2nd and 3rd harmonics as well as the 2nd IMD products of BS transmitting non-contiguous carriers in Band 66 will not fall into the BS receive band of any operating band, while the 3rd IMD products of BS supporting non-contiguous carrier aggregation of Band 66 may fall into the BS receive band of Band 34. However, Band 34 is not intended for use in the same geographical area as Band 66. Therefore, the 2nd and 3rd harmonics and IMD products caused by BS supporting CA_66A-66A will not fall into the BS receive band of any other frequency band operating in the same geographical area.

<End of change>