3GPP TSG-RAN WG4 Meeting #79	R4-163298

Nanjing, China, 23-27 May 2016

Agenda item:			6.6.2

Source:	MediaTek Inc.

Title:	RLM for 4 Rx

Document for:		Discussion

# 1	Introduction

RLM requirements and testing for 4 Rx has been under discussion for a number of meetings in RAN4. In RAN4 #78bis, contribution R4-161716 [1] made proposals in relation to RLM testing for type 2 UEs. It was suggested for 4 Rx testing to lower SNR3 (during T3) and SNR4 (during T4) by X = 3dB compared to current settings in TS 36.133.

In this contribution, we wish to suggest an amendment to the value of X whilst still supporting the approach put forward in contribution R4-161716.

# 2	Discussion

In the course of RAN4 discussions regarding 4 Rx requirements for RLM, companies have submitted results for simulated Qout levels in SNR for different antenna configurations. Table 1 summaries these contributions. [2-7].

| Channel Model | Antenna Config. | Nokia Networks [2] | Huawei [3] | Ericsson [4] | ZTE [5] | Intel [6] | CMCC [7] | MTK | Median |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AWGN | 1x2 | -11.6 | -12.8 | -13.1 | -13.2 |  | -8.4 |  | -12.8 |
|  | 1x4 | -13.8 | -14.9 | -15.4 | -15 | -12 | -11.2 | -15.3 | -14.9 |
|  | 2x2 | -11.6 | -12.4 | -13.1 | -13.2 |  | -11.4 |  | -12.4 |
|  | 2x4 | -13.9 | -15.2 | -15.4 | -15.1 | -14.6 | -13.8 | -15.4 | -15.1 |
| ETU70 | 1x2 | -10 | -9.8 | -9.6 | -10.1 |  | -7 |  | -9.8 |
|  | 1x4 | -12.8 | -12.8 | -12.8 | -12.6 | -10.5 | -9.4 | -12.7 | -12.7 |
|  | 2x2 | -10.8 | -10 | -10.6 | -10.7 |  | -9.2 |  | -10.6 |
|  | 2x4 | -13.2 | -12.9 | -13.5 | -13.1 | -13.2 | -12.4 | -13.2 | -13.2 |

Table 1: Simulated Qout in SNR (RAN4 Contributions in [2-7])

We can compare the median Qout from Table 1 with the current RLM tests in TS 36.133. These tests contain values SNR2 and SNR3 intended to bound Qout; the mid-point of this range may thus be seen as an indication of the Qout anticipated by the test. The mid-points for 2RX are captured as A in Table 2.

| Case | Qout Range (TS 36.133) | Qout Range Mid-point for 2RX (TS 36.133) A | New Mid point B = A – 3dB | Median Submitted Qout for 4RX C | B – C |
| --- | --- | --- | --- | --- | --- |
| 1x4 AWGN | [-9.5, -13.5] | -11.5 | -14.5 | -14.9 | 0.4 |
| 2x4 AWGN | [-9.5, -13.5] | -11.5 | -14.5 | -15.1 | 0.6 |
| 1x4 ETU70 | [-5.5, -11.5] | -8.5 | -11.5 | -12.7 | 1.2 |
| 2x4 ETU70 | [-6.2, -12.2] | -9.2 | -12.2 | -13.2 | 1.0 |

Table 2: Differences between Qout range mid-point proposed by [1] and median submitted Qout for 4Rx

For 4Rx case, the proposal in [1] is equivalent to subtracting 3 dB from the legacy 2Rx Qout Range Mid-point. Table 3 compares the new Mid-point (B = A – 3 dB) for 4RX with the median submitted Qout (C). In Table 2, we find that B could be 1 dB higher than C.

Observation 1: From Table 2 it can be seen that median submitted 4RX Qout could be more than 1 dB lower than expected Qout range mid-point, if a 3dB offset is used.

We therefore think it makes sense to add an additional factor to allow for this to the X = 3dB proposed in R4-161716.

Proposal: Considering Observation 1 and simulated Qout, we propose amending the proposals in [1] to use X = 4 dB rather than X = 3 dB.

# 3	Conclusion

In this contribution we propose to amend the proposals in R4-161716 for type 2 UE 4 Rx testing to use X = 4 dB rather than X = 3 dB to better align to modern receiver performance. Simulations based on PDCCH reception performance support this observation.

Observation 1: From Table 2 it can be seen that median submitted 4RX Qout could be 1 dB lower than expected Qout range mid-point, if 3dB offset is used.

Proposal: Considering Observation 1 and simulated Qout, we propose amending the proposals in [1] to use X = 4 dB rather than X = 3 dB.

# 4	Reference

[1] R4-161716, “Review of RLM test cases for type 2 UEs”, Ericsson.

[2] R4-153212, “Further discussion on 4Rx RLM”, Nokia Networks

[3] R4-151722, “On RLM feasibility for 4Rx”, Huawei, HiSilicon

[4] R4-154561, “Considerations on radio link monitoring for UE 4RX”, Ericsson

[5] R4-154713, “Updated simulation results for RLM for 4Rx”, ZTE

[6] R4-152767, “RLM simulation results for 4Rx”, Intel Corporation

[7] R4-154496, “Discussions and Evaluation on RLM for 4Rx UEs”, CMCC