3GPP TSG-RAN WG4 meeting #79	R4-163541

Nanjing, China, 23 – 27 May, 2016

Source:	Ericsson

Title:	TP to TR 36.714-02-01: coexistence analysis and UE requirements for CA_7A-66A

Agenda item:				7.2.1

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-02-01 for introduction of CA_7A-66A as fallback to the 3DL/1UL CA combination CA_2A-7A-66A.

# 2. Text Proposal for TR 36.714-02-01

##### ---Start of changes---

## 6.x		CA_2DL_7A-66A_1UL_BCS0

### 6.x.1	Operating bands for CA

Table 6.x.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_7-66 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz |  |

### 6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA band / channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Band | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| CA_7A-66A | 7 |  |  | Yes | Yes | Yes | Yes |
|  | 66 |  |  | Yes | Yes | Yes | Yes |

### 6.x.3	Co-existence studies

Table 6.x.3-1 includes the 2nd and 3rd order harmonic products for band 7 + band 66 CA with 1 UL. Table 6.x.3-2 shows the 2nd and 3rd order harmonics and IMD products identified for Band 7 and Band 66 DL:

Table 6.x.3-1: Band 4 and Band 7 UL harmonics products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 2500 | 2570 |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 5000 | 5140 |
| 3rd harmonics frequency limits (MHz) | 5130 | 5345 | 7500 | 7710 |

Table 6.x.3-2: Band 7 and Band 66 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2200 | 2620 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4220 | 4400 | 5240 | 5380 |
| 3rd harmonics frequency limits (MHz) | 6330 | 6600 | 7860 | 8070 |
| 2nd order IMD products | (f2_low – f1_low) / f2_low – f1_high | (f2_high – f1_ low) / (f2_high – f1_ high) | (f2_low + f1_low) / f2_low + f1_ high) | (f2_high + f1_low) / f2_high + f1_high) |
| MD frequency limits (MHz) | 510 / 420 | 580 / 490 | 4730 / 4820 | 4800 / 4890 |
| 2nd order IMD frequency range (MHz) | 420-580 |  | 4730-4890 |  |
| 3rd order IMD products | (2*f1_low – f2_low)/ (2*f1_low – f2_high) | (2*f1_high – f2_low) / (2* f1_high – f2_high) | (2*f2_low – f1_low) / 2*f2_low – f1_high) | (2*f2_high – f1_low) / 2*f2_high – f1_high) |
| IMD frequency limits (MHz) | 1600 / 1530 | 1780 / 1710 | 3130 / 3040 | 3270 /3180 |
| 3rd order IMD products | (2*f1_low + f2_low) / 2*f1_low + f2_high) | (2*f1_high + f2_low) / (2* f1_high + f2_high) | (2*f2_low + f1_low) / 2*f2_low + f1_high) | (2*f2_high + f1_low) / 2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 6840 / 6910 | 7020 / 7090 | 7350 / 7440 | 7490 / 7580 |
| 3rd order IMD frequency range (MHz) | 1530 – 1780 6840 - 7090 |  | 3040 – 3270 7350 - 7580 |  |

With reference to the above tables, it can be seen that there are 3rd order IMD product falling within the transmit band of Band 66. 2nd harmonics from the UE can fall into Band 22 and Band 42. Harmonics from the BS can fall into Band 46. However, it can be assumed that the harmonics will be attenuated by the duplex filter. It can be concluded that there is no issue on harmonic interference.

### 6.x.4	∆TIB and ∆RIB values

### Deriving ΔTIB,c and ΔRIB,c values for Band (7 + 66) we assume that the minimum performance for Band 7 and Band 66 is the same as that for these bands in the 3DL-combination Band (2 + 7 + 66) implemented by a hexplexer in FBAR technology as described in TR 36.714-03-01. It is likely that a UE supporting Band (7 + 66) also supports Band 2 so use of a hexplexer would reduce component count even if only 2DL combinations are supported.

Table 6.x.4-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_7A-66A | 7 | 0.5 |
|  | 66 | 0.5 |

# Table 6.x.4-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_7A-66A | 7 | 0 |
|  | 66 | 0.3 |

##### ---End of changes---