3GPP TSG-RAN WG4 #79	R4-164218

Nanjing, China, 23th – 27th May 2016

Source: 	Ericsson

Title:  	Details on multi-node tests for Rel-13 LAA

Agenda Item:	6.10.2

Document for:	Discussion

# Introduction

During RAN4#78bis, a way forward has been agreed on coexistence tests for Rel-13 LAA [1].

The coexistence tests are divided in two parts: LBT functionalities tests and multi-node tests. There are quite good agreement on LBT functionalities tests, thus, we provide a CR for TS 36.141 in [2].

In this contribution, we provide more detailed understanding on multi-node tests.

# Multi-node tests

The agreements in [1] are listed below:

- All tests should be conductive tests

- Agreed to use two nodes, the number of stations is FFS.

- Agreed to use one channel for the tests, i.e. single carrier tests.

- Agreed to use 20 MHz channel BW.

- Channel access priority:

  - Agreed to have a maximum of two priority classes

  - Details are FFS

  - Whether we test the priority classes simultaneously or not is also FFS.

- Metrics to use in the tests

  - Throughput is the metric for a best effort service.

  - Other metrics are FFS, depending on priority classes chosen.

- Scope of multi-node tests

  - Agreed to target “cross-technology fairness”.

  - Should focus on few key cases which can be easily reproduced and not add unnecessary tests.

  - Performance tests should include LAA to LAA and LAA to Wi-Fi performance

  - Agreed to test for high load, other loads FFS.

  - Whether to test Rx signals above ED threshold, or below ED threshold, or both are FFS.

- How to capture multi-node tests

  - Develop text for a new TR in RAN4

  - Propose the creation of a TR in an LS to TSG RAN.

The open issues are highlighted in yellow in the above texts.

In this section, we discuss the open issues on multi-node tests.

## Number of nodes in multi-node tests

An important issue is to decide on how many nodes are connected for the multi-node tests. We propose to include one LAA node, one WiFi AP, one LAA UE and one WiFi STA for the multi-node test setup.

As common understanding, the LAA UE will be connected to LAA BS and WiFi STA will be connected to WiFi AP. The detailed connection setup can vary depending on the implementation of the test setup.

An example setup is shown below in Figure 1.

Figure 1 Example setup for multi-node tests for Rel-13 LAA

The basic setup will be that both LAA node and WiFi AP will operate in best effort traffic, i.e. in priority class 3.

It is worth noting that we only concentrate on DL traffic in this setup. Thus, traffic is only expected in the LAA node and WiFi AP.

### Test cases

The test cases are listed below:

|  | Transmitter | Receiver | Comparison | Criteria |
| --- | --- | --- | --- | --- |
| Case 1: | LAA | WiFi | WiFi-WiFi | Impact of LAA transmitter on WiFi throughput: 2nd receiver system achieves certain level of throughput, its WiFi STA in this case. |
| Case 2: | WiFi | LAA | LAA-LAA | Impact of WiFi transmitter on LAA throughput: 2nd receiver system achieves certain level of throughput, its LAA UE in this case. |

LAA-LAA setup is quite well understood, since Rel-13 spec based LAA BS can be used as DUT for LAA-LAA setup. However, it remains to be seen how to define WiFi-WiFi setup. There can be a number of WiFi implementations, thus a number of WiFi-WiFi setups can be used for comparison.

### Metric

The metric for this test will be minimum throughput in Mbps. As an example: If there are several  WiFi-WiFi cases which  achieves a minimum (or average) throughput of TPW1 TPW2 ,…, TPWn, and LAA-WiFi case for corresponding WiFi implementations achieves throughput of TPL1, TPL2, …, TPLn, then the following conditions need to be met:

- TPW1 ≤ TPL1

- TPW2 ≤ TPL2

- …

- TPWn ≤ TPLn

For case 2, the same metric holds. Thus, the following need to be satisfied:

- {TPW1, TPW2 …. , TPWn }≤ TPL1

## Channel access priority classes

In R4-162989, RAN4 has agreed to have a maximum of two priority classes. It is worth mentioning here that, from spec point of view, different priority classes are optional. So, it’s not mandatory to implement all priority classes, thus, an LAA node may only implement one priority class (as an example). Thus, it is understood that:

Observation: Tests should only be performed for channel priority class capabilities that are declared by the LAA node.

As a result, if an LAA node is capable of performing only one channel priority class, then the LAA BS will be tested for only one priority class. Similarly, Maximum two priorities for node that can do anything more than one classes.

### Test cases

In addition to previous test cases, another set of tests can be done for LAA BS that also implements priority class 1. The test case can be described as follows:

|  | Transmitter | Receiver | Comparison | Traffic | Criterion |
| --- | --- | --- | --- | --- | --- |
| Case -3 | LAA BS | WiFi AP | WiFi-WiFi | VoIP traffic in all nodes | Outage should be better in second system receiver when the transmitter is replaced |
| Case- 4 | WiFi AP | LAA BS | LAA-LAA | VoIP traffic in all nodes |  |

### Metric

For channel priority class 1, i.e. for VoIP traffic, the metric will be actually the outage that the victim devices experience. The outage can be defined as VoIP client behavior, such as when does the VoIP client drop the packets and how many percentage of the packets are dropped within a certain time frame?

## Load

In our opinion, best effort traffic model is generally more suitable for the tests. One underlying principle can be that, same level of traffic need to be considered for cases which are compared for performance. This means that, the traffic that is assumed for WiFi-WiFi in case 3 should also be used in LAA-WiFi case in case 3. In this way, the performance comparison will be more reasonable.

More investigations are needed in this issue.

## Test signal levels

Test signal levels are an issue which should be carefully thought on. In the multi-node test design, the tests can be performed at least on two different signal levels at which the tested device listens to the channel for other active transmitter(s). These two levels are described below:

- Since LAA threshold is at -72dBm/20MHz, we propose to define one test signal level at above -72dBm/20MHz. This level is used to verify basic functionalities.

- Another test signal level can be defined which is well below the ED threshold. This can be used to verify enhanced below ED functionality. One good value could be to consider a test signal level around WiFi RSSI statistics. This can be discussed further to find a suitable level.

Thus, we describe two different RX signal level for tests, which are shown in Figure 2

Figure 2 Test signal levels for multi-node tests

# Conclusion

In this paper, we have summarized our understanding on open issues related to multi-node tests for Rel-13 LAA.

# References

R4-162989, “Way Forward on LAA coexistence tests”, Ericsson, Huawei, Qualcomm, Nokia.

R4-164217, CR for 36.141 on LBT functionalities tests