3GPP TSG-RAN WG4 Meeting #79	R4-164316

Nanjing, China, 23 - 27 May 2016

Agenda item:			6.13.5.3

Source:	Nokia

Title:	BS PUSCH Simulation Assumptions

Document for:		Discussion

# Introduction

New PUSCH formats are introduced for supporting NB-IoT UEs with 3.75KHz and 15KHz UL subcarrier spacings. In this paper, we discuss NPUSCH simulation assumptions for BS NPUSCH performance requirements.

# NB-IoT PUSCH

For NB-IoT, a single antenna port  is used for all uplink transmissions. The narrowband physical uplink shared channel supports two formats and each of the format can be transmitted with subcarrier spacings of or . Resource units are used to describe the mapping of the NPUSCH to resource elements. A resource unit is defined as  consecutive SC-FDMA symbols in the time domain and consecutive subcarriers in the frequency domain, where  and  are given by Table 1. The value of NslotsUL is 4 for both ∆f=3.75KHz and ∆f=15KHz. For , a radio frame contains 5 slots (2ms/slot) and 48 subcarriers. For ∆f=15KHz, a radio frame contains 20 slots, as legacy LTE.

Table 1: Supported combinations of , , and  [TS 36.211]

| NPUSCH format |  |  |  |
| --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 |
|  | 15 kHz | 1 | 16 |
|  |  | 3 | 8 |
|  |  | 6 | 4 |
|  |  | 12 | 2 |
| 2 | 3.75 kHz | 1 | 4 |
|  | 15 kHz | 1 | 4 |

The common PUSCH configuration is set in PUSCH-ConfigCommon-NB to specify and PUSCH-ConfigDedicated is used to specify the UE specific PUSCH configuration in TS 36.331. NB-IoT PUSCH configuration contains the following parameters [5]:

- ackNack-NumRepetitions: {r1, r2, r4, r8, r16, r32, r64, r128}

- srs-SubframeConfig: {0, …, 15}

- dmrs-Config:

  - threeTone-BaseSequence: (0..15)

  - threeTone-BaseSequence: (0..3)

  - sixTone-BaseSequence: (0..15)

  - sixTone-CyclicShift-r13: (0..3)

- pusch-allSymbols

NB-IoT PUSCH supports two new PUSCH formats:

- NPUSCH format 1, used to carry the UL-SCH

  - Modulation: BPSK or QPSK for =1, QPSK for

  - The coding steps for NPUSCH format 1 includes CRC attachment, channel coding, and rate matching

  - NPUSCH transmission is repeated in N times, where N=NRepNRUNslotsUL. where the value of repetition number  {1, 2, 4, 8, 16, 32, 64, 128} and the value of resource unit {1, …, 10} are given in the corresponding DCI and the value of NslotsUL (16 for ∆f=3.75KHz, and {2,4,8,16} for ∆f=15KHz) is the number of NB-IoT UL slots of the resource unit corresponding to the allocated number of subcarriers in the corresponding DCI.

- NPUSCH format 2, used to carry ACK/ NACK information

  - Modulation: BPSK only with =1

  - The coding steps for NPUSCH format 2 includes only channel coding, where 1 HARQ ACK/ NACK bit is coded into 16 all ‘0’ or all ‘1’ bits.

  - When a UE detects a NPDSCH transmission intended for the UE and for which an ACK/NACK shall be provided, start, the UE will need to send NPUSCH carrying ACK/NACK response using NPUSCH format 2 in N consecutive NB-IoT UL slots, where , where the value of  {1, 2, 4, 8, 16, 32, 64, 128} is given by AckNack-RURepetitions-Msg4 or AckNack-RURepetitions.

The Transport block size (TBS) table for NPUSCH are defined in the following Table in TS 36.213 [4]

Table 2: Transport block size (TBS) table for NPUSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 |  |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 |  |  |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 |  |  |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 |  |  |
| 11 | 176 | 376 | 584 | 776 | 1000 |  |  |  |
| 12 | 208 | 440 | 680 | 1000 |  |  |  |  |

# NPUSCH Simulation Assumptions

For NPUSCH Format 1, define the demodulation performance requirements under multipath fading propagation conditions for both NPUSCH subcarrier spacings. The performance requirement of NPUSCH in Format 1 should be determined by a minimum required throughput for a given SNR under multipath fading propagation conditions

Table 1: Proposal for NPUSCH format 1 Simulation Assumptions

| Parameters | Values |  |
| --- | --- | --- |
| Subcarrier Spacing | 3.75KHz | 15KHz |
| Channel model | AWGN, TU1 | AWGN, TU1 |
| Antenna configuration: 2 cases | 1x2 | 1x2 |
| Antenna correlation | Low | Low |
| Repetition level | TBD [{1, 2, 4, 8, 16, 32, 64, 128}] | TBD [{1, 2, 4, 8, 16, 32, 64, 128}] |
| pusch-allSymbols | Yes | Yes |
| Number of tones | 1 | {1, 3, 6, 12} |
| Modulation | {Pi/2-BPSK, Pi/4 QPSK} | {Pi/2-BPSK, Pi/4 QPSK} for single tone; {1/3 QPSK, QPSK} for multi-tones} |
| TBS | TBD from above Table 2 | TBD from above Table 2 |
| UE frequency error | [50] Hz | [50] Hz |

For NPUSCH Format 2, the performance requirements for HARQ ACK/NACK false detection. The ACK/NACK false detection probability for NPUSCH may be defined as the probability that ACK or NACK is detected when NACK or ACK is sent on NPUSCH. Unlike the legacy case, it may not need to define the ACK missed detection requirements since the eNB knows when the UE should transmit NPUSCH Format 2 based on previous NPDSCH/NPUSCH/DCI scheduling, and the content NPUSCH Format 2 has be either ACK or NACK

Table 2: Proposal for NPUSCH format 2 Simulation Assumptions

| Parameters | Values |  |
| --- | --- | --- |
| Subcarrier Spacing | 3.75KHz | 15KHz |
| Channel model | AWGN, TU1 | AWGN, TU1 |
| Antenna configuration: 2 cases | 1x2 | 1x2 |
| Antenna correlation | Low | Low |
| Repetition level | TBD [{1, 2, 4, 8, 16, 32, 64, 128}] | TBD [{1, 2, 4, 8, 16, 32, 64, 128}] |
| pusch-allSymbols | Yes | Yes |
| Number of tones | 1 | 1 |
| Modulation | Pi/2-BPSK | Pi/2-BPSK |
| TBS | TBD from above Table 2 | TBD from above Table 2 |
| UE frequency error | [50] Hz | [50] Hz |

# Conclusion

In this contribution, we discussed the simulation assumptions for BS NPUSCH demodulation performance requirements.

# References

- 3GPP TS 36.104, “E-UTRA: Base Station (BS) radio transmission and reception”.

- R1-163952, “Introduction of NB-IoT (rev.6)”, Ericsson

- R1-163951, “Introduction of Rel-13 feature of NB-IoT in 36.212”, Huawei, HiSilicon

- R1-163912

- R2-16xxxx, 36.331 Running CR to capture agreements on NB-IoT TS 36.331, Huawei (Rapporteur)