3GPP TSG-WG4 Meeting #79	R4-164439

# Nanjing, China, 23rd – 27th, May 2016

Agenda item: 	6.13.4.7

Source: 	Qualcomm Incorporated

Title: 	Reference Measurement Channels for NB-IoT

Document for:	Discussion

# 1		Introduction

This paper proposes a framework for Reference Measurement Channels for NB-IoT channels, NPBCH, NPDSCH, NPDCCH and receiver characteristics.

# 2	Reference Measurement Channels for NPBCH performance requirements (Category NB1)

Table 1: Reference Channel for Category NB1 UE

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.1 | R.NB1.2 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | KHz | 180 | 180 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Payload (without CRC) | Bits | 34 | 34 |

# 3	Reference Measurement Channels for NPDSCH performance requirements (Category NB1)

Table 2: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.3 | R.NB1.4 |
| Channel bandwidth | KHz | 180 | 180 |
| Allocated resource blocks |  | 1 | 2 |
| Allocated subframes per Radio Frame |  | 8 or 9 | 8 or 9 |
| Modulation |  | QPSK | QPSK |
| NB-PDSCH repetition number, NRep Allowed values: {1, 2, 4, 8, 16, 32, 64, 128, 192, 256, 384, 512, 768, 1024, 1536, 2048} |  | TBD | TBD |
| Number of subframes used for 1 codeword, NSF Allowed values: {1 to 10} |  | TBD | TBD |
| ITBS Allowed values: {0 to 12} |  | TBD | TBD |
| Target Coding Rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 0, 1,2,3,4,6,7,8 and 9(if NB-SSS is not present) | Bits | 680 | 680 |
| For Sub-Frame 5 | Bits | 0 | 0 |
| For Sub-Frame 9 (if NB-SSS is present) | Bits | 0 | 0 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 0, 1,2,3,4,6,7,8 and 9(if NB-SSS is not present) |  | 1 | 1 |
| For Sub-Frame 5 |  | 0 | 0 |
| For Sub-Frame 9 (if NB-SSS is present) |  | 0 | 0 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 0, 1,2,3,4,6,7,8 and 9(if NB-SSS is not present) | Bits | 2112 | 2112 |
| For Sub-Frame 5 | Bits | 0 | 0 |
| For Sub-Frame 9 (if NB-SSS is present) | Bits | 0 | 0 |
| Max. Throughput averaged over 1 frame | Mbps | TBD | TBD |
| UE Category |  | NB1 | NB1 |

The following are the allowed combinations of { ITBS, NSF }. Only a subset of these combinations will be defined for NPDSCH.

|  | 1 | 2 | 3 | 4 | 5 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

# 4	Reference Measurement Channels for NPDCCH performance requirements (Category NB1)

RMCs for all possible NPDCCH repetition levels are shown below i.e. allowed values = {1, 2, 4, 8, 16, 32, 64, 128, 256, 512, 1024, 2048}. Only a subset of the following RMCs will be defined for NPDCCH.

Table 3: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.5 | R.NB1.6 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | KHz | 180 | 180 |
| Repetition Level | symbols | 1 | 1 |
| Aggregation level | CCE | 1 | 1 |
| DCI Format |  | N1 | N1 |
| Cell ID |  | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD |

Table 4: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.NB1.7 | R.NB1.8 | R.NB1.9 | R.NB1.10 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Channel bandwidth | KHz | 180 | 180 | 180 | 180 |
| Repetition Level | symbols | 2 | 2 | 4 | 4 |
| Aggregation level | CCE | 2 | 2 | 2 | 2 |
| DCI Format |  | N1 | N1 | N1 | N1 |
| Cell ID |  | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD | TBD | TBD |

Table 5: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.NB1.11 | R.NB1.12 | R.NB1.13 | R.NB1.14 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Channel bandwidth | KHz | 180 | 180 | 180 | 180 |
| Repetition Level | symbols | 8 | 8 | 16 | 16 |
| Aggregation level | CCE | 2 | 2 | 2 | 2 |
| DCI Format |  | N1 | N1 | N1 | N1 |
| Cell ID |  | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD | TBD | TBD |

Table 6: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.NB1.15 | R.NB1.16 | R.NB1.17 | R.NB1.17 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 1 |
| Channel bandwidth | KHz | 180 | 180 | 180 | 180 |
| Repetition Level | symbols | 32 | 32 | 64 | 64 |
| Aggregation level | CCE | 2 | 2 | 2 | 2 |
| DCI Format |  | N1 | N1 | N1 | N1 |
| Cell ID |  | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD | TBD | TBD |

Table 7: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.NB1.18 | R.NB1.19 | R.NB1.20 | R.NB1.21 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 2 |
| Channel bandwidth | KHz | 180 | 180 | 180 | 180 |
| Repetition Level | symbols | 128 | 128 | 256 | 256 |
| Aggregation level | CCE | 2 | 2 | 2 | 2 |
| DCI Format |  | N1 | N1 | N1 | N1 |
| Cell ID |  | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD | TBD | TBD |

Table 8: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value | Value | Value |
| --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.NB1.22 | R.NB1.23 | R.NB1.24 | R.NB1.25 |
| Number of transmitter antennas |  | 1 | 2 | 1 | 1 |
| Channel bandwidth | KHz | 180 | 180 | 180 | 180 |
| Repetition Level | symbols | 512 | 512 | 1024 | 1024 |
| Aggregation level | CCE | 2 | 2 | 2 | 2 |
| DCI Format |  | N1 | N1 | N1 | N1 |
| Cell ID |  | 0 | 0 | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD | TBD | TBD |

Table 9: Fixed Reference Channel for Category NB1 UE

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB1.26 | R.NB1.27 |
| Number of transmitter antennas |  | 1 | 2 |
| Channel bandwidth | KHz | 180 | 180 |
| Repetition Level | symbols | 2048 | 2048 |
| Aggregation level | CCE | 2 | 2 |
| DCI Format |  | N1 | N1 |
| Cell ID |  | 0 | 0 |
| Payload (without CRC) | Bits | TBD | TBD |

# 5	Reference Measurement Channels for Receiver charateristics (Category NB1)

Table 10 Fixed Reference Channel for Receiver Requirements

| Parameter | Unit | Value |
| --- | --- | --- |
| Channel bandwidth | KHz | 180 |
| Allocated resource blocks |  | 1 |
| Subcarriers per resource block |  | 12 |
| Allocated subframes per Radio Frame |  | 8 or 9 |
| Modulation |  | QPSK |
| Target Coding Rate |  | 1/3 |
| NB-PDSCH repetition number, NRep Allowed values: {1, 2, 4, 8, 16, 32, 64, 128, 192, 256, 384, 512, 768, 1024, 1536, 2048} |  | TBD |
| Number of subframes used for 1 codeword, NSF Allowed values: {1 to 10} |  | TBD |
| ITBS Allowed values: {0 to 12} |  | TBD |
| Number of HARQ Processes | Processes | 1 |
| Maximum number of HARQ transmissions |  | 1 |
| Information Bit Payload |  |  |
| For Sub-Frames 0, 1,2,3,4,6,7,8 and 9(if NB-SSS is not present) | Bits | 680 |
| For Sub-Frame 5 | Bits | 0 |
| For Sub-Frame 9 (if NB-SSS is present) | Bits | 0 |
| Number of Code Blocks |  |  |
| For Sub-Frames 0, 1,2,3,4,6,7,8 and 9(if NB-SSS is not present) |  | 1 |
| For Sub-Frame 5 |  | 0 |
| For Sub-Frame 9 (if NB-SSS is present) |  | 0 |
| Binary Channel Bits |  |  |
| For Sub-Frames 0, 1,2,3,4,6,7,8 and 9(if NB-SSS is not present) | Bits | 2112 |
| For Sub-Frame 5 | Bits | 0 |
| For Sub-Frame 9 (if NB-SSS is present) | Bits | 0 |
| Max. Throughput averaged over 1 frame | Mbps | TBD |
| UE Category |  | NB1 |

# 6	Conclusion

This paper has proposed a framework for following RMCs for NB-IoT:

- Reference Measurement Channels for NPBCH performance requirements (Category NB1)

- Reference Measurement Channels for NPDSCH performance requirements (Category NB1)

- Reference Measurement Channels for NPDCCH performance requirements (Category NB1)

- Reference Measurement Channels for Receiver charateristics (Category NB1)

# 7	References

- TS36.101 v13.2.0, “User Equipment (UE) radio transmission and reception”