3GPP TSG-RAN WG4 #80	R4-165624

# Gothenburg, Sweden, 22 – 26 August, 2016

Agenda item:	6.5.1.1

Source: 	Qualcomm Incorporated

Title: 	Accuracy requirements for RS-SINR measurement

Document for:	Discussion

# Introduction

RS-SINR measurement for multicarrier load distribution has been discussed in RAN4. In this contribution we provide updated simulation results and our recommendation for RS-SINR measurement accuracy requirements.

# Discussion

## Simulations results Accuracy in AWGN channel

Our simulation results are based on simulation assumptions agreed in the RAN4 #76 way forward [1]. Key parameters of the simulation are provided in Table 1.

Table 1: Link level simulation assumptions for RS-SINR measurement

| Parameters | Value |
| --- | --- |
| Measurement bandwidth | 6 resource blocks, |
| System bandwidth | 50 resource blocks |
| Measurement sampling rate |  |
| L3 filtering | Disabled |
| Transmit antenna | 1 |
| Receive antennas | 2 |
| Propagation conditions | AWGN |
| DRX/DTX | OFF |
| CP length | Normal |
| Carrier frequency | 2 GHz |

In Figure 1, we show the average measured RS-SINR for an AWGN channel (without any frequency offset) as a function of the set RS-SINR.

Table 1: Absolute Accuracy performance of RS-SINR measurement

| Set SINR (Ec/Iot) | Distribution |  |
| --- | --- | --- |
|  | 5 percentile | 95 percentile |
| -6dB | -9dB | -3dB |
| -3dB | -5.5dB | -0.5dB |
| 0dB | -2.5dB | 2.3dB |
| 5dB | 3dB | 7dB |
| 10dB | 8dB | 12dB |
| 15dB | 13.5dB | 16.5dB |
| 20dB | 18.5dB | 21.5dB |

## Proposal for accuracy requirements

The simulation results presented in Table 1 are for an ideal AWGN scenario, i.e., it assumes no frequency error or timing error. Realistic frequency and timing estimation will further degrade the SNR estimation. Hence some additional margin is required to accommodate such errors.

### Absolute Accuracy

Based on the results, we have the following proposals for absolute accuracy.

Proposal 1: For lower side condition of SINR > -6dB, we propose an absolute accuracy requirement of 3.5dB.

Proposal 2: For higher side condition of SINR > -3dB, we propose an absolute accuracy requirement of 3dB.

### Relative Accuracy

In order to compute the relative accuracy of SNR estimate, we first find the distribution of the difference between two SNR estimates for the same set SINR. Then, we look at 5 and 95 percentile values of such a distribution and accordingly compute relative accuracy as the maximum among the absolute values of the 5 and 95 values.

Table 2: Relative accuracy performance of RS-SINR measurement

| Set SINR (Ec/Iot) | Relative accuracy |
| --- | --- |
| -6dB | 4dB |
| -3dB | 3.5dB |
| 0dB | 3.5dB |
| 5dB | 3dB |
| 10dB | 2.5dB |
| 15dB | 2.2dB |
| 20dB | 2.2dB |

Based on the results in Table 2, we propose that the relative accuracy requirements should be the following.

Proposal 3: Relative accuracy requirement for lower side condition, i.e., SINR > -6dB should be 4.5dB.

Proposal 4: Relative accuracy requirement for higher side condition, i.e., SINR > -3dB should be 4dB.

# Conclusion

In this contribution, we presented simulations results for RS-SINR measurements of in AWGN channel. Based on the simulation results we make the following proposals.

Proposal 1: For lower side condition of SINR > -6dB, we propose an absolute accuracy requirement of 3.5dB.

Proposal 2: For higher side condition of SINR > -3dB, we propose an absolute accuracy requirement of 3dB.

Proposal 3: Relative accuracy requirement for lower side condition, i.e., SINR > -6dB should be 4.5dB.

Proposal 4: Relative accuracy requirement for higher side condition, i.e., SINR > -3dB should be 4dB.

# References

[1] R4-155139, “Way forward on RS-SINR measurement”, CMCC, Intel, Ericsson, Nokia, RAN4 #76 Aug 2015