3GPP TSG-RAN WG4 RAN4 #80                                        R4-165933

Gothenburg, Sweden, 22 – 26 Aug, 2016

Agenda item:			9.3.3

Source:			Samsung, China Telecom

Title:			Discussion on Detailed Coexistence Simulation Assumptions and Parameters

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72. According to WID [1], the study item has set up objectives as follows:

- Identify operating bands for interference evaluation on NB-IoT coexistence with CDMA, such as bands around 800M (B5, B26 etc.) - For the NB-IoT stand-alone operation mode, evaluate coexistence requirement between NB-IoT and CDMA, and further evaluate whether R13 NB-IoT RF requirements could be reused or not. - Identify necessary additional RF requirements to ensure NB-IoT for co-existence with CDMA.

In this contribution, we provide our proposals for the detailed simulation assumptions and parameters for simulation alignment purpose. It should be noted that this contribution is accompanying paper on the general coexistence simulation scenarios and methodology [2], and another paper on deriving CDMA2000 RF performance [3]. Furthermore, initial simulation results are provided based on the assumptions and parameters given in this paper [4].

# Coexistence Simulation Assumptions

Based upon to WID [1], RAN4 should evaluate coexistence requirement between NB-IoT and CDMA to determine whether Rel-13 NB-IoT RF requirements could be reused or not. Therefore, we should follow the system assumption and parameters for NB-IoT utilized in TR 36.802. Specifically, the detailed parameters are provided in the following table. It should be noted that the BS/UE ACLR/ACS follows the requirement defined in Rel-13, with carrier frequency is set to be 850MHz [2].

- Proposal 1: Reuse NB-IoT standalone assumptions and parameters from Rel-13 coexistence study as much as possible.

On the other hand, the simulation assumptions for CDMA system are provided in the following table, based on the following two references:

[5] 3GPP TR 25.942, “Radio Frequency (RF) system scenarios”.

[6] 3GPP2 C.R 1002 v1.0, “CDMA2000 Evaluation Methodology”.

Table 2.1 Simulation assumption for coexistence study between NB-IoT and CDMA2000 1x system

|  | NB-IoT standalone | CDMA2000 1x Refer to TR25.942 [5] | CDMA2000 1x Refer to CDMA2000 Evaluation Methodology [6] |
| --- | --- | --- | --- |
| Carrier frequency in GHz | 0.85 | 0.85 | 0.85 |
| Size of each nominal channel BW in MHz | 0.2 | 1.23 | 1.23 |
| Transmission bandwidth in MHz | 0.18 | 1.23 | 1.23 |
| Environment | Urban macro | Urban macro | Urban macro |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 750m (Same for aggressor and victim) | 750m (Same for aggressor and victim) | 750m (Same for aggressor and victim) |
| System loading and activity | Full buffer 100% | All users with 9.6kbps speech | All users with 9.6kbps speech |
| Network location | Non co-located (at cell edge of legacy) | Non co-located with IoT | Non co-located with IoT |
| DL subcarrier spacing | 15kHz | n/a | n/a |
| UL | See RP-152284(both multi-tone and single-tone) | n/a | n/a |
| DL power control | No | TR25.942 (reuse UMTS PC) | Similar power control mechanism |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB | TR25.942 (reuse UMTS PC) | Similar power control mechanism |
| Frequency reuse | 1 | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | According to 95% users achieving target of (Eb/N0-0.5)dB; non orthogonality 0.2; target Eb/N0=5.5dB | n/a |
| Number of scheduled UE per cell (UL) | 3 for multi-tone (60kHz per UE), 12 for 15kHz single-tone, 48 for 3.75kHz single-tone | according to 5.5dB noise rise; target Eb/N0=4.0dB no MUD utilized, beta=0 | n/a |
| BS antenna height in meter | 30 | 30 | 32 |
| BS max TX power in dBm | 43/200kHz | 43/1.25MHz | 43/1.25MHz |
| BS Tx power of common channel in dBm* | n/a | 36.5 (R4-030558 22.4% Total Power) | 36.0 (20% Total Power) |
| BS antenna gain including feeder loss in dBi | 15 | 15 | 15 |
| BS antenna pattern | Horizontal (TR36.942) | Horizontal (65deg for -3dB) | Horizontal (70deg for -3dB) |
| BS antenna front-back ratio in dB | 20 | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 | 1.5 |
| UE TX power in dBm | -40 to 23 | -52 to 23 | n/a to 23 |
| UE antenna gain in dBi | 0 | 0 | -1 |
| Building penetration loss | 45.820 Annex D.1 | n/a | n/a |
| Cell selection margin in dB | 3 | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 | 70 |
| BS ACLR in dB | 40 to 60** | 36.8 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |  |
| BS ACS in dB | 40 to 50** | 81 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |  |
| UE ACLR in dB | 20 to 50** | 33.8 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |  |
| UE ACS in dB | 20 to 40** | 49.5 (own channel measurement BW: 1.23MHz adjacent channel measurement BW: 180kHz) |  |
| BS noise figure in dB | 5 | 5 | 5 |
| UE noise figure in dB | 9 | 9 | 10 |
| BS-UE path-loss model | TR36.942 macro urban | TR36.942 macro urban | Modified Hata Urban |
| Standard deviation of BS-UE log-normal shadow fading in dB | 10 | 10 | 8.9 |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Evaluation metrics | SINR vs ACS (as victim) | Capacity vs NB-IoT ACLR (as victim). For DL, capacity is the number of satisfied users. For UL, capacity is the number of users when 5.5dB noise rise is reached | Noise Rise 7dB |
| Carrier separation | 1.1MHz to CDMA(central frequency separation) Other Options are not Excluded | 1.1MHz to NB-IoT(central frequency separation) Other Options are not Excluded | n/a |

*DL overhead channel resource consumption, including pilot, paging and sync overhead.

**Similar to coexistence study in TR36.802, NB-IoT ACLR and ACS are defined for its own bandwidth size.

It should be noted that the evaluation methodology [6] is widely used to evaluate 3GPP2 technologies. By comparing the detailed parameters from [5] and [6] (with differences being highlighted in the following table), there is no big difference between each other, and we prefer to use the set of parameters by TR 25.942 [5], since it is already be utilized for similar evaluation by 3GPP.

- Proposal 2: The CDMA2000 1x parameters provided in TR25.942 should be followed.

#### 5.1 CDMA2000 System RF Performance

We provided another discussion paper on the method to determine CDMA2000 system RF parameters, in which the details for CDMA2000 BS/UE ACS/ACLR achievable performance are provided [3].

It should be noted that for CDMA2000 BS/UE ACS/ACLR, all the metrics are defined with adjacent channel measurement bandwidth being 180 kHz.

#### 5.2 NB-IoT RF Performance

The same range of NB-IoT RF performance will be utilized in the simulation, while the simulated NB-IoT RF performance required for coexistence with CDMA system will be further discussed in RAN4 (in the Step-3 mentioned in the general methodology [2]) to determine that the required NB-IoT requirement can be fulfilled by current Rel-13 NB-IoT requirement.

It should be noted that similar to coexistence study in TR36.802, NB-IoT ACLR and ACS are defined for its own bandwidth size.

#### 5.3 Assumptions and Calculations for ACLR/ACS Modeling

Similar to the assumption utilized in Rel-13 coexistence study for NB-IoT [4], the following assumptions and calculations for ACLR/ACS modeling should be applied:

•	Definition and assumption

•	CDMA2000 ACLR and ACS are defined with adjacent channel measurement bandwidth 180kHz

•	NB-IoT ACLR and ACS are defined for its own bandwidth size

•	Attenuations are flat in the adjacent channels

•	ACLR and ACS are the inputs to the simulations

•	ACLR_e is the effective ACLR in the interference calculation

•	ACS_e is the effective ACS in the interference calculation

•	Interference from CDMA2000 to NB-IoT

•	ACLR_e = ACLRCDMA – 10*log10(1/(# of IoT users))

•	ACS_e = ACSNB-IoT [pessimistic assumption]

•	Interference from NB-IoT to CDMA2000

•	ACLR_e = ACLRNB-IoT – 10*log10(1.23MHz/180kHz) [pessimistic assumption]

•	ACS_e = ACSCDMA

- Proposal 3: Following assumptions and calculations for ACLR/ACS modeling should be applied:

  - Definition and assumption

    - CDMA2000 ACLR and ACS are defined with adjacent channel measurement bandwidth 180kHz

    - NB-IoT ACLR and ACS are defined for its own bandwidth size

    - Attenuations are flat in the adjacent channels

    - ACLR and ACS are the inputs to the simulations

    - ACLR_e is the effective ACLR in the interference calculation

    - ACS_e is the effective ACS in the interference calculation

  - Interference from CDMA2000 to NB-IoT

    - ACLR_e = ACLRCDMA – 10*log10(1/(# of IoT users))

    - ACS_e = ACSNB-IoT [pessimistic assumption]

  - Interference from NB-IoT to CDMA2000

    - ACLR_e = ACLRNB-IoT – 10*log10(1.23MHz/180kHz) [pessimistic assumption]

    - ACS_e = ACSCDMA

#### 5.4 Criteria for Coexistence Evaluation

Similar to the coexistence study in TR 36.802, the criteria captured in following table have been used to conclude on NB-IoT coexistence study with CDMA system. For CDMA2000 1x system, we propose to reuse the criteria for UMTS, due to the similarity of both systems to provide voice service with capacity being the number of served users.

Table 2.2: Criteria for coexistence evaluation

| RAT | Criteria |
| --- | --- |
| NB-IoT | Average of SNR value loss is less or equal to 1 dB |
| CDMA2000 1x | Capacity loss is less or equal to 5% |

- Proposal 4: The criteria to conclude on NB-IoT coexistence study with CDMA system should be:

  - NB-IoT: Average of SNR value loss is less or equal to 1 dB;

  - CDMA2000 1x: Capacity loss is less or equal to 5%.

# Conclusion

In this contribution, we provide our proposals for the detailed simulation assumptions and parameters for simulation alignment purpose. Specifically, the following proposals are given:

- Proposal 1: Reuse NB-IoT standalone assumptions and parameters from Rel-13 coexistence study as much as possible.

- Proposal 2: The CDMA2000 1x parameters provided in TR25.942 should be followed.

- Proposal 3: Following assumptions and calculations for ACLR/ACS modeling should be applied:

  - Definition and assumption

    - CDMA2000 ACLR and ACS are defined with adjacent channel measurement bandwidth 180kHz

    - NB-IoT ACLR and ACS are defined for its own bandwidth size

    - Attenuations are flat in the adjacent channels

    - ACLR and ACS are the inputs to the simulations

    - ACLR_e is the effective ACLR in the interference calculation

    - ACS_e is the effective ACS in the interference calculation

  - Interference from CDMA2000 to NB-IoT

    - ACLR_e = ACLRCDMA – 10*log10(1/(# of IoT users))

    - ACS_e = ACSNB-IoT [pessimistic assumption]

  - Interference from NB-IoT to CDMA2000

    - ACLR_e = ACLRNB-IoT – 10*log10(1.23MHz/180kHz) [pessimistic assumption]

    - ACS_e = ACSCDMA

- Proposal 4: The criteria to conclude on NB-IoT coexistence study with CDMA system should be:

  - NB-IoT: Average of SNR value loss is less or equal to 1 dB;

  - CDMA2000 1x: Capacity loss is less or equal to 5%.

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP R4-165931, “Discussion on General Coexistence Scenarios and Simulation Methodology”, Samsung.

[3] 3GPP R4-165932, “Discussion on CDMA2000 1x RF Performance”, Samsung.

[4] 3GPP R4-165934, “Simulation Results on NB-IoT Coexistence Study with CDMA system for Uplink Scenarios”, Samsung.

[5] 3GPP TR 25.942, “Radio Frequency (RF) system scenarios”.

[6] 3GPP2 C.R 1002 v1.0, “CDMA2000 Evaluation Methodology”.