3GPP TSG-RAN WG4 Meeting #80 	R4-166666

# Gothenburg, Sweden, 22 – 26 August, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.104 | CR | 0302 | rev | - | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for Spurious responses for NB-IoT BS receiver blocking requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Alcatel-Lucent Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  |  | Date: |  | 2016-08-12 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | A limited number of spurious responses shall be allowed for NB-IoT BS receiver blocking requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that a limited number of spurious responses shall be allowed for NB-IoT BS receiver blocking requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious responses are not allowed for NB-IoT BS receiver blocking requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.1, 7.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

### 7.5.1	General minimum requirement

For a wanted and an interfering signal coupled to BS antenna input using the parameters in Table 7.5.1-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.

-	For any UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [3], subclause 7.2.

-	For any GSM/EDGE carrier, the conditions are specified in TS 45.005 [5], Annex P.2.1.

-	For any NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

For BS capable of multi-band operation, the requirement applies for each supported operating band. The in-band blocking frequency ranges of all supported operating bands according to Table 7.4.1-1 shall be excluded from the requirement.

Table 7.5.1-1: Blocking performance requirement

| Operating Band Number | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18, 19, 21-23, 24, 27, 30, 33-45, 65, 66, 68 | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| 8, 26, 28 | 1 (FUL_high +10) | to to | (FUL_low -20) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| 12 | 1 (FUL_high +13) | to to | (FUL_low -20) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| 17 | 1 (FUL_high +18) | to to | (FUL_low -20) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| 20 | 1 (FUL_high +20) | to to | (FUL_low -11) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| 25 | 1 (FUL_high +15) | to to | (FUL_low -20) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| 31 | 1 (FUL_high +5) | to to | (FUL_low -20) 12750 | -15** | PREFSENS + x dB* | CW carrier |
| NOTE*: PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2. “x” is equal to 6 in case of E-UTRA or UTRA or NB-IoT wanted signals and equal to 3 in case of GSM/EDGE wanted signal. NOTE**: For NB-IoT, spurious response frequencies are allowed for a maximum of 24 occurrences (which if below the wanted signal and grouped shall not exceed three contiguous occurrences per group), where the above throughput requirement shall be met when the blocking signal is set to a level of -40 dBm for 15 kHz subcarrier spacing and -46 dBm for 3.75 kHz subcarrier spacing. |  |  |  |  |  |  |

### 7.5.2	Co-location minimum requirement

This additional blocking requirement may be applied for the protection of BS receivers when E-UTRA BS, NB-IoT, UTRA, CDMA or GSM/EDGE operating in a different frequency band are co-located with a BS.

The requirements in this subclause assume a 30 dB coupling loss between the interfering transmitter and the BS receiver and are based on co-location with base stations of the same class.

For a wanted and an interfering signal coupled to BS antenna input using the parameters in Table 7.5.2-1, the following requirements shall be met:

-	For any E-UTRA carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

-	For any UTRA FDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.104 [2], subclause 7.2.

-	For any UTRA TDD carrier, the BER shall not exceed 0.001 for the reference measurement channel defined in TS 25.105 [3], subclause 7.2.

-	For any GSM/EDGE carrier, the conditions are specified in TS 45.005 [5], Annex P.2.1.

-	For any NB-IoT carrier, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel defined in TS 36.104 [4], subclause 7.2.

Table 7.5.2-1: Blocking requirement for co-location with BS in other frequency bands.

| Type of co-located BS | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power for WA BS (dBm) | Interfering Signal mean power for MR BS (dBm) | Interfering Signal mean power for LA BS (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- |
| GSM850 or CDMA850 | 869 – 894 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| GSM900 | 921 – 960 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| DCS1800 | 1805 – 1880 (Note 4) | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| PCS1900 | 1930 – 1990 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band I or E-UTRA Band 1 | 2110 – 2170 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band II or E-UTRA Band 2 | 1930 – 1990 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band III or E-UTRA Band 3 | 1805 – 1880 (Note 4) | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band IV or E-UTRA Band 4 | 2110 – 2155 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band V or E-UTRA Band 5 | 869 – 894 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band VI or E-UTRA Band 6 | 875 – 885 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band VII or E-UTRA Band 7 | 2620 – 2690 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band VIII or E-UTRA Band 8 | 925 – 960 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band IX or E-UTRA Band 9 | 1844.9 – 1879.9 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band X or E-UTRA Band 10 | 2110 – 2170 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XI or E-UTRA Band 11 | 1475.9 - 1495.9 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XII or E-UTRA Band 12 | 729 - 746 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XIIII or E-UTRA Band 13 | 746 - 756 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XIV or E-UTRA Band 14 | 758 - 768 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 17 | 734 - 746 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 18 | 860 - 875 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XIX or E-UTRA Band 19 | 875 - 890 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XX or E-UTRA Band 20 | 791 - 821 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXI or E-UTRA Band 21 | 1495.9 – 1510.9 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXII or E-UTRA Band 22 | 3510 – 3590 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 23 | 2180 - 2200 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 24 | 1525 – 1559 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXV or E-UTRA Band 25 | 1930 – 1995 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA FDD Band XXVI or E-UTRA Band 26 | 859 – 894 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 27 | 852 - 869 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 28 | 758 – 803 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 29 | 717 – 728 | +16** | +8 | -6 | PREFSENS + 6dB* | CW carrier |
| E-UTRA Band 30 | 2350-2360 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 31 | 462.5 – 467.5 | +16** | +8 | -6 | PREFSENS + 6dB* | CW carrier |
| UTRA FDD Band XXXII or E-UTRA Band 32 | 1452 – 1496 (NOTE 5) | +16** | +8 | -6 | PREFSENS + 6dB* | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 33 | 1900-1920 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band a) or E-UTRA TDD Band 34 | 2010-2025 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 35 | 1850-1910 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band b) or E-UTRA TDD Band 36 | 1930-1990 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band c) or E-UTRA TDD Band 37 | 1910-1930 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band d) or E-UTRA Band 38 | 2570-2620 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band f) or E-UTRA Band 39 | 1880-1920 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| UTRA TDD Band e) or E-UTRA Band 40 | 2300-2400 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 41 | 2496 - 2690 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 42 | 3400 – 3600 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 43 | 3600 – 3800 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 44 | 703 - 803 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 45 | 1447 - 1467 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 46 | 5150 - 5925 | N/A | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 65 | 2110 – 2200 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 66 | 2110 – 2200 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 67 | 738 - 758 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| E-UTRA Band 68 | 753 - 783 | +16** | +8 | -6 | PREFSENS + x dB* | CW carrier |
| NOTE 1 (*): PREFSENS depends on the RAT, the BS class and the channel bandwidth, see subclause 7.2. “x” is equal to 3 in case of GSM/EDGE wanted signal and equal to 6 in case of UTRA or E-UTRA or NB-IoT wanted signals. NOTE 2: Except for a BS operating in Band 13, these requirements do not apply when the interfering signal falls within any of the supported uplink operating band or in the 10 MHz immediately outside any of the supported uplink operating band. For a BS operating in band 13 the requirements do not apply when the interfering signal falls within the frequency range 768-797 MHz. . NOTE 3: Some combinations of bands may not be possible to co-site based on the requirements above. The current state-of-the-art technology does not allow a single generic solution for co-location of UTRA TDD or E-UTRA TDD with E-UTRA FDD on adjacent frequencies for 30dB BS-BS minimum coupling loss. However, there are certain site-engineering solutions that can be used. These techniques are addressed in TR 25.942 [7]. NOTE 4: In China, the blocking requirement for co-location with DCS1800 and Band III BS is only applicable in the frequency range 1805-1850MHz. NOTE 5: For a BS operating in band 11 or 21, this requirement applies for interfering signal within the frequency range 1475.9-1495.9 MHz. NOTE 6: Co-located TDD base stations that are synchronized and using the same or adjacent operating band can receive without special co-location requirements. For unsynchronized base stations, special co-location requirements may apply that are not covered by the 3GPP specifications. NOTE 7 (**): For NB-IoT, spurious response frequencies are allowed for a maximum of 24 occurrences (which if below the wanted signal and grouped shall not exceed three contiguous occurrences per group), where the above throughput requirement shall be met when the blocking signal is set to a level of -9 dBm for 15 kHz subcarrier spacing and -15 dBm for 3.75 kHz subcarrier spacing. |  |  |  |  |  |  |  |

<End of change>