3GPP TSG-RAN WG4 Meeting #80 	R4-165442

# Gothenburg SE, 22 - 26 Aug 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 3730 | rev |  | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR: OCNG pattern for standalone NB-IoT RRM tests |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2016-08-08 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | No OCNG pattern for standalone NB-IoT RRM tests |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define standalone NB-IoT RRM tests. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No OCNG pattern for standalone NB-IoT RRM tests |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Section A.3.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---START OF CHANGE 1---

#### A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: Standalone NB-IoT cell

Table A.3.2.3.2-1: NOP.2 FDD: OCNG FDD Pattern 2

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 - 4 | 0 (Note 2) | 0 (Note 2) | 0 | Note 1 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: =0 is anchor PRB. Value of is applicable to PRBs not used for transmission of NPSS, NSSS, and NPBCH in anchor cell. |  |  |  |  |

--- END OF CHANGE ---