3GPP TSG-RAN WG4 Meeting #80 	R4-165558

# Gothenburg, Sweden, 22 – 26 Aug, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 3720 | rev | - | Current version: | 13.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce signal model for LAA demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2016-08-5 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The signal model for LAA is agreed, but it is not captured in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce signal model for LAA demodulation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test for LAA can not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | Y |  | Test specifications |  |  | TS36.521 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

# B.7	Burst transmission models for Frame structure type 3

This clause provides a description for the modelling of burst transmission models for Frame structure type 3.

One burst is defined as downlink transmissions which occupy one or more consecutive subframes. The burst transmission format is determined according to the steps below:

- Select the number of subframes N randomly from a given set of the number of subframes S1 with equal probability as the total length of burst transmission format. The length includes both occupied OFDM symbols and non-occupied OFDM symbols within the burst format.  S1 is given per test case.

- If N is equal to 1, the subframe is set as fully occupied, otherwise:

If the higher layer parameter subframeStartPosition indicates ‘s07’, the starting position for the first subframe is randomly selected from OFDM symbol 0 and OFDM symbol 7 with equal probability. Otherwise, the starting position for the first subframe is OFDM symbol 0.

If the higher layer provides “Subframe configuration for LAA”, the configuration of occupied OFDM symbols in the last subframe is randomly selected from configuration set S2. S2 is given per test case. Otherwise, the subframe is set as fully occupied.

A uniform random variable from [0, 1] is generated. If the random variable is less than p which is given per test case,

    - If both the last subframe of previous burst and first subframe of new burst format are fully occupied, start burst transmission after deferring one subframe and the transmission is according to the determined burst format.

    - Otherwise, the transmission is started from the latest start symbol determined from the determined burst format and the transmission is according to the determined burst format.

Otherwise, the burst transmission is muted and the muting duration is the same as the number of subframes for determined burst format.

-----------------------------------------------------------------End of Change ------------------------------------------------------------