3GPP TSG-RAN WG4 #80	R4-166528

Goteborg, Sweden, 22th – 26th August 2016

Source: 	Ericsson

Title:  	TP for NR Rel-14 TR 38.803: Simulation assumptions and scenarios

Agenda Item:	10.3.1

Document for:	Approval

# Introduction

The draft skeleton of technical report of NR studies on RF and coexistence aspects has been approved in RAN4#79 in [1].

In this contribution, we propose texts related to simulations assumptions and scenarios for TR 38.803.

# Text proposal

The following text proposal is related to Section 5.

<<<<< START of TEXT PROPSOAL >>>>>

# 5	Co-existence study

## 5.1	Co-existence simulation scenario

Editor’s note: intended to capture specific scenarios such as operation system (eMBB, Massive MTC etc) systems in terms of aggressor system, victim system and their directions

## 5.2	Co-existence simulation assumption

Editor’s note: intended to capture specific simulation parameters such as frequencies, antenna pattern cell layouts and so on.

### 5.2.1 Spectrum considerations

In order to include all of mmwave spectrum ranges and also frequencies below 6GHz, the following spectrum ranges and frequencies are considered for the coexistence evaluations as described in the table below. The three frequencies 30, 45 and 70 GHz are seen as representative of the different bands to be considered for sharing and compatibility studies by ITU-R in the frequency ranges 24.24 to 33.4 GHz, 37 to 52.6 GHz and 66 to 86 GHz respectively. The table lists also the deployment scenarios that are considered for the corresponding spectrum.

| Spectrum | Urban macro | Dense urban micro | Indoor hotspot |
| --- | --- | --- | --- |
| Below 6GHz | Yes | Yes | Yes |
| 30GHz | Yes | Yes | Yes |
| 45GHz |  | Yes | Yes |
| 70GHz |  | Yes | Yes |

### 5.2.2 Deployment layouts

#### 5.2.2.1 Urban macro deployment

For macro deployment, both coordinated/collected deployments were studied in addition to uncoordinated/non-collocated deployment, as shown in the figure below.

| Coordinated Operations: identical cell layouts for each network with co-location of sites | Uncoordinated Operations: identical cell layouts for each network with worst case shift between sites: Aggressor network’s sites are located at the first network´s cell edge |
| --- | --- |
| Definition of grid shift for multi-operator deployment based on macro deployment in TR36.942/25.942 |  |

#### 5.2.2.2 Dense urban micro deployment

For dense urban micro deployment, the following deployment is studied. This deployment considers 3 or 9 micro cells deployed inside the macro cell area.

Figure 5.2.2.2-1: Layout illustration of deployment scenario of “Dense Urban”: 3 Microcells per Macro cell

Figure 5.2.2.2-2: Layout illustration of deployment scenario of “Dense Urban”: 9 Microcells per Macro cell

Table 5.2.2.2-1: Number of Microcells per Macro cell  vs. minimum distance between cells and UE cluster radius

| Number of the micro TRPs per macro TRP | Minimum distance between Micro TRPs (m) | Radius of UE dropping within a cluster: R (m) |
| --- | --- | --- |
| 3 | 57.9 | <28.9 |
| 6 | 42.4 | <21.2 |
| 9 | 32 | <16 |

#### 5.2.2.3 Indoor hotspot deployment

For indoor hotspot deployment, ITU indoor hotspot model is used, as shown below.

In the above deployment scenario, two operators deploy 4 indoor cells each in the single-floor building.  The indoor cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random, however can be anything higher than a minimum distance, e.g. 3m. The set of indoor cells for both operators is centered along the longer dimension of the building.

## 5.3	Co-existence simulation methodology

Editor’s note: intended to capture specific simulation methodology. How to handle even wider channel bandwidths for NR than those for LTE-A, different RF parameters such as [TRP or EIRP or others] etc. are captured.

## 5.4	Co-existence simulation results

Editor’s note: intended to capture the results.

## 5.5	Summary of co-existence study

Editor’s note: intended to capture the observation from the results.

<<<<< END of TEXT PROPSOAL >>>>>

# Conclusion

We propose to adopt the above mentioned text proposal for TR 38.803.

# References

R4-164535, Draft TR skeleton for New Radio SI (TR 38.803), 38.803 v0.0.1, NTT DOCOMO, INC.