3GPP TSG RAN4 Meeting #80bis                                                                       R4-167263

# Ljubljana, Slovenia, 10 – 14 October, 2016

Agenda Item:	10.5.3.1

Source: 	Intel Corporation

Title: 	PA model using a Memory Polynomial

Document for:	Approval

- Introduction

PA model discussions regarding NR systems have been carried out in previous RAN4 meetings. Further studies of the PA models with memory effects has been included as part of the discussion as a WF [1]. In this contribution, we introduce the memory polynomial to model the nonlinearities as a more attractive approach compared to the Volterra Series computation. Additionally, the algorithm for the coefficients calculation is explained in further detail and the results using the algorithm for a model below 6 GHz and above 6 GHz are shown.

- Discussion

# 2.1 Memory Polynomial

The distortions at the output of a PA are not only a result of its nonlinearity, but also from the memory effects. Memory effects and nonlinear distortion can significantly reduce the output signal quality and therefore, degrade the overall system performance. Depending on the frequency range of the signal transmission, the inclusion of memory effects can be crucial for developing a useable and realistic model. Therefore, the models of a PA operating below 6 GHz and another PA operating above 6 GHz will be shown within this contribution.

The described models utilizes envelope signals at input and output of the PA. The kind of behavioral model depends on the distortion domain. Envelope domain has less physical sense and is more mathematical. This domain is especially suitable for modulated signal because it simplifies the extraction of model parameters in the time domain.

A well-known polynomial used to describe nonlinear models is the Volterra series, which includes memory effects, which can be described with the following equation,

yn=m=1Mk=1Kamkxn-m|x[n-m]| k-1             (1)

where K is the highest nonlinear order number, M is the memory length, and amk is the polynomial coefficient. The disadvantage of Volterra series is its high complexity due to the elevated number of parameters, which makes it highly impractical for a model implementation, especially for wide bandwidth applications. Furthermore, the number of parameters to be defined, increases with the degree of nonlinearity and memory length of the system. Therefore, a Memory Polynomial (MP) model is proposed here.

The MP model is a special case of Volterra series, where the diagonal terms are retains. In other words it is a simpler Volterra series version that reduces the number of parameters to be determined. MP decreases the overall system complexity while still maintaining the accuracy of the memory effects description. Before modelling, input and output samples must be aligned. Signal correlation is the most common method to synchronize time series. After alignment and normalization, a part of the samples is used to calculate the coefficients of matrix A in equation (2).

Y=X*A                                         (2)

The coefficients can be extracted using the LMS algorithm. The LMS algorithm is one of the most used algorithm in adaptive filtering for several reasons. The main features are low computational complexity and stable behavior when implemented with finite-precision arithmetic. [2]

The low computational complexity of this algorithm enables the use of higher M and K terms for calculating the matrix coefficients. The selection of a higher memory depth degree and higher nonlinear order for the calculation (especially for the model working at high frequencies) results in a more accurate prediction of the matrix coefficients. The LMS simplifies the gradient vector computation when modifying the output function in an appropriate way. The adaptive filter models iteratively the relationship between the input and the output signal and can self-adjust the coefficients according to the adaptive algorithm shown in Table 1.

The LMS algorithm can utilize as input complex signals and the objective of the algorithm is to minimize the instantaneous square error|ek|2. The updating equation for the expression of e will be based on the difference of the desired output y and the estimated output  yest, which is equal to the vector of filter weight multiplied by the input signal. The weighted vector will be adaptively updated as can be seen in the second equation in Table 1. The coefficient amk is multiplied by a leakage factor σ, this value can be chosen between 0 and 1 and this factor will determine how substantial the relationship of the next calculated coefficient to the previous one is.

Table 1. Complex LMS Algorithm

Complex LMS Algorithm

Initialization x0=amk0=[0 0…0]T Do for k ≥ 0 ek=yk-amkHkx(k) amkk+1=σamkk-μce*kx(k)

The LMS algorithm performance uses above equations to calculate the matrix coefficients of A, where x(k) denotes the input signal, y(k) is the corresponding output signal, σ is the leakage factor, μc is the convergence factor, amk are the matrix coefficients and ek is the error signal, which is equal to the difference between the estimated output and the output of the PA.

# 2.2 PA model below 6 GHz

The PA model was simulated in Matlab using the measurement results from an Intel PA. An OFDM signal with a carrier frequency of 2.44 GHz was fed to the amplifier. Both the I(t) and Q(t) from the input and output signals PA were used to develop a model based on a memory polynomial implementation. The terms used for the evaluation of this model were polynomial degree K with a value of 5 and the polynomial memory depth M with a value of 5. The low value of both terms enabled a fast computational time and ensured good adaptive performance of the algorithm, which resulted in a very near approximation of the modelled output signal to the original output signal.

Figure 1. Power Spectral Density at 2.44 GHz

Figure 2. Input Signal VS Output Signal at 2.44 GHz

This can be perceived in Figure 1 and Figure 2 where the Power Spectral Density (PSD) and the input versus the output signal have been plotted based on the number of samples of the measured PA fitted to the simulation model. In both figures the red curves represent the model estimated data and the blue curve the original measurement data. We can find the two traces of PSD fit very well in pass band and adjacent channel. The average error in dB between value of PSD of measured data and that of model output is less than 0.3 dB.

To evaluate the performance of the MP model, the average normalized mean square error (NMSE) has been calculated, which is the most common metric to evaluate models performance. The NMSE is calculated with following equation, where y represents the measured data and yest represents the model data.

NMSE =10*log| y-yest|2 | y|2

The calculated value for the NMSE is -67 dB which is much better than the traditional NMSE values of around -38 dB. In Figure 2 it can be seen that the memory effects generate diffusion region and based on these results we propose the use of a memory polynomial model to implement the PA model below 6 GHz considering the memory effects.

# 2.3 PA model above 6 GHz

Figure 3. Power Spectral Density at 31 GHz.

The memory polynomial has been utilized as well for above 6 GHz. The PA model for above 6 GHz was implemented by using the 28nm CMOS PA at 31 GHz designed by the Katholieke Universiteit Leuven (KUL) [2]. A 16-QAM Single Carrier signal was fed to the amplifier. Since we consider that a PA operated in mmWave bands shall support a wider bandwidth compared to a PA below 6 GHz, the memory effects are expected to be more crucial. By this, we see an increase of the complexity to model the nonlinearities. In other words, the calculation effort will be more costly than in the case of the PA below 6 GHz, where the memory effects are not so dominant.

Figure 4. Input Signal VS Output Signal at 31 GHz.

The polynomial degree K used for this PA was 8 and polynomial memory depth M value was 5. The memory polynomial allows the use of high K and M terms for calculating the matrix coefficients but with a less computer complexity compared to Volterra series.

In Figure 3 and Figure 4 the Power Spectral Density (PSD) and the input versus the output signal have been plotted for the 31 GHz PA. The PSD shows more spectral regrowth compared to the PA below 6 GHz. In both figures the red curves represent the model estimated data and the blue curve the original measurement data. We can find the two traces of PSD fit also very well in pass band and adjacent channel. The average error in dB between value of PSD of measured data and that of model output is less than 1 dB. The calculated value of NMSE for this PA model is – 39 dB.

In Figure 4 it can be seen when including the memory effects that the characteristic of the PA behave no longer as a curve but rather as a diffusion region. These memory effects increase the calculation time to estimate the matrix coefficients of the memory polynomial and accurately model the PA.

- Conclusion

The high computational complexity of the Volterra series becomes not acceptable for a practical implementation as the nonlinearity level increases and memory effects become more dominant. The contribution shows that the MP model can efficiently capture dynamic characteristics of both power amplifiers for low frequencies as well as for high frequencies overcoming the complexity of a model implemented with Volterra series.

Proposal: The memory effects should be considered to model the PA for below 6 GHz and above 6 GHz. For the implementation of the model a memory polynomial is proposed.

- Acknowledgment

Part of this work was conducted within the Flex5Gware Project, a project supported by the European

Commission in the framework of the H2020-ICT-2014-2 project “Flex5Gware” (Grant agreement no.671563)

Special thanks to the project partner Katholieke Universiteit Leuven (KUL) for providing measurement data based on their designs.

- Reference

- R4-166730, “Way Forward for PA model investigation for NR” Ericsson, 3GPP RAN4 #80, August 2016

- Diniz, Paulo SR. Adaptive filtering. Boston, MA: Kluwer, 1997

- Flex5Gware Project, “Deliverable 2.1: Requirements and concepts for the analogue HW in 5G mobile systems”, Dec. 2015.