3GPP TSG RAN WG4 Meeting #80 BIS		R4-167348

Ljubljana, Slovenia 10th - 14th October 2016

Source:	Verizon

Title:	BHH test results for LTE

Agenda Item:	8.13.1

Document for:	Discussion and Decision

# Introduction

In RAN4#80 Gothenburg “Way forward on LTE TRP TRS framework improvement and work plan” was agreed [1]. This way forward stated a group of bands, including Band 2, 4, 5 and 13, for TRP TRS requirements with priority and suggested further collection of measurement data at RAN#80BIT for  purpose of framework improvement and decision on TRP and TRS requirements.

This paper presents our measurement results on LTE TRP and TRS BHH for Band 4 and 13 to input to RAN4 for building requirements.

# Test conditions

Test condition of the measurement is shown in Table 1. There were 3 DUTs measured in the condition of free space and beside head with hand right (BHHR) to confirm the values of LTE TRP and TIS. These DUTs were commercial devices selected from three different vendors. BHHR results were created from each individual device on the measurements. LTE TRP and TIS were measured per the CTIA OTA Test Plan [2], i.e., TIS was measured independently for the primary and secondary/diversity antennas [CTIA]. The results of BHHR are averaged across each operating band tested.

| Frequency Band | Measurement Setting | Number of DUT |
| --- | --- | --- |
| LTE Band 4, 13 | Free space, BHHR | 3 |

For resulting BHHR values, additional information of antenna test scenario and LTE frequency band is presented below.

Test scenario:

Head and Hand Right (BHHR):

Relevant for devices do not exceed the maximum dimensions specified for hand phantom (i.e.72 mm) [CTIA]. The values are defined considering head and hand and are relevant for right hand in this contribution.

Free Space:

Relevant for any measured device embeds at least two antennas. The applicable measurement method is data (and voice) mode(s). These measurement values include measurement uncertainty.

Settings during testing: Per the CTIA OTA Test Plan [2]

# Discussion

Measurement results from different commercial devices were taken in BHHR position. For LTE TRP and TIS, we measured three DUT’s. The TRP and TIS results are presented in the following Figures.

  - TRP BHHR CDF

Figure 3-1. Band 13 LTE TRP BHHR CDF

Figure 3-2. Band 4 LTE TRP BHHR CDF

  - TIS BHHR CDF

TIS results were measured by primary and secondary receiver antenna respectively.

Figure 3-3. Primary Band 13 LTE TIS BHHR CDF

Figure 3-4. Primary Band 4 LTE TIS BHHR CDF

Figure 3-5. Secondary Band 13 LTE TIS BHHR CDF

Figure 3-6. Secondary Band 4 LTE TIS BHHR CDF

# Summary

Table 4-1 is a summary of test TRP/TIS results. Average value here is to be an average of three commercial devices with three time measurements on each individual device.

Table 4-1 Test TRP/TIS Results

| TRP Results (dBm) |  |
| --- | --- |
| Band | Average value |
| LTE B13 | 11.34945 |
| LTE B4 | 14.59086 |
| TIS Results (dBm) |  |
| Primary B13 | -86.3553 |
| Primary B4 | -89.2807 |
| Secondary B13 | -82.2185 |
| Secondary B4 | -86.5707 |

# Conclusion

In this contribution, TRP/TIS with three commercial devices are evaluated and LTE BHHR radiated test results are presented in order to define specification requirements for LTE device. Per the results of primary receiver TIS measurements and assuming at least 1 dB of diversity gain, the values for TRS BHH should be -86.0 dBm for Band 13 and -90.0 for Band 4.

Per the TRP measurements, we suggest the values of 9.40 dBm for Band 13 and 13.25 dBm for Band 4 which are similar to those proposed in [3].

Further, these values for Band 13 and Band 4 TRS should be extended to Band 5 and Band 2, respectively.

# Reference

- R4-167201, “Way forward on LTE TRP TRS framework improvement and work plan”, Vodafone, Telecom Italia, 3GPP TSG-RAN WG4 Meeting #80  Goteborg, Sweden, 22 – 26 August, 2016

- CTIA Test Plan for Wireless Device Over the Air Performance, v3.6

- R4-166224, “ E-UTRA BHH TRP/TRS performance requirements”,  Motorola Mobility LLC, SONY Mobile, 3GPP RAN4 Meeting #80, Gothenburg, Sweden, August 22-26 2016