3GPP TSG-RAN WG4 RAN4 #80bis                                       R4-167366

Ljubljana, Slovenia, 10 - 14 Oct, 2016

Agenda item:			9.3.1

Source:			Samsung

Title:	TP for TR36.752: Coexistence simulation methodology

Document for:			Discussion

# 1	Introduction

The study item on NB-IoT RF requirement for coexistence with CDMA was approved at RAN #72 [1]. In RAN4 #80, the details of coexistence simulation methodology has been well discussed with the way forward agreed [2]. In this contribution, we provide a text proposal on the part of coexistence simulation methodology, which we believe is an important part to complete the coexistence study description in the final technical report TR36.752 [3].

# Reference

[1] 3GPP RP-161187, “WID: Study on NB-IoT RF requirement for coexistence with CDMA”, China Telecom.

[2] 3GPP R4-166994, “Way Forward on Simulation Assumption and Methodology for the coexistence study between CDMA and NB-IoT”, China Telecom, Samsung, Huawei, Qualcomm, Ericsson.

[3] R4-165273, “RAN4 TR skeleton for NB-IoT coexistence with CDMA”, China Telecom

# Text Proposal

<<< Start of text proposal for TR >>>

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	RP-161187, “Study on NB-IoT RF requirement for coexistence with CDMA”

[4]	3GPP TR 36.802: " Narrowband Internet of Things (NB-IoT); Technical Report for BS and UE radio transmission and reception (Release 13)".

<<< Unchanged sections omitted >>>

## 6.2	Coexistence simulation assumptions

<Text will be added>

## 6.3	Coexistence simulation methodolgoy

The following methodology for coexistence analysis will be used:

Step-1: RAN4 get agreement on RF performance assumptions for legacy CDMA system.

- The RF performance assumptions for legacy CDMA system should be determined firstly, e.g., ACLR/ACS performance for both CDMA UE and BS.

Step-2: Based upon operators’ input on the NB-IoT deployment scenarios, the carrier separation between NB-IoT and CDMA systems are determined.

- The carrier separation should be determined based on practical operators’ deployment needs.

Step-3: Conduct coexistence simulation based on agreed simulation assumptions, under certain agreed RF performance assumptions for legacy CDMA system and certain carrier separation options between NB-IoT and CDMA systems.

- After obtaining coexistence simulation results, RAN4 should compare the simulation results with Rel-13 NB-IoT RF requirement to determine it could be reused or not.

- If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements could be reused to guarantee the coexistence between NB-IoT and legacy system, go to step 4.

- If the aligned simulation results indicate that Rel-13 NB-IoT RF requirements is not enough to guarantee the coexistence between NB-IoT and legacy system, go to step 5.

Step-4: Conclusion can be reached that Rel-13 NB-IoT RF requirements could be reused to make sure a proper coexistence with CDMA system.

Step-5: Identify the necessity of introducing new NB-IoT RF requirement to make sure a proper coexistence with CDMA system.

Similar to the methodology utilized in Rel-13 coexistence study for NB-IoT [4], the evaluation metrics used in this coexistence study is

- For NB-IoT, SINR loss should be less or equal to 1dB

- For CDMA , Capacity loss should be less or equal to 5%

Similar to the methodology utilized in Rel-13 coexistence study for NB-IoT [4], the following calculation for ACLR/ACS modeling is applied in the coexistence study:

•	Definition and assumption

•	CDMA2000 ACLR and ACS are defined with adjacent channel measurement bandwidth 180kHz

•	NB-IoT ACLR and ACS are defined for its own bandwidth size

•	Attenuations are flat in the adjacent channels

•	ACLR and ACS are the inputs to the simulations

•	ACLR_e is the effective ACLR in the interference calculation

•	ACS_e is the effective ACS in the interference calculation

•	Interference from CDMA2000 to NB-IoT

•	ACLR_e = ACLRCDMA – 10*log10(1/(# of IoT users))

•	ACS_e = ACSNB-IoT [pessimistic assumption]

•	Interference from NB-IoT to CDMA2000

•	ACLR_e = ACLRNB-IoT – 10*log10(1.23MHz/180kHz) [pessimistic assumption]

•	ACS_e = ACSCDMA

## 6.4	Coexistence simulation results

<Text will be added>

<<< End of text proposal for TR >>>