3GPP TSG-RAN WG4 Meeting #80bis	R4-167378

# Ljubljana, Slovenia, 10 - 14 Oct, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.842 | CR | 0003 | rev | - | Current version: | 2.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional Aspects for the test method limitations of the near field EIS measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kathrein |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AAS_BS_LTE_UTRA-Perf |  |  |  |  |  |  | Date: |  | 2016-09-28 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The aspects to use a sniffer antenna is not covered. Sniffer antennas open possibilities to perform near field measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extension of chapter 10.3.2.1.4.2: Sniffer antenna is mentioned as additional aspect for the near field measurement - Small editorial corrections in 10.3.2.1.4.2 and 10.3.2.1.4.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Limitations of the near field EIS measurement is not described completely. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.1.4.2, 10.3.2.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X | N | Other core specifications |  |  |  |  |  |  |
| affected: |  | X | N | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  | X | N | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## --------------Start of text proposal------------

##### 10.3.2.1.4	Near Field Test range

###### 10.3.2.1.4.1	Description

The Near Field measurement techniques consist in measuring amplitude and phase of the modulated signal at the AAS BS under test, on some specific surfaces such as planar, cylindrical, and spherical. Each of the near field test methods can be implemented by one or more mechanical rotations of the probe and/or AAS under test as shown in Figure 10.3.2.1.4.1-1. One or more mechanical movement can be substitute by a probe array. All the scanning methods will need an RF transmit and receive system equipped with an automated scanning, a data collection and control system, and computerized analysis ability.

Figure 10.3.2.1.4.1-1: Probe/scanner near field system

In case the radiated field is sampled on a sphere surrounding completely the AAS under test, the 3D full sphere EIS values can be measured in Near Field when the AAS is receiving a defined modulated signal. The near field measurement techniques would imply the use of mathematical artefact, NF to FF transform in order to have the EIS value in the Far Field. The Near Field to Far Field transform is based on the well-known Huygens-Fresnel principle. The spherical modal wave expansion is the implicit application of the Huygens principle. A direct solution of the Helmholtz equations is found by applying boundary conditions on the surface S at infinity. From the tangential fields over the surface, the modal coefficients can be determined using the orthogonality of the modal expansion.  Based on this formulation, the Near Field sampling criteria does play an important role.

###### 10.3.2.1.4.2	Test Method limitations and scope

The Near Field Test Method is suitable for OTA sensitivity measurements of AAS BS implementations where the beam forming is done in a fixed or preset manner for the measured OSDD declaration, i.e. there is only one interface to digital/baseband processing. The near field test method is not suitable if for the measured OSDD declaration any combining is not fixed or preset. Some examples of architectures for which near field test method is possible and for which it is not possible are depicted on Figure 10.3.2.1.4.2-2. The AAS BS under test should not have any other non-linear behaviour that would cause the near field to far field transformation to fail or cause increased OTA sensitivity measurement uncertainty.

In order to determine the OTA sensitivity with the near field test method, it is necessary to perform a near field far field transformation. This requires power and phase information for each azimuth and elevation angle setting. As the AAS BS under test, usually made up of an active antenna system and a base station server, delivers only power values of the received signal a further measurement element is needed to receive the required phase information. One proposal to solve this problem using standard 3GPP interfaces only is implementing an additional sniffer antenna and calculating the phase via superimposing of two signals.

Figure 10.3.2.1.4.2-1:	Examples of limit of OSDD functionality testable with near field test method for a single OSDD

Figure 10.3.2.1.4.2-2:	Examples of limit of OSDD functionality not testable with near field test method for a single OSDD

10.3.2.1.4.3	Procedure

Calibration accounts for the various factors affecting the measurements of the EIS. These factors include components such as range length path loss, cable losses, gain of the receiving antenna, etc. Each measured data point for radiated sensitivity is transformed from a relative value in dB to an absolute value in dBm. For doing that the total path loss from the DUT to the measurement receiver, named L path loss is calibrated out. The calibration measurement is usually done by using a reference antenna with known gain. This approach is based on the so called gain-comparison method [13]. Figure 10.3.2.1.4.3-1 shows the typical configuration for measuring path loss.

Figure 10.3.2.1.4.3-1 Typical Lpath loss measurement configuration

The Lpath loss can be determined from the power into the reference antenna by adding the gain of the reference antenna:

Piso = Pref + GrefPIsotropic Radiator=Preference antenna+Greference antenna

so that:

Lpath loss = Pref + Gref - PtestLpath loss=Preference antenna+ Greference antenna- Ptest equipment

In order to determine Pref, a cable reference measurement is performed in order to calibrate out the A, and B pathsLtest equipment-receiver. Assuming that the power at the source is fixed, it can be showed that:

Pref-Ptes=Prec'-Prec

Where Prec Preceiver and Prec' are the power measured at the receiver during the calibration measurement with the reference antenna and the power measured at the receiver during the cable reference measurement respectively. Lpath loss Lpath loss is then given by:

Lpath loss = Gref + Prec' - Prec

Measurement

The testing procedure consists in the following steps:

1)	AAS BS near field radiation pattern measurement: this is performed when the AAS will be receiving a defined modulated signal, as defined in appropriate conformance test specification.

2)	AAS BS near field Sensitivity Measurement: this is performed by measuring the sensitivity in near field when a modulated signal is received by the AAS BS under test,

3)	AAS BS near field to far field EIS transformation: sensitivity measurement is applied.

The near field to far field transformation is a mathematical computation which is applied to the near field measured radiation pattern in order to compute the far field radiation pattern. It is typically performed expanding the measured near field over a set of orthogonal basis functions. The near to far field transform is then performed in two steps:

1)	Expansion (or projection) of the measured near field (i.e. Emeas(r)) over a set of orthogonal basis functions (i.e. Fbasis(r)) in order to evaluate the transformed spectrum:

Emeas(r) = Spectrum * Fbasis(r)

2)	FF (i.e. EFF) computation using a the previuosly calculated spectrum and with the basis functions evaluated at r∞ (i.e. Fbasis(r  ∞)):

EFF = Spectrum * Fbasis(r  ∞)

When performing the near field to far field transformation, the sensitivity measurement is applied so that the near field pattern will be transformed from relative power (i.e. dB) to absolute power (i.e. dBm). In this specific case, the EIS far field pattern is expressed in terms of the absolute power received by the AAS BS.

1)	AAS BS OTA Sensitivity: once the full 3D far field EIS pattern has been computed, the OTA sensitivity can be derived.

10.3.2.1.4.4	Uncertainty budget format

Table 10.3.2.1.4.4-1: Near Field Test Range Uncertainty contributors in AAS EIS measurement

| UID | Description of uncertainty contribution | Details in paragraph |
| --- | --- | --- |
|  | Stage 1, EIS Near Field Radiation Pattern Measurement and EIS Near Field DUT power measurement |  |
| 1 | Axes Intersection | C.4.1 |
| 2 | Axes Orthogonality | C.4.2 |
| 3 | Horizontal Pointing | C.4.3 |
| 4 | Probe Vertical Position | C.4.4 |
| 5 | Probe H/V pointing | C.4.5 |
| 6 | Measurement Distance | C.4.6 |
| 7 | Amplitude and Phase Drift | C.4.7 |
| 8 | Amplitude and Phase Noise | C.4.8 |
| 9 | Leakage and Crosstalk | C.4.9 |
| 10 | Amplitude Non-Linearity | C.4.10 |
| 11 | Amplitude and Phase Shift in rotary joints | C.4.11 |
| 12 | Channel Balance Amplitude and Phase | C.4.12 |
| 13 | Probe Polarization Amplitude and Phase | C.4.13 |
| 14 | Probe Pattern Knowledge | C.4.14 |
| 15 | Multiple Reflections | C.4.15 |
| 16 | Room Scattering | C.4.16 |
| 17 | DUT support Scattering | C.4.17 |
| 18 | Scan Area Truncation | C.4.18 |
| 19 | Sampling Point Offset | C.4.19 |
| 20 | Spherical Mode Truncation | C.4.20 |
| 21 | Positioning | C.4.21 |
| 22 | Probe Array Uniformity | C.4.22 |
| 23 | Mismatch of transmitter chain (i.e. between receiving antenna and vector signal generator) | C.4.23 |
| 24 | Insertion loss of transmitter chain | C.4.24 |
| 25 | Uncertainty of the absolute gain of the probe antenna | C.4.25 |
| 26 | Vector Signal Generator a) uncertainty of the absolute power level b) stability c) stability over temperature d) linearity | C.4.26 |
| 27 | Measurement repeatability - Positioning Repeatability | C.4.27 |
|  | Stage 2, Calibration measurement |  |
| 28 | Uncertainty of network analyser a)drift (temp, oscillators, filters, etc.) start to end time of measurements | C.4.28 |
| 29 | Mismatch of transmitter chain | C.4.29 |
| 30 | Insertion loss of transmitter chain | C.4.30 |
| 31 | Mismatch in the connection of the calibration antenna | C.4.31 |
| 32 | Influence of the calibration antenna feed cable | C.4.32 |
| 33 | Influence of the probe antenna cable | C.4.33 |
| 34 | Uncertainty of the absolute gain of the calibration antenna | C.4.34 |
| 35 | Short term repeatability | C.4.35 |
|  | Expanded uncertainty (2σ - confidence interval of 95 %) |  |

10.3.2.1.4.5	Uncertainty assessment

Table 10.3.2.1.4.5-1: Near Field Test Range uncertainty assessment for EIS measurement

| UID | Uncertainty source | Uncertainty value f ≦ 3GHz | Uncertainty value 3GHz ≦ f < 4.2 GHz | Distribution of the probability | Divisor based on distribution shape | Ci* | Standard uncertainty (σ) [dB] f ≦ 3GHz | Standard uncertainty (σ) [dB] 3GHz ≦ f < 4.2 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage 1: DUT measurement |  |  |  |  |  |  |  |  |
| 1 | Axes Intersection | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 2 | Axes Orthogonality | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 3 | Horizontal Pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 4 | Probe Vertical Position | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 5 | Probe H/V pointing | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 6 | Measurement Distance | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 7 | Amplitude and Phase Drift | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 8 | Amplitude and Phase Noise | 0.02 | 0.02 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 9 | Leakage and Crosstalk | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 10 | Amplitude Non-Linearity | 0.04 | 0.04 | Gaussian | 1.00 | 1 | 0.04 | 0.04 |
| 11 | Amplitude and Phase Shift in rotary joints | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 12 | Channel Balance Amplitude and Phase | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 13 | Probe Polarization Amplitude and Phase | 0.0001 | 0.0001 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 14 | Probe Pattern Knowledge | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 15 | Multiple Reflections | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 16 | Room Scattering | 0.09 | 0.09 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| 17 | DUT support Scattering | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 18 | Scan Area Truncation | 0.003 | 0.003 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 19 | Sampling Point Offset | 0.0058 | 0.0058 | Gaussian | 1.00 | 1 | 0.01 | 0.01 |
| 20 | Spherical Mode Truncation | 0.015 | 0.015 | Gaussian | 1.00 | 1 | 0.02 | 0.02 |
| 21 | Positioning | 0.03 | 0.03 | Rectangular | 1.73 | 1 | 0.02 | 0.02 |
| 22 | Probe Array Uniformity | 0.055 | 0.055 | Gaussian | 1.00 | 1 | 0.06 | 0.06 |
| 23 | Mismatch of transmitter chain | 0.284 | 0.284 | U-Shaped | 1.41 | 1 | 0.20 | 0.20 |
| 24 | Insertion loss of transmitter chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 25 | Uncertainty of the absolute gain of the probe antenna | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 26 | Vector Signal Generator | 0.46 | 0.46 | Gaussian | 1.00 | 1 | 0.46 | 0.46 |
| 27 | Measurement repeatability - positioning repeatability | 0.15 | 0.15 | Gaussian | 1.00 | 1 | 0.15 | 0.15 |
| Stage 2: Calibration measurement |  |  |  |  |  |  |  |  |
| 28 | Uncertainty of network analyser | 0.13 | 0.20 | Gaussian | 1.00 | 1 | 0.13 | 0.20 |
| 29 | Mismatch of transmitter chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 30 | Insertion loss of transmitter chain | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 31 | Mismatch in the connection of the calibration antenna | 0.02 | 0.02 | U-Shaped | 1.41 | 1 | 0.01 | 0.01 |
| 32 | Influence of the calibration antenna feed cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 33 | Influence of the probe antenna cable | 0 | 0 | Gaussian | 1.00 | 1 | 0.00 | 0.00 |
| 34 | Reference antenna | 0.50 | 0.25 | Rectangular | 1.73 | 1 | 0.29 | 0.25 |
| 35 | Short term repeatability | 0.088 | 0.088 | Gaussian | 1.00 | 1 | 0.09 | 0.09 |
| Combined standard uncertainty (1σ) [dB] |  |  |  |  |  |  | 0.63 | 0.63 |
| Expanded uncertainty (1.96σ - confidence interval of 95 %) [dB] |  |  |  |  |  |  | 1.24 | 1.24 |

NOTE:	Standard uncertainty values for the signal generator, network analyzer and reference antenna are according to the test equipment vendors proposal, as captured in annex E.