3GPP TSG-RAN WG4 #80bis	R4-167753

October 10th ‒ 14th, 2016

Ljubljana, SI

Agenda item:	10.3.1

Source: 	Qualcomm Incorporated

Title: 	Simulation results for NR coexistence study in Urban Macro scenario: uplink case

Document for:	Discussion

# Introduction

RAN4 is currently working on the adjacent channel coexistence study to determine the main RF parameters for NR in millimiter wave (mmW) scenarios. In previous RAN4 meeting (RAN4 #80), very good progress was made about the simulation assumptions to be adopted in the coexistence study [1]. In this paper we provide a collection of simulation results for the Urban Macro (UMa) scenario. The Up Link (UL) case is analysed.

# Discussion

The simulation assumptions adopted in this paper are the ones agreed in [1][2][3]. Regarding the power control, we use the same scheme as the one specified in TR 36.942, with the following parameters:

- γ=1

- CLxile=90dB

A detailed analysis of UL simulation scenario and power control setting is provided in [4].

Figure 1 shows the coupling loss CDFs obtained considering different configurations, in particular:

- No antenna gain at both UE and BS

- Element gain (EG) only at UE

- Element gain only (EG) at BS

- Element gain only (EG) at UE and BS

- Element gain only (EG) at UE and BS + BS beamforming

- Element gain only (EG) at UE and BS + BS beamforming + BS beamforming

As also explained in [5], an interesting observation is about the gap between green curve and the dashed black curve. Looking at the median value, this gap is about 24dB and represents the gain due to the base station beamforming. Given the agreed assumption, we have 8x16 elements with 2 polarizations (which is emulated by additional 3dB gain), this brings to: 10*log10(8*16)+3 ~24dB. The last (right side) black curve brings additional 6dB, which is due to the 2x2 elements at the UE.

For the same antenna configurations, SIR (signal to co-channel interference) and SINR (signal to co-channel interference plus noise) curves are shown in Figure 2 and Figure 3.

Finally, Figure 4 shows the UL mean throughput degradation due to adjacent channel interference (ACI) as a function of ACIR. To obtain this data, we consider beam forming at both BS and UE side, i.e. exactly the same configuration as agreed in [1]. As it can be observed, in this scenario an ACIR of 15dB would allow to keep the mean throughput degradation due to ACI within 5%. It is also worth noticing that for this particular scenario the 5%-tile throughput degradation is not a useful metric because outage percentage is more than 5% (this can be seen in Figure 3, where the 5%-tile of SINR distribution has less than -10dB, i.e. 0 throughput in the simulation).

As already observed for the DL case in [5], the message we get from Figure 4 is clear, i.e. the ballpark number for ACIR is much lower compared to legacy LTE operating below 6GHz. In particular for the simulated scenario we do not observe significant throughput degradation for the simulated ACIR range.

Figure 1. Coupling loss CDFs for UMa scenario.

Figure 2. SIR CDFs for UMa scenario.

Figure 3. SINR CDFs for UMa scenario.

Figure 4. UL mean throughput degradation due to adjacent channel interference in UMa scenario.

# Conclusions

In this contribution we provided simulation results for NR coexistence study in Urban Macro scenario. The assumptions adopted in the study are the ones agreed in [1]. Our data show that, for the simulated scenario, the interference due to adjacent channel interference is negligible even in case of very low ACIR.

# Reference

- R4-167079, “WF on Simulation assumptions of Co-existence study for WP5D”, NTT DOCOMO INC.

- R4-167123, “WF on UE Beamforming Model for the ITU WP5D coexistence simulations”, Qualcomm Incorporated.

- R4-167127, “Way forward on BS Beamforming Model for the ITU WP5D coexistence simulations”, Huawei, HiSilicon.

- R4-167751, “On the Uplink power control for NR coexistence study”, Qualcomm Incorporated.

- R4-167753,	“Simulation results for NR coexistence study in Urban Macro scenario: uplink case”,  Qualcomm Incorporated