3GPP TSG RAN WG4 Meeting #81	R4-1609276

Reno, USA, 14 – 18 NOV 2016

Agenda Item:	8.25.3.1.2

Source: 	Nokia, Alcatel-Lucent Shanghai Bell

Title: 	Updated eMTC RSTD Simulation Results

Document for:	Discussion

# Introduction

Positioning is important for supporting many eMTC applications. In Rel-13, only limited positioning functionalities are supported by eMTC UEs [1]. For Rel-14, the new WI on Further Enhanced MTC includes following objective to improve eMTC positioning [2]:

Positioning [RAN4, RAN1]

- E-CID: RSRP/RSRQ measurement

- E-CID: UE Rx-Tx time difference measurement

- OTDOA: core requirements

- OTDOA: consider improvements of accuracy, UE complexity and power consumption

In RAN4#80, OTDOA RSTD measurement performance with Rel-13 specifications was discussed in [3]. In addition, simulation assumptions for investigating eMTC RSTD performance was approved [4]. In [5], we provided initial simulation results with AWGN channel based on the simulation assumptions. In this paper, we further provide simulation results with fading channels based on the simulation assumptions.

# Simulation Assumptions

Simulation assumptions for investigating eMTC RSTD performance was approved [4]. The simulation scenarios and assumptions in this paper are shown in the following Table 1.

Table 1. Simulation assumptions for RSTD measurement

| Parameter |  | Value |
| --- | --- | --- |
| Cell layout |  | - 3 cells at distinct locations - The distances between each cell and target UE are identical - Same ISD between cells |
| Cell ID scenarios |  | (0, 1, 2), (0, 6, 12) (baseline) |
| Network synchronization |  | - Synchronous with time shifts <0,0, CP/2> |
| Duplex modes |  | FDD |
| Cyclic prefix |  | Normal |
| DRX |  | OFF |
| Carrier frequency |  | 2 GHz |
| Carrier bandwidth |  | 10 MHz |
| Channel model |  | EPA5, ETU30 |
| SINR for three cells, [dB] |  | (Reference cell, neighbour cell 1, neighbour cell 2) = (-6,-13,-13) |
| Number of transmit antennas | PRS | 1 |
|  | CRS | 1 |
| Number of receive antennas |  | 1 |
| MPDSCH transmission |  | No MPDSCH transmission in PRS transmission occasions |
| Number of consecutive positioning subframes |  | 6 |
| Number of positioning occasions for a positioning fix |  | 1 |
| PRS pattern |  | 6-reuse in frequency, vshift = mod(PCI,6) |
| PRS transmission bandwidth |  | 10 MHz |
| Measurement bandwidth |  | 6 RB, 25RB (Note: 24RB was used in [4].) |
| Sampling errors |  | Random number in half of sampling intervals |

# eMTC RSTD Simulation Results

In this section, the simulation results are provided for FDD system with SNR=(-6,-13,-13) for both 1.4MHz and 5MHz eMTC systems. The number of consecutive PRS subframes for a RSTD measurement is 6 for all simulations.

Figures 1 - 4 show the simulation results with EPA5 channel for the following scenarios:

- Case 1: 1.4MHz, Cell IDs(0, 6, 12) , 6PRS SFs, EPA5

- Case 2: 1.4MHz, Cell IDs(0, 1, 2) , 6PRS SFs, EPA5

- Case 3: 5MHz, Cell IDs(0, 6, 12) , 6PRS SFs, EPA5

- Case 4: 5MHz, Cell IDs(0, 1, 2) , 6PRS SFs, EPA5

Figures 5- 8 show the simulation results with ETU30 channel for the following scenarios:

- Case 5: 1.4MHz, Cell IDs(0, 6, 12) , 6PRS SFs, ETU30

- Case 6: 1.4MHz, Cell IDs(0, 1, 2) , 6PRS SFs, ETU30

- Case 7: 5MHz, Cell IDs(0, 6, 12) , 6PRS SFs, ETU30

- Case 8: 5MHz, Cell IDs(0, 1, 2) , 6PRS SFs, ETU30

Figure 1: 1.4MHz, Cell IDs(0, 6, 12), 6PRS SFs, EPA5

Figure 2: 1.4MHz, Cell IDs(0, 1, 2), 6PRS SFs, EPA5

Figure 3: 5MHz, Cell IDs(0, 6, 12), 6PRS SFs, EPA5

Figure 4: 5MHz, Cell IDs(0, 1, 2), 6PRS SFs, EPA5

Figure 5: 1.4MHz, Cell IDs(0, 6, 12), 6PRS SFs, ETU30

Figure 6: 1.4MHz, Cell IDs(0, 1, 2), 6PRS SFs, ETU30

Figure 7: 5MHz, Cell IDs(0, 6, 12), 6PRS SFs, ETU30

Figure 8: 5MHz, Cell IDs(0, 1, 2), 6PRS SFs, ETU30

# Conclusions

In this paper, we provided the eMTC RSTD simulation results for EPA5 and ETU30 channels according to the agreed simulation assumptions.

# References

- 3GPP TS 36.133

- RP-161464, “Revised WID for Further Enhanced MTC for LTE”, Ericsson

- R4-165458, “Discussion on eMTC OTDOA RSTD Measurements”, Nokia

- R4-166819, “Simulation assumption on RSTD”, Huawei, HiSilicon

- R4-168554, Initial eMTC RSTD Simulation Results, Nokia

- 3GPP TS 36.355, E-UTRA: LTE Positioning Protocol (LPP)