3GPP TSG-RAN WG4 Meeting #81	R4-1609519

Reno, USA, 14 - 18 November, 2016

Title: 	Impairment results of NPDSCH Demodulation for NB-IoT

Source: 	CATT

Agenda Item:	6.3.4.1.3

Document for:	Discussion

- Introduction

In RAN4#80bis Meeting, the Narrow Band IoT (NB-IoT) work item (WI) has reached a way forward [1]. Interested companies are encouraged to provide the simulation results including impairment error using the repetition numbers specified [2]. In this contribution, we provide our simulation results to show the throughput performance for NPDSCH.

- NPDSCH Simulation Assumptions and Results

For NPDSCH, five test cases have been agreed in RAN4 [2]. In this contribution, the main simulation parameters for Simulation Case 1-3 of In-band scenarios are accordingly configured as in Table 1 and Table 2.  And the main simulation parameters for Simulation Case4-5 of Standalone/Guard-band scenarios are accordingly configured as in Table 3 and Table 4. And the TBS table for NPDSCH is also given in Table 5.

Table 1: NPDSCH Minimum performance for In-band with 2 NRS ports

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 1 | 200kHz | Anchor | R.NB.5 FDD | 1 | EPA5 | 2x1 Low | 70% | TBD |
| 2 | 200kHz | Anchor | R.NB.5 FDD | 32 | EPA5 | 2x1 Low | 70% | TBD |
| 3 | 200kHz | Non-anchor | R.NB.5-1 FDD | 256 | ETU1 | 2x1 Low | 70% | TBD |

Table 2 NPDSCH Reference Channel with 2 TX Antennas

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.5 FDD | R.NB.5-1 FDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 2 | 10 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 4/0 | 4/0 |
| Target Coding Rate |  | 1/3 | 1/3 |
| Coding Rate |  | 0.4 | 0.4 |
| Information Bit Payload |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 | Bits | 56 | 56 |
| For Sub-Frame 0,5 | Bits | N/A | 56 |
| For Sub-Frame 4,9 | Bits | Note 3 | 56 |
| Number of Code Blocks |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 and 9(if NB-SSS is not present) |  | 1 | 1 |
| For Sub-Frame 0,5 | Bits | N/A | 1 |
| For Sub-Frame 4,9 | Bits | Note 4 | 1 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 and 9(if NB-SSS is not present) | Bits | 200 | 200 |
| For Sub-Frame 0,5 | Bits | N/A | 200 |
| For Sub-Frame 4,9 | Bits | Note 5 | 200 |
| Max. Throughput averaged over 2 frame for 1 repetition (Note 6) | Mbps | 36.4kb/s | 56kb/s |
| UE Category |  | NB1 | NB1 |
| Note 1: For in-band, the first 3 symbols are used for LTE PDCCH and the number of LTE CRS ports is 4 Note 2: 6 when mod 2 = 0, otherwise 8. is specified in TS36.211 [4]. Note 3: N/A when mod 2 = 0, otherwise 56. Note 4: N/A when mod 2 = 0, otherwise 1. Note 5: N/A when mod 2 = 0, otherwise 200. Note 6: For repetition number N, the maximum throughput averaged over 1 frame equals to (Max. Throughput averaged over 1 frame for 1 repetition) / N. |  |  |  |

Table 3: Minimum performance for NPDSCH under Standalone/Gurard-band with 1 NRS port

| Test number | Bandwidth | Carrer Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |
| 4 | 200kHz | Anchor | R.NB.6 FDD | 32 | EPA5 | 1 | 70% | TBD |
| 5 | 200kHz | Non-anchor | R.NB.6-1 FDD | 256 | ETU1 | 1 | 70% | TBD |

Table 4: NPDSCH Reference Channel with 1Tx Antenna

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.6 FDD | R.NB.6-1 FDD |
| Carrier Type |  | Anchor | Non-Anchor |
| Channel bandwidth | KHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | 10 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/2 | 1/3 |
| Coding Rate |  | 0.5 | 0.33 |
| Information Bit Payload |  |  |  |
| For a four continuous sub-frame group made up of Sub-Frames 1,2,3,6,7,8 and 9 (if NB-SSS is not present) | Bits | 616 | N/A |
| For a four continuous sub-frame group made up of Sub-Frames 0,1,2,3,4,5,6,7,8 and 9 |  | N/A | 392 |
| Transport block CRC | Bits | 24 | 24 |
| Number of Code Blocks |  |  |  |
| For a four continuous sub-frame group made up of Sub-Frames 1,2,3,6,7,8 and 9 (if NB-SSS is not present) |  | 1 | N/A |
| For a four continuous sub-frame group made up of Sub-Frames 0,1,2,3,4,5,6,7,8 and 9 | Bits | N/A | 1 |
| Available Binary Channel Bits per Sub-frame |  |  |  |
| For Sub-Frames 1,2,3,6,7,8 and 9(if NB-SSS is not present) | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 2 | 320 |
| Max. Throughput averaged over 1 frame for 1 repetition (Note 3) | Mbps | 100kbit/s | 98kbit/s |
| UE Category |  | NB1 | NB1 |
| Note 1: 6 when mod 2 = 0, otherwise 8. is specified in TS36.211 [4]. Note 2: N/A when mod 2 = 0, otherwise 320. Note 3: For repetition number N, the maximum throughput averaged over 1 frame equals to (Max. Throughput averaged over 1 frame for 1 repetition) / N. |  |  |  |

Table 5	TBS table for NPDSCH

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 |  |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 |  |  |
| 7 | 104 | 224 | 328 | 472 | 584 | 680 |  |  |
| 8 | 120 | 256 | 392 | 536 | 680 |  |  |  |
| 9 | 136 | 296 | 456 | 616 |  |  |  |  |
| 10 | 144 | 328 | 504 | 680 |  |  |  |  |
| 11 | 176 | 376 | 584 |  |  |  |  |  |
| 12 | 208 | 440 | 680 |  |  |  |  |  |

The Throughput vs. SNR for Simulation Case 1 is shown in Fig.1.

Fig.1. Throughput vs. SNR for Simulation Case 1

The Throughput vs. SNR for Simulation Case 2 is shown in Fig.2.

Fig.2. Throughput vs. SNR for Simulation Case 2

The Throughput vs. SNR for Simulation Case3 is shown in Fig.3.

Fig.3. Throughput vs. SNR for Simulation Case 3

The Throughput vs. SNR for Simulation Case 4 is shown in Fig.4.

Fig.4. Throughput vs. SNR for Simulation Case 4

The Throughput vs. SNR for Simulation Case 5 is shown in Fig.5.

Fig.5. Throughput vs. SNR for Simulation Case 5

Result summary table:

| Simulation Number | Carrier Type | Propagation Condition | Antenna Configuration | Repetition Number | Target SNR (dB) | Target SNR (dB) with impairment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Anchor | EPA5 | 2x1 Low | 1 | 1.7 | 4 |
| 2 | Anchor | EPA5 | 2x1 Low | 32 | -9.3 | -7 |
| 3 | Non-anchor | ETU1 | 2x1 Low | 256 | -14.1 | -11.8 |
| 4 | Anchor | EPA5 | 1x1 | 32 | -7.3 | -5 |
| 5 | Non-anchor | ETU1 | 1x1 | 256 | -13.9 | -11.6 |

- Conclusion

In this contribution, we provided simulation results to show the throughput performance for NPDSCH for different simulation cases.

Proposal: For Simulation Case 1 to 5, to achieve 70% of the maximum NPDSCH throughput, the corresponding SNR is around 4dB, -7dB,-11.8dB,-5dB and -11.6dB respectively.

- Reference

[1] R4-168936, “Way forward on NB-IoT UE demodulation requirements”, Ericsson, OCT 2016

[2] R4-168741, “Draft CR: Updates to NPDSCH demodulation requirements for NB-IoT (Rel-13)”, Huawei, HiSilicon, OCT 2016.