3GPP TSG-RAN WG4 #81	R4-1609630

November 14-18, 2016 Reno, USA

Agenda item:	8.13.1

Source: 	Sprint, AT&T, Verizon, SouthernLINC,

Title: 	LTE UE TRP and TRS Requirements for North American Bands

Document for:	Approval

# Introduction

At RAN4#80bis there was discussion on establishing TRP and TRS requirements for a limited set of E-UTRA Bands contained in the agreed way forward [1]. This paper seeks approval for TRP and TRS requirements for Bands 2,4,5,7,12,13,17,25,26,30,41 PC3,41 PC2, and 66.

# Discussion

US operators have an established a minimum set of TRP and TIS criteria used on their network deployment needs. These criteria apply to phones distributed by the North American carriers.

Most of the US based carriers use the TIA testing methodology [2] to evaluate TRP and TIS. The TRP results for both methodologies should be very similar. Table 2-1 shows the proposed TRP values based on composite of measured and calculated results.

Table 2-1 shows the proposed TRP values for various E-UTRA bands:

| Band | Duplex Mode | TX Power (dBm) | FS TRP | HL/HR TRP | BHHL/ BHHR TRP |
| --- | --- | --- | --- | --- | --- |
| 2 | FDD | 23 | 19.0 | 15.0 | 13.0 |
| 4 | FDD | 23 | 19.0 | 15.0 | 13.0 |
| 5 | FDD | 23 | 18.0 | 13.0 | 10.0 |
| 7 | FDD | 23 | 19.0 | 15.5 | 13.0 |
| 12 | FDD | 23 | 18.0 | 13.0 | 10.0 |
| 13 | FDD | 23 | 18.0 | 13.0 | 10.0 |
| 17 | FDD | 23 | 18.0 | 13.0 | 10.0 |
| 25 | FDD | 23 | 19.0 | 15.0 | 13.0 |
| 26 | FDD | 23 | 18.0 | 13.0 | 10.0 |
| 30 | FDD | 23 | 19.0 | 15.0 | 12.0 |
| 41 PC3 | TDD | 23 | 21.0 | 17.0 | 13.0 |
| 41 PC2 | TDD | 26 | 24.0 | 20.0 | 16.0 |
| 66 | FDD | 23 | 19.0 | 15.0 | 13.0 |

Proposal 1 : Approve the values in Table 2-1 as the minimum TRP requirements

There are significant differences between the way that CTIA measures TIS [2] and the way that 3GPP measures TRS [3].  TIS measures the receiver sensitivity on a single receiver chain with the other receiver chains disabled. Whereas 3GPP TRS values are measured with all of receiver chains active.  Logically the TIS values will be less stringent that TRS values.

Low Cost MTC applied a 2.5 dB delta to account for difference between dual receiver chains and a single receiver chain [4].  Similar values were proposed for NB-IOT at RAN4#80bis [5][6][7]. The 2.5 dB delta applies to conducted requirements, however TIS is an over the air receiver test so the values in Table 2-2 were adjusted by 2dB.

Table 2-2 contains the proposed TRS values based on current REFSENS requirements in 36.101 and measured data.

Table 2-2 TRS values for various E-UTRA bands

| Band | Duplex Mode | REFSENS (dBm) | Channel BW (MHz) | Free Space TRS | HL/HR TRS | BHHL/ BHHR TRS |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | FDD | -95 | 10 | -95.0 | -91.0 | -89.0 |
| 4 | FDD | -97 | 10 | -95.0 | -91.0 | -89.0 |
| 5 | FDD | -95 | 10 | -94.0 | -89.0 | -86.0 |
| 7 | FDD | -92 | 20 | -93.0 | -89.5 | -87.0 |
| 12 | FDD | -97 | 5 | -95.0 | -90.0 | -87.0 |
| 13 | FDD | -94 | 10 | -94.0 | -89.0 | -86.0 |
| 17 | FDD | -94 | 10 | -92.0 | -87.0 | -84.0 |
| 25 | FDD | -96.5 | 5 | -96.5 | -92.5 | -90.5 |
| 26 | FDD | -97.5 | 5 | -96.5 | -91.5 | -88.5 |
| 30 | FDD | -96 | 10 | -95.0 | -91.0 | -88.0 |
| 41 | TDD | -92 | 20 | -95.0 | -91.0 | -87.0 |
| 66 | FDD | -96.5 | 10 | -95.0 | -91.0 | -89.0 |

Proposal 2 : Approve the values in Table 2-2 as the minimum TRS requirements

Note that the proposed TRP and TRS requirements applies to Category 1 or higher device utilizing two antennas, UE’s that don’t support CA and is limited to devices with a width of 72 mm or less.

# For Approval

Proposal 1 : Approve the values in Table 2-1 as the minimum TRP requirements

Proposal 2 : Approve the values in Table 2-2 as the minimum TRS requirements

# Conclusion

This contribution discusses the TRP and TIS requirements used by US operator’s and it seeks RAN4 approval for Proposal 1 and  Proposal 2.

Proposal 1 : Approve the values in Table 2-1 as the minimum TRP requirements

Proposal 2 : Approve the values in Table 2-2 as the minimum TRS requirements

# Reference

[1] R4-167201	“WF on LTE TRP/TRS framework and work plan”	Vodafone

[2] CTIA Test Plan for Wireless Device Over-the-Air Performance, http://www.ctia.org/policy-initiatives/certification/certification-test-plans

[3] 3GPP TR 37.902 V13.0.0

[4] R4-147954	“Reference sensitivity WF for MTC”, Vodafone

[5] R4-168597, “Single Rx Category 1 RF scope”, 	Qualcomm Incorporated

[6] R4-168598, “	Single Rx Category 1 reference sensitivity”, 	Qualcomm Incorporated

- [7] R4-167738, “	RX requirements for new UE category with single receiver based on Category 1”, 	Sony