3GPP TSG-RAN WG4 #81	R4-1609833

Reno, USA, November 14 – 18, 2016

Agenda item:	8.27.2.1

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	CAT 1 Single Rx REFSENS definition

Document for:	Approval

# 1	Introduction

In last RAN a WI [1] was approved for specifying requirements for single receiver UE based on Category 1 for LTE. RAN 4 discussion started in RAN4#80bis [2] – [5]. Two proposals were agreed as a basis for the work

From [3] Proposal 1: Category 1 single Rx requirements are to be developed for full-duplex mode only.

From [4] Proposal 4:  Other than reference sensitivity and the indirect impact it has on other Rx requirements, all other RF requirements for LTE apply to a 1RxCat1 device.

Hence what RAN4 needs to decide is the REFSENS requirement for CAT 0 single Rx device and how to capture that into the specification.

# 2	Discussion

In last RAN4 meeting [4] discussed the REFSENS for CAT 1 single Rx UE and proposed that requirement should be initially set only to those bands that MTC Cat 0 has REFSENS defined. However already during the online discussion two operators raised interest to have the feature enable from the beginning also for bands 7 and 26 and some companies needed more time to think. It seems that it might be difficult to reach a conclusion in RAN4 which bands should be part of the WI. In our view it might be best to apply CAT 1 single Rx feature from the beginning to all LTE bands because the intended use case mentioned in the WID is wearables and as such all bands are potentially interesting bands. Also as the WI is for extension of current CAT 1 UE category also that would justify to include all bands from the beginning to the specifications whereas CAT 0 is a new feature and justifies gradual introduction of bands.

Proposal: CAT 1 single Rx requirements are defined to all existing E-UTRA bands

In [2] [4] it was proposed to follow MTC Cat 0 approach and define the single Rx REFSENS 2.5 dB higher that standard LTE requirement except for challenging bands which in case of Cat 0 are band 13 and band 20 (20 MHz BW) for which the delta is 3 dB. In addition to [2] in [4] it was proposed that for those channel bandwidths where REFSENS uplink allocation is > 36 RB the  delta should be 3 dB due to increased Tx noise (does not apply to TDD bands). In general we think approach in [4] is quite reasonable and below discuss how that could be captured into the specification.

## 2.1 New Table

It was proposed in [4] that a new Table is added for REFSENS and that LTE standard UL allocation Table is applicable also for CAT1 single Rx UE. Bands included into the new Table were same as currently applicable for CAT 0.

Table 7.3.1x-1x: Reference sensitivity for FDD and TDD UE single receivercategory 1 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 | -100.2 | -97.2 | -95.5 | -92 | -90.2 | -89 | FDD |
| 3 | -99.2 | -96.2 | -94.5 | -91 | -89.2 | -88 | FDD |
| 4 | -102.2 | -99.2 | -97.5 | -94 | -92.2 | -91 | FDD |
| 5 | -100.7 | -97.7 | -95.5 | -92.5 |  |  | FDD |
| 8 | -99.7 | -96.7 | -94.5 | -91.5 |  |  | FDD |
| 13 |  |  | -94 | -91 |  |  | FDD |
| 20 |  |  | -94.5 | -91.5 | -88.2 | -87 | FDD |
| 39 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
| 41 |  |  | -95.5 | -92.5 | -90.7 | -89.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |  |

This solution is suitable especially in case that the initial requirements are defined only for sub-set of LTE bands. Drawbacks are that this adds yet another Table to REFSENS sub-clause which is about 50 pages long already and the additional information compared to Table 7.3.1A is minimal.

## 2.2 Re-use of existing LTE REFSENS Tables

In this approach the CAT 1 single Rx REFSENS delta to two Rx LTE requirement is captured as a text and LTE REFSENS Tables are referred to. Drawback of this approach is that those bands that are difficult and need 3 dB relaxation needs to be listed separately. If the bands that are applicable initially for CAT 1 single Rx operation are restricted for example to be same as CAT 0 then also those need to be listed. If all bands are applicable then bands where the delta is 2.5 dB do not need to be listed as those are the baseline.

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 6 |  |  | -100 | -97 |  |  | FDD |
| 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 9 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| 10 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 11 |  |  | -100 | -97 |  |  | FDD |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 14 |  |  | -97 | -94 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 17 |  |  | -97 | -94 |  |  | FDD |
| 18 |  |  | -1007 | -977 | -95.27 |  | FDD |
| 19 |  |  | -100 | -97 | -95.2 |  | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 21 |  |  | -100 | -97 | -95.2 |  | FDD |
| 22 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| 23 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.56 | -94.56 | -92.76 |  | FDD |
| 27 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 30 |  |  | -99 | -96 |  |  | FDD |
| 31 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 34 |  |  | -100 | -97 | -95.2 |  | TDD |
| 35 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 36 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 37 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
| 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 43 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 44 |  | [-100.2] | [-98] | [-95] | [-93.2] | [-92] | TDD |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| ... |  |  |  |  |  |  |  |
| 65 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 66 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 68 |  |  | -98.5 | -95.5 | -93.7 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. NOTE 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

For category 1 UE equipped with only one antenna port the operating band reference sensitivity defined in Table 7.3.1-1 is relaxed by 2.5 dB with uplink allocation defined in Table 7.3.1-2. For those FDD operating bands where uplink allocation of a channel bandwidth is NRB > 36 in Table 7.3.1-2 the operating band reference sensitivity is relaxed by 3.0 dB for those channel bandwidths. Reference sensitivity of operating band 13 is relaxed by 3.0 dB for all channel bandwidths. For band 20 the 20 MHz channel bandwidth reference sensitivity is relaxed by 3 dB.

# 3	Conclusions

In this contribution we have made following proposal and discussed how the reference sensitivity could be specified for CAT1 single Rx UE.

Proposal: CAT 1 single Rx requirements are defined to all exiting E-UTRA bands

# References

- RP-161898, New Work Item Proposal: Requirements for single receiver UE based on Category 1 for LTE, RAN#73

- R4-167738, RX requirements for new UE category with single receiver based on Category 1, SONY

- R4-168262, 1 RX UE Category, Nokia

- R4-168597, Single Rx Category 1 RF scope, Qualcomm

- R4-168598, Single Rx Category 1 reference sensitivity, Qualcomm