3GPP TSG-RAN WG4 Meeting #81	R4-1609891

Reno, NV, USA, November 14-18, 2016

Agenda Item:	5.2.3.2

Source: 		Ericsson

Title: 	Simulation results for IRC TM2/3/3 test with 4Rx

Document for:	Discussion

# Introduction

In the WF [1],  it was agreed to add new 4Rx testcases for IRC performance in 36.101 Rel-13.

  - Type A IRC,

    - Define new 4Rx tests with TM2/3/3 (8.2.1.2.4 and 8.2.2.2.4)

    - Finalize the tests in maintenance.

# Simulation Assumptions

The simulations below are based on the CR in tdoc R4-168632 [2] with the parameters as shown below.

## FDD

Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  | Unit |  | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB | -3 | -3 |  |
|  |  |  | dB | -3 (NOTE 1) | -3 |  |
|  |  |  | dB | 0 | 0 |  |
| Cell-specific reference signals |  |  |  | Antenna ports 0,1 | Antenna ports 0,1 |  |
| at antenna port |  |  | dBm/15kHz | -98 | N/A |  |
| DIP (NOTE 2) |  |  | dB | N/A | -2.23 |  |
| BWChannel |  |  | MHz | 10 | 10 |  |
| Cyclic Prefix |  |  |  | Normal | Normal |  |
| Cell Id |  |  |  | 0 | 1 |  |
| Number of control OFDM symbols |  |  |  | 2 | 2 |  |
| PDSCH transmission mode |  |  |  | 2 | N/A |  |
| Interference model |  |  |  | N/A | As specified in clause B.5.2 |  |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 |  | % | N/A | 80 |  |
|  | Rank 2 |  | % | N/A | 20 |  |
| Reporting interval |  |  | ms | 5 | N/A |  |
| Reporting mode |  |  |  | PUCCH 1-0 | N/A |  |
| Physical channel for CQI reporting |  |  |  | PUSCH(Note 5) | N/A |  |
| cqi-pmi-ConfigurationIndex |  |  |  | 2 | N/A |  |
| NOTE 1: NOTE 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. NOTE 3: Cell 1 is the serving cell. Cell 2 is the interfering cells. NOTE 4: Cell 2 transmission is delayed with respect to Cell 1 by 0.33. Note 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#5 and #0. |  |  |  |  |  |

Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (NOTE 3) | Reference Value |  |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (NOTE 2) |  |  |
| 1 | R.46 FDD | OP.1 FDD | N/A | EVA70 | EVA70 | 2x4 Low | 70 | TBD | ≥1 |  |
| NOTE 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. NOTE 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. NOTE 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |  |

## TDD

Table 8.10.1.2.1A-1: Test Parameters for Transmit diversity Performance (FRC) with TM3 interference model

| Parameter |  | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 | -3 |
|  |  | dB | -3 (Note 1) | -3 |
|  |  | dB | 0 | 0 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 | Antenna ports 0,1 |
| at antenna port |  | dBm/15kHz | -98 | N/A |
| DIP (Note 2) |  | dB | N/A | -1.73 |
| BWChannel |  | MHz | 10 | 10 |
| Cyclic Prefix |  |  | Normal | Normal |
| Cell Id |  |  | 0 | 1 |
| Number of control OFDM symbols |  |  | 2 | 2 |
| PDSCH transmission mode |  |  | 2 | N/A |
| Interference model |  |  | N/A | As specified in clause B.5.2 |
| Probability of occurrence of transmission rank in interfering cells | Rank 1 | % | N/A | 80 |
|  | Rank 2 | % | N/A | 20 |
| Reporting interval |  | ms | 5 | N/A |
| Reporting mode |  |  | PUCCH 1-0 | N/A |
| ACK/NACK feedback mode |  |  | Multiplexing | N/A |
| Physical channel for CQI reporting |  |  | PUSCH(Note 5) | N/A |
| cqi-pmi-ConfigurationIndex |  |  | 4 | N/A |
| Note 1: Note 2: The respective received power spectral density of each interfering cell relative to is defined by its associated DIP value as specified in clause B.5.1. Note 3: Cell 1 is the serving cell. Cell 2 is the interfering cell. Note 4: The cells are time-synchronous. Note 5: To avoid collisions between CQI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#4 and #9 to allow periodic CQI to multiplex with the HARQ-ACK on PUSCH in uplink subframe SF#8 and #3. |  |  |  |  |

Enhanced Performance Requirement Type A, Transmit Diversity (FRC) with TM3 interference model

| Test Number | Reference Channel | OCNG Pattern |  | Propagation Conditions |  | Correlation Matrix and Antenna Configuration (Note 3) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 |  | Fraction of Maximum Throughput (%) | SINR (dB) (Note 2) |  |
| 1 | R.46 TDD | OP.1 TDD | N/A | EVA70 | EVA70 | 2x4 Low | 70 | TBD | ≥1 |
| Note 1: The propagation conditions for Cell 1 and Cell 2 are statistically independent. Note 2: SINR corresponds to of Cell 1 as defined in clause 8.1.1. Note 3: Correlation matrix and antenna configuration parameters apply for each of Cell 1 and Cell 2. |  |  |  |  |  |  |  |  |  |

# Discussions and Results

These are the simulation results for the testcases for 4Rx IRC based on the simulation assumptions above and introduced in 36.101 section 8.10.1.1.1A (FDD) and 8.101.2.1A (TDD) in the approved CR in tdoc R4-168632 [2]

Figur 1: The FDD simulation results when 8.10.1.1.1A is simulated with 2 and 4Rx

Figur 3: The TDD simulation results when 8.10.1.2.1A is simulated with 2 and 4Rx

Based on these simulations and after an alignment of simulation results it is proposed to specify a new requirement for 4Rx based on the 2Rx IRC requirement in 8.2.1.2.4 and 8.2.2.2.4 in the 4Rx performance section.

# Conclusions

Based on these simulation results with alignment with other companies a CR as proposed in [2]

# References

- R4-167008: WF on antenna connection for legacy 2Rx tests for 4Rx UEs,

- R4-168632: CR for IRC TM2/3/3 tests with 4Rx in Rel-13, Ericsson