3GPP TSG RAN4 Meeting #81	R4-1609975

Reno, USA, 14 – 18 November, 2016

Title: 	MSD for 1+3+42 2UL/3DL CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.2

Document for:	Approval

# 1	Introduction

This contribution discusses MSD requirements for 1+3+42 2UL/3DL CA.

# 2	Discussion

This CA combination has IMD4 falling on top of own B42 RX . The test was suggested in [2]:

| B1+B3+B42 | B1 | IMD4 | 3*fB3 -fB1 | 1925 | 2115 | 5 | 25 | B42 | 3415 | 3415 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B3 |  |  | 1780 | 1875 | 5 | 25 |  |  |  |  |

We use conventional common triplexer based architectures in our analysis with the following component assumptions. Diplexer/Triplexer is not assumed in diversity. Please note that using triplexer allows implementing B3 with for instance B1+B3 quadplexer in the middle branch.

| Component | IP4 (dBm) |
| --- | --- |
| Antenna switch | 56 |
| Diplexer/Triplexer | 55 |
| Quadplexer | 55 |
| LNA | -6 |

B1+B3 quadplexer B3 TX rejection at B42								    9

B1+B3 quadplexer B1 TX rejection at B42								  25

B3 PA forward mixing IMD4 at PA output with 60dB isolation	       -41

B42 FE loss (switch+trace+filter)										    6

General

Antenna isolation 														  15

PCB isolation PA-PA 												   	  60

PCB isolation PAout-RX													  65

Diplexer isolation															  15

LNA NF																   	    5

RFIC Pout																	    0

RFIC TX pin-pin isolation											     	  40

We note that the B3 TX forward attenuation at B42 frequency is only 9dB, which is heavily dominating MSD for B42.

Based on these parameters we get the following MSD’s. PA forward mixing results are based on measurements with commercial PA’s. CF is 2dB in this analysis.

IMD4 dominant factors:

# 3	Conclusion

MSD analysis for CA_1A-3A-42A was provided with the following results.

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B1+B3+B42 | B1 | IMD4 | fB3 +fB5 | 1925 | 5 | 25 | 3415 | 5 | 2 | 22.8 |
|  | B3 |  |  | 1780 | 5 | 25 |  |  |  |  |

# 4	References

[1] R4-168857, “MSD Test configurations for xDL/2UL CA band combinations with self interference issues”, LG, RAN4#80bis

[2] R4-168105, “2UL 3DL Possible testpoints”, Qualcomm Inc, RAN4#80bis