3GPP TSG RAN4 Meeting #81	R4-1609989

Reno, US, 14 – 18 November, 2016

Title: 	CA_3A-3A REFSENS

Source: 	Huawei, HiSilicon

Agenda item:	x.y.z

Document for:	Other (Approval)

# 1	Introduction

This contribution analyses REFSENS for intra-band non-contiguous CA on B3 (CA_3A-3A).

# 2	Discussion

In previous meeting companies were encouraged to provide analysis for B3 intra-band non-contiguous CA REFSENS.

The following parameters were used in our analysis

| PARAMETER | PCC+SCC 25RB+25RB | PCC+SCC 25RB+15RB | PCC+SCC 15RB+25RB |
| --- | --- | --- | --- |
| TX noise@RX | -143.5 | -144.5 | -143.5 |
| RX phase noise | -145 | -145 | -144 |
| B3 duplexer TX-iso | 50 |  |  |
| B3 duplexer RX-iso | 46 |  |  |
| B3 duplexer loss | 4.5 |  |  |
| RX filter loss | 3.0 |  |  |
| B3duplexer attenuation @RX | 43 |  |  |
| RX filter attenuation @TX | 32 |  |  |
| Antenna isolation | 10 |  |  |
| RFIC NF+IM | 6 |  |  |
| IP2 | 50 |  |  |
| PA gain | 28 |  |  |

We get the following results. Please note that we re-analyzed also case 25+25RB even we are not suggesting any changes into that.

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_3A-3A | 15RB+25RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.4 | FDD |
|  |  | 0.0 < Wgap ≤ 47.0 | 151 | 0 |  |
|  | 25RB+15RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 47.0 | 251 | 0 |  |
|  | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 121 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
| NOTE 1: 1refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |

# 3	Conclusion

REFSENS for CA_3A-3A was analyzed with the following results.

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_3A-3A | 15RB+25RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.4 | FDD |
|  |  | 0.0 < Wgap ≤ 47.0 | 151 | 0 |  |
|  | 25RB+15RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 47.0 | 251 | 0 |  |
|  | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 121 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
| NOTE 1: 1refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. |  |  |  |  |  |

# 4	References