TSG-RAN Working Group 4 (Radio) meeting #81	 R4-1610285

Reno, USA, 14 - 18 Nov, 2016

Source:	Airbus DS SLC

Title:	Band 68 compliance with ECC Decision (16)02: A-MPR simulations for normal and extreme environmental conditions

Agenda item:	8.32.2

Document for:	Discussion

# Introduction

At RAN4 meeting #80bis, simulation assumptions to be used to derive A-MPR requirement for band 68 to meet European requirements specified in Decision (16)02 [1] were discussed [2].

It was noted that for the lower part of the band 68 (698-703MHz), two requirements for DTV protection are applicable:

- -42dBm/8MHz in normal environmental conditions

- -30dBm/8MHz in extreme environmental conditions

It was then proposed to agree on simulation assumptions to derive A-MPR requirement for both types of environmental conditions, to be able to assess which of the two above requirements is the most stringent and shall be used to defined requirements in TS36.101.

A proposal for simulation assumptions for extreme and normal environmental conditions can be found in [3].

This contribution provides A-MPR value for 5MHz LTE signal in the 698-703MHz frequency range, for which the two requirements above are applicable

# Discussion

## Simulation assumptions for band 68 work item

The simulation assumptions proposed in [3] are summarized in the Table below.

| Tx chain performances |  |  | Worst case relative filter rejection (dB) |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Extreme | Normal | Frequency (MHz) | Extreme | Normal |
| Modulator IQ – image | 28dB | 28dB | 693.5 | 5.7 | 10.9 |
| Modulator carrier leakage | 28dBc | 28dBc | 692.5 | 10.9 | 18.4 |
| Modulator C_IM3 | 60dBc | 60dBc | 691.5 | 18.4 | 26.2 |
| UTRA ACLR 1 | 33dB | 36.5dB | 690.5 | 26.2 | 30.6 |
|  |  |  | 689.5 | 30.6 | 31.7 |
|  |  |  | 688.5 | 31.7 | 32.2 |
|  |  |  | 687.5 | 32.2 | 32.2 |
|  |  |  | 686.5 | 32.2 | 32.2 |

## Simulation results

The simulations are performed for a 5MHz LTE signal centred at Fc = 700,5MHz.

The needed MPR + A-MPR to meet -30dBm/8MHz in extreme environmental conditions are depicted on the Figure below.

The needed MPR+A-MPR to meet -42dBm/8MHz in normal environmental conditions are depicted on the Figure below.

## Proposal for A-MPR requirement specification

As shown in the above section, meeting the -42dBm/8MHz requirements in normal conditions is more stringent than -30dBm/8MHz in extreme conditions. In both cases, the most stringent configurations to meet DTV protection requirements are for a small number of RBs allocated to the lowest frequency.

Since maximum output power is a key parameter to ensure large uplink coverage for Public Safety, it is proposed that the A-MPR table in the 36.101 specification limits as much as possible the  configurations where A-MPR is needed.

Focusing only on LTE 5MHz, Fc=700.5dB, the structure of the table defining A-MPR requirement could be:

| Channel bandwidth [MHz] | Parameters |  |  |  |
| --- | --- | --- | --- | --- |
| 5 | Channel higher edge [MHz] | ≤ 703 |  |  |
|  | RBstart | =0 | =0 | ≥1 |
|  | LCRB [RBs] | ≤2 | >2 | ≥1 |
|  | A-MPR [dB] | ≤ [2] | [0] |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe |  |  |  |  |

## 3	Conclusion

In this contribution, A-MPR simulation results for extreme and normal environmental conditions are presented for a 5MHz LTE signal in the frequency range 698-703MHz.

Based on the results, a structure for the table defining A-MPR requirement for band 68 UEs that comply with European requirement defined in ECC Decision (16)02 [1] is proposed with TBC values.

## 4	References

[1]	Decision (16)02: Harmonised technical conditions and frequency bands for the implementation of Broadband Public Protection and Disaster Relief (BB-PPDR) systems - http://www.erodocdb.dk/Docs/doc98/official/pdf/ECCDEC1602.PDF

[2]	R4-168556 , Band 68 compliance with ECC Decision 6(02): Impacts on TS36.101, Airbus DS SLC

[3]	R4-1610238 Band 68 compliance with ECC Decision (16)02: Simulation assumptions for normal environmental conditions, Airbus SLC