3GPP TSG-RAN WG4 #82	R4- 1700590

Athens, Greece, 13 – 17 Febuary, 2017

# Source:	Intel Corporation

# Title:	On defining NR spectrum in mmWave

# Agenda Item:	10.2

# Document for:	Discussion

# 1	Introduction

This contribution provides our view on NR duplex mode and NR band definition for mmWave.

# 2	Discussion

# 2.1 Duplex mode

FDD and TDD have unique characteristics one over the other. In earlier stage in LTE deployment, FDD had been the dominant duplex method. However, TDD had been introduced on BAND 38, 39, 40, etc. starting from mid-2010. Since then both FDD and TDD have been two favourable duplex techniques.

Dynamic Bandwidth Allocation

An advantageous aspect is that TDD has more flexible to support key NR features which requires dynamic bandwidth allocation. Unlink FDD which requires a pair of spectrum, TDD only needs a single spectrum for both downlink and uplink which provides greater flexibility for adaptation to variable global spectrum allocations. For MTC or V2V/V2X as an example, bandwidth will be very dynamic. Most of bandwidth requires for UL at one time, but the same device would need most of traffics for DL at the other time. Also NSA operation would be another example in which TDD enables dynamic spectrum allocation of DL and UL resources to support asymmetric DL/UL traffic efficiently.

Front-End Loss & Size

In mm-Wave frequency range, we are expecting to combat with higher noise figure and it would be critical to eliminate front-end losses as much as possible. FDD requires a duplexer to protect the sensitive receiver signal from transmitter noise as well as other interferences and blockers. In current LTE system, insertion loss of a duplexer is 1.5 – 2.0 dB depending on operating bands. Even with technology evolving, this loss would be one of the largest portions in front-end loss. By nature of Tx/Rx switching in TDD, it is possible to replace this duplexer loss with Tx/Rx switch loss which is normally 0.3 – 0.5dB. By adopting TDD, we could take the advantage of more than 1dB in front-end loss.

On size perspective, TDD also has an advantage over FDD. Considering NR supports very broadband signal compared to any other existing communication technologies, the rejection requirement of a duplexer in NR bands would be very challenging in terms of Q-factor. With high-Q requirement, a duplexer should be implemented in discrete off-chip type. In TDD, a typical switch could be integrated in on-chip which saves total foot-print of RF front-end.

Cell Coverage and Battery Life

Due to a switching nature of TDD, cell coverage of TDD would be smaller than FDD. In LTE system, a PA output power for TDD LTE needs to be 1dB bigger than FDD LTE PA to achieve the same cell coverage. In NR which operates mm-Wave, shrinking cell coverage is expected and generating higher output power would be feasible with evolving PA technology.

In battery life perspective, a PA in FDD would consume larger amount of energy to overcome a duplexer loss which makes the PA in FDD inefficiency compared to a PA in TDD.

A summary of characteristics of TDD and FDD is shown in Table 1.

|  | TDD | FDD |
| --- | --- | --- |
| UL/DL Symmetry | Support Asymmetrical UL/DL | Usually 50/50 |
| Dynamic Bandwidth Allocation | Support | None |
| Front-End Loss | Lower | Higher |
| Front-End Size | Smaller | Larger |
| Front-End Cost | Relatively Cheap | Relatively Expensive |
| Cell Coverage | Smaller | Larger |
| Battery Life | Could be longer than FDD | Could be shorter than TDD |
| Latency | Depends on range; Tx/Rx switching time | Little or none |

Table 1. A summary of characteristics of TDD and FDD.

As described above, FDD and TDD have unique characteristics one over the other. To enable key NR features, TDD is more suitable method at least for new NR spectrums.

Proposal 1: TDD should be adapted exclusively for new spectrums to enable key NR features.

# 2.2 New bands for NR

In current LTE there are 51 bands and more than 800 CA/DC band combinations. The following figure shows the current LTE bands, CA, and the issues of LTE band structure [1]. Since there are so many different bands, UE needs to support many bands for global roaming. It is extremely complicated for UE implementation in order to support so many bands and CA.

Figure 1. LTE bands, CA, and issues of LTE band structure

Companies provided initial inputs of possible frequency ranges or bands for NR deployment in the last RAN4 adhoc meeting. The following table summarizes the possible frequency ranges [2]. Other frequency ranges are not precluded.

| Company | R4-1700xxx | Proposal |
| --- | --- | --- |
| Sprint, CTC, C-Spire, China Unicom | 255 | 2496MHz – 2690MHz |
| China Telecom | 256 | 824- 849MHz/869 – 894MHz, 1920 – 1980MHz/2110 – 2170MHz, 1710 – 1785MHz/1805 – 1880MHz, 2496 – 2690MHz, 3300 - 3400MHz, 3400 – 3600MHz, 4400 - 4500MHz, 4800 - 4990MHz |
| AT&T | 034 | 37-37.6GHz, 37.6-40GHz |
| DISH Network | 193 | 12.2-12.7GHz |
| Orange | 194 | 703-748MHz/758-803MHz, 832–862MHz/791-821MHz、1920-1980MHz/2110-2170MHz,1710-1785MHz/1805-1880MHz,2500-2570MHz/2620MHz-2690MHz,3400-3600MHz,3600-3800MHz,5.925-8.5GHz,24.25-27.5GHz,31.8-33.4GHz |
| KT | 210 | 880 – 915MHz/925 – 960MHz, 1920 – 1980MHz/2110 – 2170MHz, 1710 – 1785MHz/1805 – 1880MHz, 2300 – 2400MHz, 3400-3700MHz, 26.5-29.5GHz, 24.25-27.5GHz, 31.8-33.4GHz, 37-40.5GHz |
| CMCC | 162 | Around 3.5GHz |
| DOCOMO | 139 | 3.3-4.2GHz, 4.4-4.99GHz, 26.5-29.5GHz. |
| China Unicom | 241 | 3400 - 3600 MHz, 4800 - 4990 MHz, 4400 – 4500 MHz, 3300 - 3400 MHz, 2496 - 2690 MHz, 24.5 GHz-27.5 GHz, 37 GHz-43.5 GHz |
| Verizon, T-mobile | 244 | 27.5 – 28.35 GHz |
| Etisalat | 267 | 1427 – 1518MHz, 3400 - 3600MHz, 10.7 – 11.7GHz, 24.25- 27.5GHz |
| Huawei, Hisilicon | 124, 267 | 3.3-[3.8GHz~4.2GHz], 4.4-4.99GHz, 24.25-27.5GHz, 26.5-29.5GHz, 37-40GHz, 40.5-43.5GHz, 1427 – 1518MHz, 3400 - 3600MHz |
| ZTE | 249 | 3.6-3.6GHz, 24.25-27.5GHz |
| Ericsson | 255, 162 | 2496MHz – 2690MHz, Around 3.5GHz |
| Nokia | 255 | 2496MHz – 2690MHz |

Table 1. Proposals of NR spectrum from companies

From the above table, it can be seen that the frequency allocations are very diverse. It becomes very important to harmonize the frequency range among different regions/countries, and define as few bands as possible. Fewer bands can lead to wider spectrum resources, fewer potential CA combinations, less complexity device implementation, and less standardization work.

# 2.2.1 NR Band Definition for mmWave

From the table above, it would be hard to come up with a common band which satisfies all the operators’ need. In this document, we would like to propose below bands as two NR bands:

- Band C: 24.25 – 29.5 GHz

- Band D: 37.0 – 40.0 GHz

In this proposal, Band D cover 37.0 – 40.0 GHz, but it could be extended up to 43.5 GHz later.

Figure 2. NR band proposals in mmWave

Supporting channel bandwidth is up to 800 MHz within each of Band C and Band D.

Proposal 2: Define a single band in the frequency range of 24.25 – 29.5 GHz

Proposal 3: Define a single band in the frequency range of 37.0 – 40.0 GHz

# 2.2.2 Front-end Architecture

Since the supporting bandwidth (5250 MHz for Band C and 3000 MHz for Band D) does prevent implementation of an acoustic type of filter with sharp roll-off, co-existence could be a concern depending on the scenarios. We will describe more detail about the scenarios that we could avoid and/or mitigate the potential coexistence problems.

In the front-end architecture, we will use TDD as the duplexer mode. Also we will cover the bandwidths with a single PA. We believe that a PA supporting a broadband BW is not a big challenge and fairly common for PA’s in LTE radios to cover large BW’s with switches to route to different filters for each channels or bands. Similar principal also works for LNA to mitigate an interference from other channels or bands.

Detail band usage plan and associated potential co-existence scenarios are FFS.

Proposal 4: Detail band usage plan and associated co-existence scenarios are FFS.

# 3	Conclusions

In this contribution, the following proposals were made:

Proposal 1: TDD should be adapted exclusively for new spectrums to enable key NR features.

Proposal 2: Define a single band in the frequency range of 24.25 – 29.5 GHz

Proposal 3: Define a single band in the frequency range of 37.0 – 40.0 GHz

Proposal 4: Detail band usage plan and associated co-existence scenarios are FFS.

# 4	Reference

- RP-162214, “Consideration on new band structure for 5G”, Huawei

- R4-1700304, “WF on NR spectra related work”, NTT DoCoMo