3GPP TSG-RAN WG4 Meeting #82	R4-1700682

# Athens, Greece, 13-17 February 2017

Source:	SoftBank Corp.

Title:	Handling of the 2nd harmonic note for CA_11A-28A

Agenda item:	7.2.2

Document for:	Approval

1.  Introduction

2DL/1UL CA of 11A-28A was completed in the last RAN-P. However, there was a lack of some consideration on 2nd harmonic, i.e. ∆FHD and additional exception in REFSENS exception table (Note 9 and 11 of Table-7.3.1A-0a). This paper is to fill the void around the issues.

Companion CR [1] and TP [2] are also provided upon agreement of this document.

2.  Issues specific to 2nd Harmonic

Interference of 2nd harmonic within a UL is not a popular case, typically 3+8 (but no exception applied) or 3+42. Then 3+42 was the first case as a matter of fact and additional sentences were put to indicate REFSENS exception is extended beyond (2 × Aggressor’s BW) as below:

NOTE 9:	These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_3A-19A-42A, and CA_1A-3A-19A-42A.

Then, it looks like that exception due to 2nd harmonic may need to evaluate extended range (∆FHD) in combination by combination basis. 10MHz ∆FHD was agreed for B3+B42. In addition, there was another set of requirements with a note:

NOTE 11:	The requirements are only applicable to channel bandwidths with a carrier frequency at  MHz offset from  in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively.

This requirement is to be applied for the 20MHz away from the edge of victim BW to measure the REFSENS. In short, at least for 3+42,

- REFSENS exception is extended to 10MHz (∆FHD) from both high/low edges of victim DL transmission BW,

- Additional exception is defined for 20MHz offset from the edges of victim DL transmission BW.

In case of B3+B42, both are popular bands with relatively broader passband. So the definition as such could be beneficial to give some guidance on where or to what extent desense happens.

3.  For B11+B28

To apply a similar scheme to B11+B28, first thing to draw an attention is that this CA is specific to two operators in Japan at present, KDDI and SoftBank. The spectrum holdings of the operators are as below:

Table 1: Current spectrum holdings of Band 11 and Band 28 in Japan

|  | KDDI | SoftBank |
| --- | --- | --- |
| B28 Tx | 718 - 728 | 738 - 748 |
| 2*B28 Tx | 1436 - 1456 | 1476 - 1496 |
| B11 Rx | 1485.9 – 1495.9 | 1475.9 – 1485.9 |
| Edge to edge difference between H2 and B11 Rx | 29.9 | 0 (Direct Hit) |

From this, it is apparent that we do not need to consider case 2), REFSENS exception at 20MHz offset. In B3+B42 where MSD is slightly bigger than that of B11+B28, exception for case 2) is not so big, less than 2dB to single band REFSENS. Taking 29.9MHz minimum offset and 5MHz/10MHz aggressor uplink into account, it seems there is no need to define exception for 718-728MHz and 1485.9 – 1495.9MHz combination. Then,

[Proposal - 1] No REFSENS exception for offset case is defined for B11+B28.

Secondly, for ∆FHD, the degree of extension depends on the characteristics of PA or its operation point (or mode) and there might not be drastic difference between Band 3 and Band 28 PA. Then, if a specific value is required, 10MHz is a rational candidate. On the other hand, based upon spectrum holding, it is almost no use to utilize 10MHz extension since two cases for B11+B28 are direct hit or at least 30MHz apart. Therefore,

[Proposal - 2] ∆FHD = 0 is proposed for B11+B28.

For UE testing, frequency positions are to be confined to the operator specific scenarios mentioned above [3].

4.  Feedback to TS/TR

Since CA_11A-28A has been approved and implemented, a companion CR for 36.101 [1] is provided to reflect proposal-1 and 2. TP to add the thought above is also provided for TR36.714-02-01[2].

For 3CAs to be handled in this meeting [4-6], the TPs have already included the two proposals.

5.  Conclusion

This paper is to discuss handling of 2nd harmonic for 11A-28A. Proposals are:

[Proposal - 1] No REFSENS exception for offset case is defined for B11+B28.

[Proposal - 2] ∆FHD = 0 is proposed for B11+B28.

Relevant CR [1] and TP [2] should also be approved to reflect the thought.

6.  Reference

[1]     R4-1700699	 Correction of notes on harmonic exception in 2DL CA 11A-28A, SoftBank Corp.

[2]     R4-1700702   TP for TR36.714-02-01: REFSENS exception of LTE-A Inter-band Carrier Aggregation (2DL/1UL) of Band 11 and Band 28, SoftBank Corp.

[3]     R5-170843     Introduction of CA_11A-28A to section 7, Ericsson, SoftBank

[4]     R4-1700704   TP for TR36.714-03-01: Support of LTE-A Inter-band Carrier Aggregation (3DL/1UL) of Band 1, Band 11 and Band 28, SoftBank

[5]     R4-1700706   TP for TR36.714-03-01: Support of LTE-A Inter-band Carrier Aggregation (3DL/1UL) of Band 3, Band 11 and Band 28, SoftBank

[6]     R4-1700707   TP for TR36.714-03-01: Support of LTE-A Inter-band Carrier Aggregation (3DL/1UL) of Band 8, Band 11 and Band 28, SoftBank

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"