3GPP TSG-RAN WG4 Meeting #82 	R4-1700702

Athens, Greece, 13 – 17 February, 2017

Source:	SoftBank Corp.

Title:	TP for TR36.714-02-01: REFSENS exception of LTE-A Inter-band Carrier Aggregation (2DL/1UL) of Band 11 and Band 28

Agenda item:	7.2.2

Document for:	Approval

1.  Introduction

As discussed in a companion document [1], consideration is needed for 2nd harmonic exception for REFSENS on CA_11A-28A. This paper is to reflect the thought (when agreed) to TR36.714-02-01. TP is attached as Annex.

Note that CA_11A-28A has already been implemented in 36.101, the modification to 36.101 is conducted with a separate CR.

2.  Reference

[1]     R4-1700682  	Handling of the 2nd harmonic note for CA_11A-28A, SoftBank

Annex: TP for TR36.714-02-01 (v0.5.0)

[Unchanged portions skipped]

6.22	CA_11A-28A_BCS0

6.22.1	Operating bands for CA

Table 6.22.1-1: Inter-band CA

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_11-28 | 11 | 1427.9 MHz | – | 1447.9 MHz | 5, 10 | 1475.9 MHz | – | 1495.9 MHz | 5, 10 | FDD |
|  | 28 | 703 MHz | – | 748 MHz | 3, 5, 10, 15, 20 | 758 MHz | – | 803 MHz | 3, 5, 10, 15, 20 | TDD |

6.22.2		Channel bandwidths per operating band for CA

Table 6.22.2-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| CA operating / channel bandwidth |  |  |  |  |  |  |  | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| CA_11A-28A | 11 |  |  | Yes | Yes |  |  | 0 |
|  | 28 |  |  | Yes | Yes | Yes | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of  TS 36.101 [6] when operating in single carrier mode.

6.22.3	Co-existence studies

The harmonic relations of Band 11 and Band 28 are calculated as Table 6.22.3-1. It is observed that the 2nd harmonic of Band 28 Tx will cover the whole receiving band of Band 11.

Table 6.22.3-1: Impact of UL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 11 | 1427.9 | 1447.9 | 1475.9 | 1495.9 | 2855.8 | 2895.8 | 4283.7 | 4343.7 | 2951.8 | 2991.8 | 4427.7 | 4487.7 |
| 28 | 703 | 748 | 758 | 803 | 1406 | 1496 | 2109 | 2244 | 1516 | 1606 | 2274 | 2409 |

Table 6.22.3-2: Band 11 and Band 28 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 1475.9 | 1495.9 |
| 2nd order harmonics frequency range (MHz) | 1516 | 1606 | 2951.8 | 2991.8 |
| 3rd order harmonics frequency range (MHz) | 2274 | 2409 | 4427.7 | 4487.7 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 672.9 | 737.9 | 2233.9 | 2298.9 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 20.1 | 130.1 | 2148.8 | 2233.8 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 2991.9 | 3101.9 | 3709.8 | 3794.8 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 738 | 823 | 1430.9 | 1540.9 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 748 | 813 | 1455.9 | 1515.9 |

It can be seen from Table 6.22.3-2 that:

- the 2nd harmonics of BS transmitting in Bands 11 and 28 may fall into none of the BS receive band of 3GPP

- the 3rd harmonics may fall into BS receive band of Bands 30 and 40

- the 2nd IMD products may fall into BS receive band of Bands 12, 17, 28, 44 and 68

- the 3rd IMD products may fall into the BS receive band of Bands 11, 13, 14, 18, 21, 26, 27, 28, 43, 44 and 45,

assuming (except the last row) the whole 20MHz DL frequency of Band 11 and the whole 45 MHz DL frequency of Band 28.

If the BS is only transmitting up to 10 MHz DL in Band 11 and 20MHz DL in Band 28 as stated in the WID, then the 3rd IMD products will not fall into the BS receive band of Band 11, 18, 26 and 28 as shown in the last row in table 4. (Note that even so, Band 28 is still subject to 2nd IMD fall down.)

It should be noted that Bands 12, 13, 14, 17, 27, 30, 40, 43 and 45 are not intended for use in the same geographical area as Bands 11 and 28. In addition, co-existence between Band 28 and Band 44 is unlikely as Band 44 is TDD variant of APAC-700 spectrum then it could be expected that the deployment of Band 28 and 44 is mutually exclusive. Therefore, the focus here will be on the harmonics and IMD falling into Bands 21 and 28.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 21 or 28 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 11 and 28 BS transmitters do not share the same antenna with Band 21 or 28 BS receiver.

.

6.22.4	ΔTIB and ΔRIB values

Concerning CA of a mid-band and a low-band, default values of ∆TIB=0.3dB and ∆RIB=0dB were agreed as a framework but conventionally for CA including B11 and B21, ∆TIB=0.4dB has been proposed/accepted for mid-band to accommodate “extended Low-High” diplexer. In addition, a harmonic trap filter (HTF) is assumed in Band 28 path that introduces additional relaxation of 0.3dB for Tx and 0.2dB for Rx.

For two simultaneous DL and only one UL, the TIB,c and RIB values are given in the tables below.

Table 6.22.4-2: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_11A-28A | 11 | 0.4 |
|  | 28 | 0.6 |

Table 6.22.4-3: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_11A-28A | 11 | 0 |
|  | 28 | 0.2 |

As pointed out in section 3, there is a harmonic relation for the 2nd harmonic of Band 28 Tx falling down to Band 11 Rx. Thus, we need to evaluate MSD in Band 11 when Band 28 is used as Pcell.

6.22.5	REFSENS requirements

As noted above, the 2nd harmonic of B28 Tx will fall into B11 Rx. The following table is REFSENS exception when harmonic fall down occurs.

Table 6.22.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1,4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_1A-3A-42A, CA_3A-19A-42A, and CA_1A-3A-19A-42A. FHD = 0MHz for CA_11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Table 6.22.5-2: Uplink configuration for the low band

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in TS 36.101 Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to TS 36.101 Table 7.3.1-2 applies. |  |  |  |  |  |  |  |  |

The availability of CA_11A-28A is, at the time of writing, limited to two operators in Japan. Based on their spectrum holding, one operator will be suffered by direct fall down of the 2nd harmonic of B28 Tx while the other will have around 30MHz separation as minimum between its own Band 11 Rx and possible 2nd harmonic of Band 28 Tx. Taking this situation into account, REFSENS relaxation near the harmonic fall down is not necessary to be defined and tested so ∆FHD is set to 0MHz. Furthermore, additional relaxation (such as defined for CA_3A-42A) is not required also.

[Unchanged portions skipped]

form change history:

2013-12-06 v1.14.1 modified §11 to read: <FamilyName>, <GivenName>, (If the person is new to 3GPP work, give full contact coordinates, in particular, email address.)

2013-10-03 v1.14.0 removal of embedded help text

v1.13.2: adds tdoc header

v1.13.1: minor changes resulting from discussions at CT#41 & SA#41

v1.13.0: mods to enforce linkage amongst stages 1, 2, 3

draft mods Scarrone-Meredith 2008-07 ff

v1.12.1: removes revision marks following approval at SP-29
v1.12.0: includes provision for Study Items (SP-29)

v1.11.0: includes those changes from v1.8.0 agreed at SP-25.

v1.10.0: full circle

v1.9.0: a clean sheet

v1.8.0: includes comments from SA#24

v1.7.0: includes comments from RAN, CN and T #24; also includes “early implementation” data

v1.6.0: includes comments made during review period prior to TSGs#24

v1.5.0: includes comments made at TSGs#23 (Phoenix)

v1.4.0: offered to SA#23 for approval

v1.3.0: offered to CN#23, RAN#23 and T#23 for comments

DRAFT4 v1.3.0: 2004-03-09: Incorporation of comments from Leaders list

DRAFT3 v1.3.0: 2004-02-19: Incorporation of comments from MCC members

DRAFT2 v1.3.0: 2004-01-29: Complete redraft:

v1.2.0: 2002-07-04: "USIM" box changed to "UICC apps"

2003-05-28: spelling of “rapporteur” corrected

2002-07-04: "USIM" box changed to "UICC apps"