3GPP TSG-RAN WG4 Meeting #82	R4-1700754

Athens, Greece, 13 – 17 February, 2017

Source:	Ericsson

Title:	TP to TR 36.714-02-02: CA_2DL_2A-66A_2UL_2A-66A_BCS0

Agenda item:	7.6.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-02-02 for inclusion of CA_2DL_2A-66A_2UL_2A-66A_BCS0 as defined in RP-162148 [1].

# 2. Text Proposal for TR 36.714-02-02

##### ---Start of changes---

## 6.x	CA_2DL_2A-66A_2UL_2A-66A_BCS0, BCS1, BCS2

### 6.x.1	Channel bandwidths per operating band for CA

Table 6.x.1-1: E-UTRA CA configuration and bandwidth combination sets for CA_2A-66A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-66A | CA_2A-66A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 66 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 40 | 2 |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

### 6.x.2	Co-existence studies

#### 6.x.2.1	UE-UE co-existence

Table 6.x.2-1 lists B2+B66 2UL CA 2nd and 3rd order harmonics. Table 6.x.2-2 lists the IMDs from 2nd order to 5th order for UE-UE co-existence analysis.

Table 6.x.2.1-1: Band 2 and Band 66 UL harmonics

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 5550 | 5730 |

Table 6.x.2.1-2: 2UL CA_2A-66A IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 1850 | 1910 |
| 2nd order IMD products | ¦fx_low – fy_high¦ | ¦fx_high – fy_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 200 | 70 | 3560 | 3690 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1510 | 1710 | 1920 | 2110 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5270 | 5470 | 5410 | 5600 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 400 | 140 | 7120 | 7380 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3220 | 3490 | 3770 | 4020 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 6980 | 7250 | 7260 | 7510 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5930 | 5620 | 5270 | 4930 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 9110 | 9420 | 8690 | 9030 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 2310 | 1990 | 1640 | 1310 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8970 | 9290 | 8830 | 9160 |

Based on Table 6.x.2.1-2:

3rd order IMD may fall into own Rx of Band 2.

5th order IMD may fall into own Rx of Band 66.

Based on table 6.x.2.1-1:

2nd order harmonics may fall into Rx frequencies of band 22, 42, 43 and 48.

3rd order harmonics may fall into Rx frequencies of band 46.

Based on table 6.x.2.1-2:

2nd order IMD may also fall into Rx frequencies of band 22, 42, 43 and 48.

3rd order IMD may also fall into Rx frequencies of band 21, 24, 25, 34, 37, 46 and 70.

4th order IMD may also fall into Rx frequencies of band 42 and 43.

5th order IMD may also fall into Rx frequencies of band 1, 4, 10, 11, 21, 23, 24, 25, 32, 34, 40, 45, 46, 47, 65 and 70.

#### 6.x.2.2	Co-existence with other systems

When 2ULs inter-band CA UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.x.2.2-1 lists if up to 3rd order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.x.2.2-1: 2UL B2+ B66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | IMD3, IMD5 |
| Galileo | 1559 | - | 1591 | Yes |  | IMD3, IMD5 |
| GLONASS | 1591 | - | 1610 | Yes |  | IMD3, IMD5 |
| GPS | 1563 | - | 1587 | Yes |  | IMD3, IMD5 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | - | US/Europe | - |
|  | 2400 | - | 2494 | - | Asia | - |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | IMD3, IMD5, 3rd order harmonics of B2 and B66 |
|  | 5150 | - | 5350 | Yes | Europe | IMD3, IMD5, 3rd order harmonics of B2 and B66 |
|  | 5470 | - | 5725 | Yes |  | IMD3, IMD5, 3rd order harmonics of B2 |
|  | 5150 | - | 5825 | Yes | Asia | IMD3, IMD5, 3rd order harmonics of B2 and B66 |

### 6.x.3 	∆TIB and ∆RIB values

For inter-band CA_2A-66A, same requirements on values for ∆TIB and ∆RIB can apply with 1UL-2DL case. Therefore, for two simultaneous UL, the TIB,c and RIB,c values are shown in table 6.1.3-1 and in table 6.1.3-2.

Table 6.x.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-66A | 2 | 0.5 |
|  | 66 | 0.5 |

Table 6.x.3-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-66A | 2 | 0.3 |
|  | 66 | 0.3 |

### 6.x.4 	MSD

Based on the co-existence study in section 6.x.2, there are IMD3 issues for own Rx of Band 2 as well as IMD5 issues for own Rx Band 66. The actual MSD requirements needed need to be studied further.

##### ---End of changes---

# Reference

[1]	 RP-162148, “Revised WID: LTE Advanced inter-band CA Rel-14 for 2DL/2UL”, RAN#74, Huawei, HiSilicon