3GPP TSG-RAN Working Group 4 Meeting #82	R4-1701806

Athens, Greece, 13th Feburary – 17th Feburary 2017

Source:	DISH Network

Title:	TP for TR 36.714-02-01:  Operating bands, Channel bandwidths, Co-existence study and Delta Tib/Rib values for CA_46A-70A

Agenda item:	7.2.2

Document for:	Approval

## 1	Introduction

This contribution is a text proposal for CA_46A-70A which were approved in RAN #74[1].  Co-existence studies for this configuration is also included.

## 2	Text Proposal for TR 36.714-02-01

----- Start of Text proposal -----

## 6.X	CA_46A-70A_BCS0

### 6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit1 |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_46-70 | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configurations and bandwidth combination sets defined for CA_46A-70A

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
|  |  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_46A-70A | - | 46 |  |  |  |  |  | Yes | 35 | 0 |
|  |  | 70 |  |  | Yes | Yes | Yes |  |  |  |

### 6.X.3	Co-existence studies

Table 6.X.3-1 summarizes frequency ranges where harmonics occur due to Band 70 or Band 46 for both UL and DL. It can be seen the harmonic frequencies of Band 70 in UL are away from the UE receive Band 46. Therefore, there is no need for MSD since there is no self-interference.

Table 6.X.3-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 70 | 1695 | 1710 | 1995 | 1710 | 3390 | 3420 | 5085 | 5130 | 3990 | 4040 | 5985 | 6050 |
| 46 |  |  | 5150 | 5925 |  |  |  |  | 10300 | 11850 | 15450 | 17775 |

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 46 and Band 70 DL carriers can be calculated as shown in Table 6.X.3-2 below:

Table 6.X.3-2: Band 46 and Band 70 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1995 | 2020 | 5150 | 5925 |
| 2nd harmonics frequency limits (MHz) | 3990 | 4040 | 10300 | 11850 |
| 3rd harmonics frequency limits (MHz) | 5985 | 6060 | 15450 | 17775 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 3130 | 3930 | 7145 | 7945 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1110 | 1935 | 8280 | 9855 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 9140 | 9965 | 12295 | 13870 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 1220 | 2795 | 5125 | 5950 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1980 | 2035 | 5130 | 5945 |

It can be seen from Table 6.X.3-2 the 2nd IMD products supporting CA of Band 46 and Band 70 may fall into the BS receive band of Bands 22, 42 and 43, while the 3nd IMD products supporting CA of Band 46 and Band 70 may fall into the BS receive band of Bands 11, 21 and 45, and the 3rd IMD products may fall into the BS receive band of Bands 1, 2, 3, 4, 7, 9, 10, 11, 21, 23, 24, 25, 30, 33, 34, 35, 36, 37, 38, 39, 40, 41, 45, 46, 47, 65, 66, and 70.

Note that Bands 1, 9, 11, 21, 33, 34, 37, 38, 39, 40, and 45 are not intended for use in the same geographical area as Bands 46 and 70. Therefore, the focus here is on the harmonics and IMD falling into Bands 2, 3, 4, 7, 10, 23, 24, 25, 30, 35, 36, 41, 46, 47, 65, 66, and 70.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 2, 3, 4, 7, 10, 23, 24, 25, 30, 35, 36, 41, 46, 47, 65, 66, and 70 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 46 and 70 BS transmitters do not share the same antenna with Band 2, 3, 4, 7, 10, 23, 24, 25, 30, 35, 36, 41, 46, 47, 65, 66, or 70 BS receiver.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 46 and 70 BS transmitters should not share the same antenna with Band 2, 3, 4, 7, 10, 23, 24, 25, 30, 35, 36, 41, 46, 47, 65, 66, or 70 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 2, 3, 4, 7, 10, 23, 24, 25, 30, 35, 36, 41, 46, 47, 65, 66, or 70 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

### 6.X.4	∆TIB and ∆RIB values

For the UE which supports CA_46A-70A the TIB = 0 dB and RIB = 0 dB apply respectively assuming separate antenna architecture. No harmonic terms overlap with downlink in Band 46 from uplink in Band 70.

### 6.X.5 		REFSENS requirements

The REFSENS is specified in the following:

# Table 6.X.5-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA-46A-70A | 70 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |

# Table 6.X.5-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_46A-70A | 70 |  |  | 25 | 50 | 75 |  | FDD |

Note that MSD is not required since there is 1) no self-interference as shown in table 6.X.3-1 and 2) the band 70 UL harmonics does not fall directly into the Band 46 exclusion region.

----- End of Text proposal -----

## References

RP-161996, “Revised WID: LTE Advanced inter-band CA Rel-14 for 2DL/1UL”, Qualcomm, 3GPP RAN#74, Vienna, Austria, December, 2016