3GPP TSG-RAN Working Group 4 Meeting #82	R4-1701821

Athens, Greece, 13th Feburary – 17th Feburary 2017

Source:	DISH Network

Title:	TP for TR 36.714-02-01:  Operating bands, Channel bandwidths and Co-existence study for CA_66A-70A

Agenda item:	7.2.2

Document for:	Approval

## 1	Introduction

This contribution is a text proposal for CA_66A-70A which were approved in RAN #74[1].  Co-existence studies for this configuration is also included.

## 2	Text Proposal for TR 36.714-02-01

## ----- Start of Text proposal -----

## 6.X	CA_66A-70A_BCS0

### 6.X.1	Operating bands for CA

Table 6.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_66-70 | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz |  |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: E-UTRA CA configurations and bandwidth combination sets defined for CA_66A-70A

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 | 3 | 5 | 10 | 15 | 20 | Maximum aggregated bandwidth | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz | [MHz] |  |
| CA_66A-70A | 66 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  | 70 |  |  | Yes | Yes | Yes |  |  |  |

### 6.X.3	Co-existence studies

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 66 and Band 70 DL carriers can be calculated as shown in Table 6.X.3-1 below:

Table 6.X.3-1: Band 66 and Band 70 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1995 | 2020 | 2110 | 2200 |
| 2nd harmonics frequency limits (MHz) | 3990 | 4040 | 4220 | 4400 |
| 3rd harmonics frequency limits (MHz) | 5985 | 6060 | 6330 | 6600 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 90 | 205 | 4105 | 4220 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 1790 | 1930 | 2200 | 2405 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 6100 | 6240 | 6215 | 6420 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 1905 | 2110 | 2085 | 2225 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 1980 | 2035 | 2090 | 2220 |

It can be seen from Table 6.X.3-1 the 2nd and 3rd harmonics of BS transmitting in Band 66 will not fall into any other BS receive Bands, while the 3rd IMD products may fall into the BS receive band of Bands 1, 2, 23, 25, 30, 33, 34, 35, 36, 37, 39, 40, and 65.

Note that Bands 1, 33, 34, 35, 36, 37, 39, and 40 are not intended for use in the same geographical area as Bands 66 and 70. Therefore, the focus here is on the harmonics and IMD falling into Bands 1, 2, 23, 25, 30, and 40.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 12, 17, 22, 23, 34 or 36 BS receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 66 and 70 BS transmitters do not share the same antenna with Band 12, 17, 22, 23, 34, or 36 BS receiver.

Therefore, in order to prevent BS receiver desensitization, it is recommended that Bands 66 and 70 BS transmitters should not share the same antenna with Band 12, 17, 22, 23, 34, or 36 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 17, 22, 23, 34, or 36 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

----- End of Text proposal -----

## References

RP-160945, “Revised WID: LTE Advanced inter-band CA Rel-14 for 2DL/1UL”, Qualcomm, 3GPP RAN#81, Reno, Nevada, USA, November 2016