3GPP TSG-RAN WG4 #82	R4-1702457

# Athens, Greece, 13th – 17th February 2017

Agenda Item:	7.18.7

Source:	Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, Huawei

Title:	Simulation assumption for eLAA PUSCH demodulation

Document for:	Discussion

# Introduction

In this contribution, simulation assumptions are provided for eLAA PUSCH simulation.

# Simulation assumption for eLAA PUSCH demodulation

In Table 1, the detail simulation parameters are provided for PUSCH. In Table 2 and Table 3, the FRC for each candidate MCS is given. All the setups are just for simulator alignment, it may be updated in the final test setup.

Table 1: Parameters setup for PUSCH Performance requirements

| Parameter | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| System bandwidth for LAA Scell | 20 MHz |
| Channel model | EPA 5Hz |
| Antenna setup | 1Tx, 2Rx and 4 RX LOW |
| Number of interlaces | 2 |
| Number of PRBs per interlace | 10 |
| PUSCH frequency hopping | No |
| Starting PUSCH symbol configuration | 01 |
| Ending symbol configuration | Up to OFDM symbol 13 |
| MCS | QPSK 1/3, 16QAM ¾, FFS:64QAM 5/6 |
| CP type | Normal |
| Receiver noise | AWGN |
| Number of user | 1 |
| Resource allocation | continuous |
| Frequency error | 0 |
| Time error | 0 |
| Interference | no |
| Reference Receiver | MRC |
| Performance Metric | SNR values [dB] for which 70% of maximum throughput is achieved |
| Max number of HARQ Transmissions | 4 |
| LBT model | No |

Table 2: FRC for different MCS (option 1: 2 interlaces)

| Reference channel | QPSK 1/3 | 16QAM ¾ | 64QAM 5/6 |
| --- | --- | --- | --- |
| Total number of interlaces | 10 | 10 | 10 |
| Allocated number of interlaces | 2 | 2 | 2 |
| DFT-OFDM Symbols per sub frame | 11 | 11 | 11 |
| Modulation | QPSK | 16QAM | 64QAM |
| Code rate | 1/3 | 0.76 | 0.79 |
| MCS index | 5 | 19 | 27 |
| Payload size (bits) | 1736 | 7992 | 12576 |
| Transport block CRC (bits) | 24 | 24 | 24 |
| Code block CRC size (bits) | 0 | 24 | 24 |
| Number of code blocks-C | 1 | 2 | 3 |
| Coded block size including 12bits trellis termination(bits) | 5292 | 12108 | 12684 |
| Total number of bits per sub-frame | 5280 | 10560 | 15840 |
| Total symbols per sub-frame | 2640 | 2640 | 2640 |

Table 2: FRC for different MCS (option 2: 1 interlace)

| Reference channel | QPSK 1/3 | 16QAM ¾ | 64QAM 5/6 |
| --- | --- | --- | --- |
| Total number of interlaces | 10 | 10 | 10 |
| Allocated number of interlaces | 1 | 1 | 1 |
| DFT-OFDM Symbols per sub frame | 11 | 11 | 11 |
| Modulation | QPSK | 16QAM | 64QAM |
| Code rate | 1/3 | 0.76 | 0.79 |
| MCS index | 5 | 19 | 27 |
| Total symbols per sub-frame | 1320 | 1320 | 1320 |

Note: For the option 1 and option 2 regarding the number of interlace(s), it will be down-selected in the test setup.

# Conclusion

In this paper, simulation assumptions are provided for eLAA PUSCH. Interested companies are encouraged to provide link level simulation based on the provided simulation assumption.

# References

R4-1610706, Way forward on eLAA demodulation performance, Huawei, RAN4#82, Nov. 2016.