3GPP TSG-RAN WG4 Meeting #82 bis	R4-1702738

Spokane, USA, 3 – 7 April, 2017

Source:	Ericsson

Title:	TP to TR 36.714-02-02: Spurious emission requirements

Agenda item:	7.6.2

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.714-02-02 for inclusion of spurious emission requirements for the 2DL/2UL CA combinations listed below. A text proposal for a CR towards TS 36.101 is also included.

CA_2DL_5A-66A_2UL_5A-66A_BCS0
CA_2DL_2A-66A_2UL_2A-66A_BCS0
CA_2DL_12A-30A_2UL_12A-30A_BCS0
CA_2DL_2A-30A_2UL_2A-30A_BCS0
CA_2DL_5A-30A_2UL_5A-30A_BCS0
CA_2DL_30A-66A_2UL_30A-66A_BCS0
CA_2DL_12A-66A_2UL_12A-66A BCS0

# 2. Text Proposal for TR 36.714-02-02

##### ---Start of changes---

Table 5.1-x: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_2A-30A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| CA_2A-66A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 24, 26, 27, 28, 29, 30, 41, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 42, 43 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| CA_5A-30A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 4 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 4 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3, 4 |
| CA_5A-66A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 43, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA band 41, 42 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| CA_12A-30A | E-UTRA Band 2, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_12A-66A | E-UTRA Band 2, 5, 7,13, 14, 17, 22, 24, 25, 26, 27, 30, 41, 43, 48, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 10. 42, 66, 70 | FDL_low | - | FDL_high | -50 | 1 | 1 |
|  | E-UTRA Band 12 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA 30A-66A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 66, 70 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 |
| NOTE 1: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 2: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 4: Applicable only for UE category M1 and NB1 |  |  |  |  |  |  |  |

##### ---End of changes---

# 3. Text Proposal for TS 36.101

##### ---Start of changes---

Table 6.6.3.2A-0: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band |  |  | Frequency range (MHz) |  |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |  |
| CA_1A-3A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 38, 40, 41, 43, 44, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA band 11,18,19, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 10 |  |
|  | E-UTRA band 22, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 7, 10 |  |
|  | Frequency range |  |  | 1880 |  |  |  | 1895 | -40 | 1 | 3,12 |  |
|  | Frequency range |  |  | 1895 |  |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1915 |  |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |  |
| CA_1A-5A | E-UTRA Band 1, 5, 7, 8, 22, 28, 31, 38, 40, 42, 43, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3,34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA band 41 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_1A-7A | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31,32, 40, 42, 43, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 1880 |  |  |  | 1895 | -40 | 1 | 3,12 |  |
|  | Frequency range |  |  | 1895 |  |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1915 |  |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 2570 |  | - |  | 2575 | +1.6 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2575 |  | - |  | 2595 | -15.5 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2620 | -40 | 1 | 3, 14 |  |
| CA_1A-8A | E-UTRA Band 1, 20, 26, 28, 31, 32, 38, 40, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2,3 |  |
|  | E-UTRA band 7, 22, 41, 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 8, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 11 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 | 3, 11 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 7, 11 |  |
|  | Frequency range |  |  | 1880 |  |  |  | 1895 | -40 | 1 | 3,12 |  |
|  | Frequency range |  |  | 1895 |  |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1915 |  |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |  |
| CA_1A-18A | E-UTRA Band 1, 11, 21, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 758 |  | - |  | 799 | -50 | 1 |  |  |
|  | Frequency range |  |  | 799 |  | - |  | 803 | -40 | 1 | 3 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 |  |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 3, 7 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_1A-19A | E-UTRA Band 1, 11, 21, 28, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 | 3, 8 |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 3, 7 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_1A-21A | E-UTRA Band 11 |  |  | FDL_low |  | - |  | FDL_high | -35 | 1 | 3, 16 |  |
|  | E-UTRA Band 1, 18, 19, 28, 34, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 16 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 7 |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_1A-26A | E-UTRA Band 1,5, 7, 11, 18, 19, 20, 21, 22, 26, 27, 31, 38, 40, 42, 43, 44, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 1880 |  | - |  | 1895 | -40 | 1 | 3, 12 |  |
|  | Frequency range |  |  | 1895 |  | - |  | 1915 | -15.5 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1915 |  | - |  | 1920 | +1.6 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 7 |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | E-UTRA Band 41 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 3, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 703 |  | - |  | 799 | -50 | 1 |  |  |
|  |  |  |  | 799 |  | - |  | 803 | -40 | 1 | 3 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 | 3 |  |
| CA_1A-28A | E-UTRA Band 5, 7, 8, 18, 19, 20, 26, 27, 31, 32, 38, 40, 41 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 22, 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 3, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 21 |  |
|  | E-UTRA Band 1, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 6 |  |
|  | Frequency range |  |  | 470 |  | - |  | 694 | -42 | 8 | 3, 22 |  |
|  | Frequency range |  |  | 470 |  | - |  | 710 | -26.2 | 6 | 23 |  |
|  | Frequency range |  |  | 758 |  | - |  | 773 | -32 | 1 | 3 |  |
|  | Frequency range |  |  | 773 |  | - |  | 803 | -50 | 1 |  |  |
|  | Frequency range |  |  | 662 |  | - |  | 694 | -26.2 | 6 | 3 |  |
|  | Frequency range |  |  | 1880 |  |  |  | 1895 | -40 | 1 | 3,12 |  |
|  | Frequency range |  |  | 1895 |  |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1915 |  |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 5, 7 |  |
| CA_1A-42A | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 44, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 1880 |  |  |  | 1895 | -40 | 1 | 3,12 |  |
|  | Frequency range |  |  | 1895 |  |  |  | 1915 | -15.5 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1915 |  |  |  | 1920 | +1.6 | 5 | 3, 12, 13 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 3, 7 |  |
| CA_2A-4A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 24, 26, 27, 28, 29, 30, 41, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_2A-5A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 24, 28, 29, 30, 42, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_2A-7A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 29, 30, 42, 65, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_2A-12A | E-UTRA Band 5, 13, 14, 17, 24, 26, 27, 30, 41 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 12, 25 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 4, 10, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_2A-13A | E-UTRA Band 4, 5,10,12,13,17, 22, 26, 27, 29, 41, 42, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2,14, 25 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 24, 30, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 769 |  | - |  | 775 | -35 | 0.00625 | 3 |  |
|  | Frequency range |  |  | 799 |  | - |  | 805 | -35 | 0.00625 | 3, 9 |  |
| CA_2A-30A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 42, 48, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_2A-66A | E-UTRA Band 4, 5, 10, 12, 13, 14, 17, 22, 24, 26, 27, 28, 29, 30, 41, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 2, 25 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_3A-5A | E-UTRA Band 1, 5, 7, 8, 22, 28, 31, 38, 40, 42, 43, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3,34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
| CA_3A-7A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 40, 43, 44, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA band 22, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 2570 |  | - |  | 2575 | +1.6 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2575 |  | - |  | 2595 | -15.5 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2620 | -40 | 1 | 3, 14 |  |
| CA_3A-8A | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 44, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 8 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2, 3 |  |
|  | E-UTRA band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 10,11 |  |
|  | E-UTRA band 7, 22, 41, 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4, 10, 11 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 | 3,11,17 |  |
| CA_3A-19A | E-UTRA Band 1, 11, 21, 28, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 | 3, 8 |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 3, 4 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_3A-20A | E-UTRA Band 1, 7, 8, 31, 32, 33, 34, 40, 43, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3, 20 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 22, 38, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 758 |  | - |  | 788 | -50 | 1 |  |  |
| CA_3A-21A | E-UTRA Band 1, 18, 19, 28, 34, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |
| CA_3A-26A | E-UTRA Band 1, 5, 7, 26, 34, 39, 40, 43, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA band 11, 18, 19, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 10 |  |
|  | E-UTRA band 22, 41, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4, 10 |  |
|  | Frequency range |  |  | 703 |  | - |  | 799 | -50 | 1 |  |  |
|  |  |  |  | 799 |  | - |  | 803 | -40 | 1 | 3 |  |
|  | Frequency range |  |  | 851 |  | - |  | 859 | -53 | 0.00625 | 15 |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
| CA_3A-28A | E-UTRA Band 1, 22, 42, 43, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 6 |  |
|  | E-UTRA band 3 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 5, 7, 8, 20, 26, 27, 31, 34, 38, 40, 41 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11, 18, 19, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 10 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 10 |  |
|  | Frequency range |  |  | 470 |  | - |  | 710 | -26.2 | 6 | 23 |  |
|  | Frequency range |  |  | 758 |  | - |  | 773 | -32 | 1 | 3 |  |
|  | Frequency range |  |  | 773 |  | - |  | 803 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4, 5 |  |
| CA_3A-42A | E-UTRA Band 1, 5, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 3 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 15 |  |
|  | E-UTRA Band 11, 18, 19, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 13 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 8, 13 |  |
| CA_4A-5A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 43, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA band 41, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_4A-7A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 26, 27, 28, 29, 30, 43, 66 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 2570 |  | - |  | 2575 | +1.6 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2575 |  | - |  | 2595 | -15.5 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2620 | -40 | 1 | 3, 14 |  |
| CA_4A-12A | E-UTRA Band 2, 5, 7,13, 14, 17, 22, 24, 25, 26, 27, 30, 41, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 10. 42, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 12 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
| CA_4A-13A | E-UTRA Band 2,4, 5, 7, 10,12,13,17, 22, 25, 26, 27, 29, 41, 43, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 14 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 24, 30, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 769 |  | - |  | 775 | -35 | 0.00625 | 3 |  |
|  | Frequency range |  |  | 799 |  | - |  | 805 | -35 | 0.00625 | 3, 9 |  |
| CA_4A-17A | E-UTRA Band 2, 5, 7,13, 14, 17, 22, 24, 25, 26, 27, 30, 41, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 10. 42, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 12 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
| CA_5A-7A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 22, 28, 29, 30, 31, 40, 42, 43, 65, 66 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | Frequency range |  |  | 2570 |  | - |  | 2575 | +1.6 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2575 |  | - |  | 2595 | -15.5 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2620 | -40 | 1 | 3, 14 |  |
| CA_5A-12A | E-UTRA Band 2, 5, 13, 14, 17, 22, 24, 25, 30, 31, 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 4, 10, 41, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA band 12 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
| CA_5A-17A | E-UTRA Band 2, 5, 13, 14, 17, 22, 24, 25, 30, 31, 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 4, 10, 41, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA band 12 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
| CA_5A-30A | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA Band 41 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 18, 19 |  |  | FDL_low |  | - |  | FDL_high | -40 | 1 | 24 |  |
|  | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 24 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4, 24 |  |
| CA_5A-66A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 28, 29, 30, 43, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 26 |  |  | 859 |  | - |  | 869 | -27 | 1 |  |  |
|  | E-UTRA band 41, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA_7A-8A | E-UTRA Band 1, 5, 10, 20, 26, 27, 28, 31, 32, 34, 38, 40, 65, 67, 68, 69 |  | FDL_low |  | - |  | FDL_high |  | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43 |  | FDL_low |  | - |  | FDL_high |  | -50 | 1 | 2 |
|  | E-UTRA Band 8 |  | FDL_low |  | - |  | FDL_high |  | -50 | 1 | 3 |
|  | Frequency range |  | 2570 |  | - |  | 2575 |  | +1.6 | 5 | 3, 13, 14 |
|  | Frequency range |  | 2575 |  | - |  | 2595 |  | -15.5 | 5 | 3, 13, 14 |
|  | Frequency range |  | 2595 |  | - |  | 2620 |  | -40 | 1 | 3, 14 |
| CA_7A-20A | E-UTRA Band 1,3, 7, 8, 22, 28, 31, 32, 33, 34, 40, 43, 65, 67 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 20 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
|  | E-UTRA Band 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | Frequency range |  |  | 2570 |  | - |  | 2575 | +1.6 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2575 |  | - |  | 2595 | -15.5 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2620 | -40 | 1 | 3, 14 |  |
| CA_7A-28A | E-UTRA Band 2, 3, 5, 7, 8, 20, 26, 27, 31, 34, 40 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 1, 4, 10, 22, 42, 43, 65, 66 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 6 |  |
|  | Frequency range |  |  | 758 |  | - |  | 773 | -32 | 1 | 3 |  |
|  | Frequency range |  |  | 773 |  | - |  | 803 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2570 |  | - |  | 2575 | +1.6 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2575 |  | - |  | 2595 | -15.5 | 5 | 3, 13, 14 |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2620 | -40 | 1 | 3, 14 |  |
| CA_8A-39A | E-UTRA Band 1, 40, 45 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |
|  | E-UTRA band 22, 41, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |
| CA_8A-41A | E-UTRA Band 1, 28, 34, 39, 40, 45, 65, |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA band 3, 42 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 23, 30 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 8, 23, 30 |  |
| CA_12A-30A | E-UTRA Band 2, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 41, 48, |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 10, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 12 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
| CA_12A-66A | E-UTRA Band 2, 5, 7,13, 14, 17, 22, 24, 25, 26, 27, 30, 41, 43, 48, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 4, 10. 42, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 12 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3 |  |
| CA_18A-28A | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 21 |  |
|  | E-UTRA Band 1, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 6 |  |
|  | E-UTRA Band 42, 43 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 470 |  | - |  | 710 | -26.2 | 6 | 23 |  |
|  | Frequency range |  |  | 758 |  | - |  | 773 | -32 | 1 | 3 |  |
|  | Frequency range |  |  | 773 |  | - |  | 799 | -50 | 1 |  |  |
|  | Frequency range |  |  | 799 |  | - |  | 803 | -40 | 1 | 3 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 |  |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 | 3 |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_19A-21A | E-UTRA Band 1, 18, 19, 28, 34, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 3, 16 |  |
|  | E-UTRA Band 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 16 |  |
|  | Frequency range |  |  | 860 |  | - |  | 890 | -40 | 1 | 3, 8 |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4 |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_19A-42A | E-UTRA Band 1, 11, 21, 28, 34, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_21A-28A | E-UTRA Band 1, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 6 |  |
|  | E-UTRA Band 18, 19, 34 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 470 |  | - |  | 710 | -26.2 | 6 | 23 |  |
|  | Frequency range |  |  | 773 |  | - |  | 803 | -50 | 1 |  |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4, 5 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_21A-42A | E-UTRA Band 1, 18, 19, 28, 34, 42, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 945 |  | - |  | 960 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1839.9 |  | - |  | 1879.9 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4 |  |
|  | Frequency range |  |  | 2545 |  | - |  | 2575 | -50 | 1 |  |  |
|  | Frequency range |  |  | 2595 |  | - |  | 2645 | -50 | 1 |  |  |
| CA_28A-42A | E-UTRA Band 1, 4, 10, 65 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
|  | E-UTRA Band 1 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 6 |  |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 66 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 11, 21 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 5, 21 |  |
|  | Frequency range |  |  | 470 |  | - |  | 710 | -26.2 | 6 | 23 |  |
|  | Frequency range |  |  | 758 |  | - |  | 773 | -32 | 1 | 3 |  |
|  | Frequency range |  |  | 773 |  | - |  | 803 | -50 | 1 |  |  |
|  | Frequency range |  |  | 1884.5 |  | - |  | 1915.7 | -41 | 0.3 | 4, 5 |  |
| CA 30A-66A | E-UTRA Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 66, 70 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 42, 48 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 | 2 |  |
| CA 39A-41A | E-UTRA Band 1, 8, 26, 34, 40, 42, 44 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 1805 |  | - |  | 1855 | -40 | 1 | 20 |  |
|  | Frequency range |  |  | 1855 |  | - |  | 1880 | -15.5 | 5 | 3, 13, 20 |  |
| CA_39A-41C | E-UTRA Band 1, 8, 26, 34, 40, 42, 44 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
|  | Frequency range |  |  | 1805 |  | - |  | 1855 | -40 | 1 | 20 |  |
|  | Frequency range |  |  | 1855 |  | - |  | 1880 | -15.5 | 5 | 3, 13, 20 |  |
| CA_39C-41A | E-UTRA Band 34, 40, 42, 44 |  |  | FDL_low |  | - |  | FDL_high | -50 | 1 |  |  |
| CA_41A-42A | E-UTRA Band 1, 3, 5, 8, 26, 28, 33, 34, 39, 40, 44, 45, 65, | FDL_low |  |  |  | - |  | FDL_high | -50 | 1 |  |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low |  |  |  | - |  | FDL_high | -50 | 1 | 18 |  |
|  | Frequency range | 1839.9 |  |  |  |  |  | 1879.9 | -50 | 1 | 18 |  |
|  | Frequency range | 1884.5 |  |  |  |  |  | 1915.7 | -41 | 0.3 | 4, 18 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 7: Applicable when NS_05 in section 6.6.3.3.1 is signalled by the network. NOTE 8: Applicable when NS_08 in subclause 6.6.3.3.3 is signalled by the network NOTE 9: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD. NOTE10: This requirement applies for 5, 10, 15 and 20 MHz E-UTRA channel bandwidth allocated within 1744.9MHz and 1784.9MHz. NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE13: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 14: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 15: Applicable when NS_15 in subclause 6.6.3.3.8 is signalled by the network. NOTE 16: Applicable when NS_09 in subclause 6.6.3.3.4 is signalled by the network NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19: Void NOTE 20: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 21: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 24: Applicable only for UE category M1 and NB1 |  |  |  |  |  |  |  |  |  |  |  |  |

##### ---End of changes---