3GPP TSG-RAN4 Meeting #82bis	R4-1702876

# Spokane, United States, 3rd Apr 2017 - 7th Apr 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 4624 | rev | - | Current version: | 13.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NB-IoT RRM: Correction to used RMC and NOCNG (Rel-13) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  | Date: |  |  | 2017-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | NB-IoT Stand alone TC A.7.2.9 is refering to non-existing NB-IoT RMCs (NPDSCH R.20 FD-FDD, NPDCCH R.30 HD-FDD) andvoid clauses (A.3.1.5.4, A.3.1.6.3) |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TC A.7.2.9, Table A.7.2.7.1-2 following corrections done: - NPDSCH: R.20 FD-FDD changed to R.80 FD-FDD - Respective reference A.3.1.5.4 changed to A.3.1.5.3 - NPDCCH: R.32 FD-FDD changed to R.30 FD-FDD - Respective reference A.3.1.6.4 changed to A.3.1.6.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases non implementable since RMCs not existing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.7.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.3.1.5	NPDSCH Reference Channel for UE category NB1

#### A.3.1.5.1	HD-FDD in-band operation

Table A.3.1.5.1-1: NPDSCH Reference Channel for UE category NB1 for in-band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.14 HD-FDD | R.15 HD-FDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 40Note 2 | 40Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1Note 4 | 1Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| NPDCCH start symbol | symbols | 3 | 3 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling. Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 40 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: Only apply for subframes scheduled with NPDSCH. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.5.2	Void

#### A.3.1.5.3	HD-FDD standalone operation

Table A.3.1.5.3-1: NPDSCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.18 HD-FDD | R.19 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72Note 2 | 72Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1Note 4 | 1Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling. Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: only apply for subframes scheduled with NPDSCH.. Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.5.4	Void

#### A.3.1.5.5	HD-FDD guard band operation

Table A.3.1.5.5-1: NPDSCH Reference Channel for UE category NB1 for guard band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDSCH Reference channel | - | R.22 HD-FDD | R.23 HD-FDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks for NB-IoT | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| Allocated subframes per Radio Frame | subframes | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| Target coding rate |  | 1/3 | 1/3 |
| Information Bit Payload |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 | Bits | 72Note 2 | 72Note 2 |
| For Sub-Frame 4, 9 | Bits | Note 3 | Note 3 |
| For Sub-Frame 0, 5 | Bits | 0 | 0 |
| Number of Code Blocks per Sub-Frame |  |  |  |
| For Sub-Frame 1, 2, 3, 6, 7, 8 |  | 1Note 4 | 1Note 4 |
| For Sub-Frame 4, 9 |  | Note 5 | Note 5 |
| For Sub-Frame 0, 5 |  | 0 | 0 |
| Maximum number of repetitions | - | Note 6 | Note 6 |
| Cell ID | - | Note 7 | Note 7 |
| Note 1: Shall depend upon the NPDSCH scheduling. Note 2: Only apply for subframes scheduled with NPDSCH. Note 3: 72 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 4: Only apply for subframes scheduled with NPDSCH.Note 5: 1 for subframes scheduled with NPDSCH when mod 2 ≠ 0. Otherwise 0. Note 6: Maximum number of repetitions shall depend upon the test configuration. Note 7: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.5.6	Void

### A.3.1.6	NPDCCH Reference Channel for UE category NB1

#### A.3.1.6.1	HD-FDD in-band operation

Table A.3.1.6.1-1: NPDCCH Reference Channel for UE category NB1 for in-band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.26 HD-FDD | R.27 HD-FDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks Note1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | N1 | N1 |
| Aggregation level | NCCE | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 |
| NPDCCH start symbol | symbols | 3 | 3 |
| Payload (without CRC) | Bits | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Maximum number of repetitions shall depend upon the test configuration. Note 3: Payload size shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.6.2	Void

#### A.3.1.6.3	HD-FDD standalone operation

Table A.3.1.6.3-1: NPDCCH Reference Channel for UE category NB1 for standalone operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.30 HD-FDD | R.31 HD-FDD |
| Channel bandwidth | KHz | 200 | 200 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | N1 | N1 |
| Aggregation level | NCCE | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 |
| Payload (without CRC) | Bits | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Maximum number of repetitions shall depend upon the test configuration. Note 3: Payload size shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.6.4	Void

#### A.3.1.6.5	HD-FDD guard band operation

Table A.3.1.6.5-1: NPDCCH Reference Channel for UE category NB1 for guard band operation

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| NPDCCH Reference channel | - | R.34 HD-FDD | R.35 HD-FDD |
| LTE Carrier bandwidth | MHz | 10 | 10 |
| Allocated resource blocks for NB-ioTNote1 | PRB | 1 | 1 |
| Number of transmitter antennas | - | 1 | 2 |
| DCI Format | - | N1 | N1 |
| Aggregation level | NCCE | 2 | 2 |
| Maximum number of repetitions | - | Note 2 | Note 2 |
| Payload (without CRC) | Bits | Note 3 | Note 3 |
| Cell ID | - | Note 4 | Note 4 |
| Note 1: Allocation is located in the middle of narrowband. Note 2: Maximum number of repetitions shall depend upon the test configuration. Note 3: Payload size shall depend upon the test configuration. Note 4: Cell ID shall depend upon the test configuration. |  |  |  |

#### A.3.1.6.6	Void

< Unchanged sections omitted >

### A.3.2.3	OCNG Patterns for Narrowband IoT

The following Narrowband OCNG patterns (NOCNG) are used for modelling allocations to UEs not under test in a Narrowband IoT cell. Depending on scenario, allocations may be for UEs of category NB1only, or for UEs of category NB1 as well as of other categories. The former is applicable to guard-band and stand-alone deployments of Narrowband IoT, whereas the latter is applicable to in-band deployment. In order to allow different power levels for the LTE cell and the Narrowband IoT cell, a distinction is made between OCNG and NOCNG where the latter is used for category NB1 UEs and the former is used for other UE categories.

OCNG in the LTE cell is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the PDSCH-to-RS EPRE ratio in OFDM symbols with and without LTE cell-specific reference symbols, respectively. The relative power, which is used for modelling boosting per virtual LTE UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual LTE UE.

Moreover in each test case NOCNG is expressed by parameters NOCNG_RA and NOCNG_RB which together with a relative power level () specifies the <channel>-to-RS EPRE ratio in OFDM symbols with and without Narrowband reference symbols (NB-RS), respectively. The relative power, which is used for modelling boosting per virtual UE category NB1 allocation, is expressed by:

where  denotes the relative power level of the k:th virtual NB-IoT UE, and channel may be either of NPDCCH and NPDSCH.

The parameter settings of OCNG_RA, OCNG_RB, NOCNG_RA, NOCNG_RB and the set of relative power levels are chosen such that when also taking allocations to the UE category NB1 under test into account, as given by a NPDCCH and NPDSCH reference channels, a transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

#### A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell

Table A.3.2.3.1-1: NOP.1 FDD: OCNG FDD Pattern 1

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |  |
| 0 – 29, 35 – 49 | 0 | 0 | 0 | Note 1 | N/A |
| 30 – 34 | 0 (Note 3) | 0 (Note 3) | N/A | N/A | Note 2 |
| 30 – 34 | N/A | N/A | 0 | Note 1 | N/A |
| Note 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS, and NPBCH in anchor cell (= 30). Note 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. N/A: Not Applicable |  |  |  |  |  |

#### A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell

Table A.3.2.3.2-1: NOP.2 FDD: OCNG FDD Pattern 2

| Allocation | Relative power level [dB] |  |  | PDSCH Data | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |  |
| 0 – 49 | 0 | 0 | 0 | Note 1 | N/A |
| 50-52 | 0 (Note 3) | 0 (Note 3) | 0 | N/A | Note 2 |
| Note 1: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of PDSCH. If two or more transmit antennas with CRS are used in the test, the PDSCH part of OCNG shall be transmitted to the virtual users by all the transmit antennas with CRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the PDSCH part of OCNG is equal between all the transmit antennas with CRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 3: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and PBCH in anchor cell (= 50). Note 4: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over OCNG and NOCNG. N/A: Not Applicable |  |  |  |  |  |

#### A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT

Table A.3.2.3.3-1: NOP.3 FDD: OCNG FDD Pattern 3

| Allocation | Relative power level [dB] |  |  | NPDCCH and NPDSCH Data |
| --- | --- | --- | --- | --- |
|  | Subframe |  |  |  |
|  | 0, 4 | 5, 9 | 1-3, 6-8 |  |
| 0 – 5 | 0 (Note 1) | 0 (Note 1) | 0 | Note 2 |
| Note 1: This physical resource block is assigned to a virtual UE for transmission of NPDCCH or NPDSCH; the data transmitted over the NOCNG NPDCCH or NPDSCH shall be uncorrelated QPSK modulated pseudo random data. The parameter is used to scale the power of NPDCCH and NPDSCH. If two transmit antennas with NBRS are used in the test, the NPDCCH and NPDSCH parts of NOCNG shall be transmitted to the virtual users by both transmit antennas with NBRS and according to the antenna transmission mode 2. The parameter applies to each antenna port separately, so the transmit power of the NPDCCH and NPDSCH parts of NOCNG is equal between both the transmit antennas with NBRS used in the test. The antenna transmission modes are specified in clause 7.1 in TS 36.213. Note 2: Value of is applicable to PRBs not used for transmission of NPSS, NSSS and NPBCH in anchor cell (= 0). Note 3: SI transmissions, and NPDCCH and NPDSCH for the UE under test, have precedence over NOCNG. |  |  |  |  |

< Unchanged sections omitted >

### A.7.2.9	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage

#### A.7.2.9.1	Test Purpose and Environment

The purpose of the test is to verify E-UTRAN Timing Advance adjustment accuracy requirements for UE category NB1 in enhanced coverage, defined in clause 7.22.2.2, in an AWGN model.

The test parameters are given in tables A.7.2.9.1-1 and A.7.2.9.1-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. In each time period, timing advance commands are sent to the UE and the UE is scheduled in every uplink subframe to transmit NPUSCH, which is received by the test equipment. By measuring the reception of the NPUSCH, the transmit timing, and hence the timing advance adjustment accuracy, can be measured.

During time period T1, the test equipment shall send one message with a Timing Advance Command MAC Control Element, as specified in Clause 6.1.3.5 in TS 36.321. The Timing Advance Command value shall be set to 31, which according to Clause 16.1.2 in TS 36.213 results in zero adjustment of the Timing Advance. In this way, a reference value for the timing advance used by the UE is established.

During time period T2, the test equipment shall send a sequence of messages with Timing Advance Command MAC Control Elements, with Timing Advance Command value specified in table A.7.2.9.1-2. This value shall result in changes of the timing advance used by the UE, and the accuracy of the change shall then be measured, using the NPUSCH sent from the UE.

As specified in Clause 7.22.2.1, the UE adjusts its uplink timing at sub-frame n+12 for a timing advance command received in sub-frame n, where sub-frame n refers to the last subframe in the repetition period in which the MAC control element containing timing advance command was received. In addition, the UE shall not apply a TA command during an uplink repetition period. In the test the NPUSCH repetition period is equal to 32 subframes. This delay must be taken into account when measuring the timing advance adjustment accuracy, via the NPUSCH sent from the UE.

The UE Time Alignment Timer, described in Clause 5.2 in TS 36.321, shall be configured so that it does not expire in the duration of the test.

Table A.7.2.9.1-1: General Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| NB-IoT operational mode |  |  | Standalone |  |
| CP Length |  |  | Normal |  |
| Timing Advance Command (TA) value during T1 |  |  | 31 | NTA = 0 for the purpose of establishing a reference value from which the timing advance adjustment accuracy can be measured during T2 |
| Timing Advance Command (TA) value during T2 |  |  | 39 | NTA = 128 |
| Number of repetitons | NPDCCH |  | 128 |  |
|  | NPUSCH |  | 32 |  |
| DRX |  |  | OFF |  |
| T1 |  | s | 5 |  |
| T2 |  | s | 5 |  |

Table A.7.2.7.1-2: Cell specific Test Parameters for E-UTRAN Timing Advance Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel Number |  | 1 |  |
| BWchannel | KHz | 200 |  |
| NPDSCH parameters: DL Reference Measurement Channel defined in A.3.1.5.3 |  | R.18 HD-FDD |  |
| NPDCCH parameters: DL Reference Measurement Channel defined in A.3.1.6.3 |  | R.30 HD-FDD |  |
| NOCNG Patterns defined in A.3.2.3.3 |  | NOP.3 FDD |  |
| NPBCH_RA | dB | 0 |  |
| NPBCH_RB | dB |  |  |
| NPSS_RA | dB |  |  |
| NSSS_RA | dB |  |  |
| NPDCCH_RA | dB |  |  |
| NPDCCH_RB | dB |  |  |
| NPDSCH_RA | dB |  |  |
| NPDSCH_RB | dB |  |  |
| NOCNG_RANote1 | dB |  |  |
| NOCNG_RBNote1 | dB |  |  |
| Timing Advance Command (TA) |  | 31 | 39 |
|  | dB | -12 |  |
|  | dBm/15 KHz | -88 |  |
|  | dB | -12 |  |
| IoNote2 | dBm/ 180 KHz | -76.9 |  |
| Antenna Configuration |  | 1x1 |  |
| Propagation Condition |  | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Io level has been derived from other parameters for information purpose. It is not a settable parameter. |  |  |  |

#### A.7.2.9.2	Test Requirements

The UE shall apply the signalled Timing Advance value to the transmission timing at subframe n+40, where subframe n is the last subframe in the repetition period of NPDCCH in which the timing advance command is received by the UE.

The Timing Advance adjustment accuracy shall be within the limits specified in clause 7.22.2.2.

The rate of correct Timing Advance adjustments observed during repeated tests shall be at least 90%.

NOTE:	In the test requirement the timing advance adjustment delay of 40 subfames include:

-	NPUSCH repetition period equal to 32 subframes and

-	Uplink delay of 8 subframes. This is the time between the last subframe in the repetition period of NPDCCH and the first subframe in the repetition period of the corresponding NPUSCH.

< End of changes >