3GPP TSG-RAN WG4 Meeting #82bis	R4-1703007

# Spokane, WA, USA, 3rd-7th April, 2017

Title:	Reply LS to RAN1 on possible RF impacts related to sPUSCH design

Release:	Release 15

Work Item:	LTE_sTTIandPT

Source:	RAN4

To:	RAN1

Contact Person:

#### Name:	Dominique Everaere

E-mail Address:	dominique.everaere@ericsson.com

Attachments: None

- Description:

RAN4 would like to thank RAN1 for the questions in R1-1613334 regarding the possible RF impacts related to sPUSCH design.

After discussing the issue in RAN4, we have the following answers to the questions:

Q1: If there is any RF impacts that would degrade sPUSCH demodulation performance when having shared DMRS symbols in between 2 sTTIs but using different output power levels?

A1:   To guaranty performance, both sTTIs should have same ouput power when sharing DMRS symbols.

Q1: If there is any RF impacts that would degrade sPUSCH demodulation performance when having shared DMRS symbols in between 2 sTTIs but using differentbadnwidth allocations?

A2: To guaranty performance, both sTTIs should have same bandwidth allocation when sharing DMRS symbols.

Q1: If there is any RF impacts that would degrade sPUSCH demodulation performance when allocating DMRS symbol(s) non-contiguously in time?

A3:   DMRS symbol could be shared in between 2 non-contiguous (in time) sTTI if the gap is equal to up to 2 sTTIs. There should not be any power change, neither any bandwidth allocation change in between such non-contiguous sTTIs. When such non-contiguous sTTI would be scheduled, BS should indicate UE to keep its PLL ON to limit power consumption impact.

- Actions:

RAN4 asks RAN1 to consider above answers in their evaluation.

- Date of Next TSG-RAN4 Meetings:

TSG-RAN4 Meeting#83		May 15-19				Hangzhou, China

TSG-RAN4 Meeting#84		August 21-25			Berlin, Germany