3GPP TSG-RAN WG4 Meeting #82bis 	R4-1703296

# Spokane, USA, 3th – 7th April 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4328 | rev | - | Current version: | 14.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce LBT model for multile LAA Scell(s) in LAA demodualtion |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_LAA-Perf |  |  |  |  |  |  | Date: |  | 2017-03-24 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The group agreed to extend performance requirements of one LAA Scell into multiple LAA Scell(s). Currently, the LBT model can be applied only for single LAA Scell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LBT model for multiple LAA Scell(s) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There would have no performance requirements for multiple LAA Scell(s) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | B.8.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------------------Beginning of Change ----------------------------------------------------------

## B.8.2	Burst transmission model for multiple LAA SCell(s)

This clause provides a description for burst transmission models for Frame structure type 3 when there are multiple LAA Scell(s) in the test.

One burst is defined as downlink transmissions which occupy one or more consecutive subframes. Assuming M carriers are configured, the burst transmission format is determined according to the steps below:

1)	For each carrier cm(m=0,⋯,M-1), select the number of subframes Nm randomly from a given set of the number of subframes  with equal probability as the total length of burst transmission format used for carrier cm. The length includes both occupied OFDM symbols and non-occupied OFDM symbols within the burst format.   is given per test case.

2)	If any Nm is equal to 1, the first subframe is set as fully occupied for all carriers, otherwise:

-	For demodulation test, the starting position for the first subframe is randomly selected from OFDM symbol 0 and OFDM symbol 7 with equal probability. For CSI test, the starting position for the first subframe is OFDM symbol 0. The starting position is common for all carriers.

-	The configuration of occupied OFDM symbols in the last subframe is randomly selected from configuration set  for each carrier cm. is given per test case.

A uniform random variable from [0, 1] is generated for each carrier cm. If the random variable is less than p which is given per test case,

-	If both the last subframe of previous longest burst over M carriers and first subframe of new burst format are fully occupied, start burst transmission according to the determined burst transmission format for each carrier cm after deferring one subframe from the last subframe of previous longest burst. Otherwise, start burst transmission for each carrier  cm  at the end of last subframe of previous longest burst.

Otherwise, the burst transmission is muted and the muting duration is Nmax, wherein Nmax=max⁡(N0, N1,⋯,NM-1)

---------------------------------------------------------------End of Change---------------------------------------------------------------