3GPP TSG-RAN WG4 Meeting #82bis                                                           R4-1703424

Spokane, USA, 3rd-7th April 2017

Agenda Item:	7.25.4.2

Source: 		Ericsson

Title: 	Discussion and simulation results for HST unidirectional performance test

Document for:	Discussion

# Introduction

In RAN4#80bis, the specification of the unidirectional deployment was discussed based on a proposed WF [1] from Ericsson, Vodafone, Telecom Italia and NTT Docomo. In the WF it was discussed how to introduce a testcase where it is shown that LTE supports higher speed than 350 km/h.

Since bidirectional only supports speed up to 350 km/h it is proposed to introduce a testcase with unidirectional deployment for higher speed. However, since also legacy UEs supports very high speed with unidirectional deployment, it is not critical to test the performance of the UEs.

Based on this it was proposed to introduce the unidirectional scenario and a testcase which can be an alternative to an existing testcase.

# Discussions

Testcase #4 in 8.2.1.1.1 is simulated both as the HST scenario as specified in the testcase and with the HST scenario with unidirectional deployment at 1250 Hz (500 km/h).

Here it is seen that the performance of the legacy HST scenario and with the HST unidirectional deployment at 70% of the max throughput is within 0.4 dB.

The Unidirectional deployment is here simulated at 1000 m distance between the radioheads and 300 m from the rail to the radiohead. The direction of the mainlobe of the antennas in the RRH are 45 degrees from the direction of the railway.

Based on this it is proposed to add a new testcase which is an alternative testcase to test#4 in 8.2.1.1.1.

Table 8.2.1.1.1-2: Minimum performance (FRC)

| Test num. | Band-width | Reference channel | OCNG pattern | Propa-gation condi-tion | Correlation matrix and antenna config. | Reference value |  | UE cate gory |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |  |
| 1 | 10 MHz | R.2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -1.0 | ≥1 |
| 2 | 10 MHz | R.2 FDD | OP.1 FDD | ETU70 | 1x2 Low | 70 | -0.4 | ≥1 |
| 3 | 10 MHz | R.2 FDD | OP.1 FDD | ETU300 | 1x2 Low | 70 | 0.0 | ≥1 |
| 4 | 10 MHz | R.2 FDD | OP.1 FDD | HST | 1x2 | 70 | -2.4 | ≥1 |
| 5 | 1.4 MHz | R.4 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 0.0 | ≥1 |
| 6 | 10 MHz | R.3 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 6.7 | ≥2 |
|  | 5 MHz | R.3-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 6.7 | 1 |
|  | 5 MHz (NOTE 4) | R.3-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 6.7 | ≥2 |
| 7 | 10 MHz | R.3 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.4 | ≥2 |
|  | 5 MHz | R.3-1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.4 | 1 |
|  | 5 MHz (NOTE 4) | R.3-1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.4 | ≥2 |
| 8 | 10 MHz | R.3 FDD | OP.1 FDD | ETU300 | 1x2 High | 70 | 9.4 | ≥2 |
|  | 5 MHz | R.3-1 FDD | OP.1 FDD | ETU300 | 1x2 High | 70 | 9.4 | 1 |
|  | 5 MHz (NOTE 4) | R.3-1 FDD | OP.1 FDD | ETU300 | 1x2 High | 70 | 9.4 | ≥2 |
| 9 | 3 MHz | R.5 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥1 |
| 10 | 5 MHz | R.6 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.4 | ≥2 |
|  | 5 MHz | R.6-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.5 | 1 |
| 11 | 10 MHz | R.7 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.7 | ≥2 |
|  | 10 MHz | R.7-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 16.7 | 1 |
| 12 | 10 MHz | R.7 FDD | OP.1 FDD | ETU70 | 1x2 Low | 70 | 19.0 | ≥2 |
|  | 10 MHz | R.7-1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 70 | 18.1 | 1 |
| 13 | 10 MHz | R.7 FDD | OP.1 FDD | EVA5 | 1x2 High | 70 | 19.1 | ≥2 |
|  | 10 MHz | R.7-1 FDD | OP.1 FDD | EVA5 | 1x2 High | 70 | 17.8 | 1 |
| 14 | 15 MHz | R.8 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.7 | ≥2 |
|  | 15 MHz | R.8-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 16.8 | 1 |
| 15 | 20 MHz | R.9 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.6 | ≥3 |
|  | 20 MHz | R.9-2 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 17.3 | 2 |
|  | 20 MHz | R.9-1 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | 16.7 | 1 |
| 16 | 3 MHz | R.0 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.9 | ≥1 |
| 17 | 10 MHz | R.1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.9 | ≥1 |
| 18 | 20 MHz | R.1 FDD | OP.1 FDD | ETU70 | 1x2 Low | 30 | 1.9 | ≥1 |
| 19 | 10 MHz | R.41 FDD | OP.1 FDD | EVA5 | 1x2 Low | 70 | -5.4 | ≥1 |
| NOTE 1: Void. NOTE 2: Void. NOTE 3: Void. NOTE 4: Test case applicability is defined in 8.1.2.1. |  |  |  |  |  |  |  |  |

Figure 1: Simulated FDD results for Unidirectional deployment with the scenario in testcase 4 of section 8.2.1.1.1 together with the corresponding HST simulation

Figure 2 : Simulated TDD results for Unidirectional deployment with the scenario in testcase 4 of section 8.2.2.1.1 together with the corresponding HST simulation

In Figure 1 and Figure 2 it is seen that the performance of the legacy HST test and the corresponding test for HST unidirectional deployment at 70% of the max throughput is within 0.4 dB.

Observation 1: The performance of the legacy HST performance results and the new unidirectional HST1250 performance are algned with 0.4 dB difference.

Proposal 1: Introduce an alternative testcase to 8.2.1.1.1 and 8.2.2.1.1 testcase 4 so that the UE vendor can select whether the old HST propagation condition shall be used or a new HST1250 testcase showing it supports 500 km/h.

# Conclusions

Observation 1: The performance of the legacy HST performance results and the new unidirectional HST1250 performance are algned with 0.4 dB difference.

Proposal 1: Introduce an alternative testcase to 8.2.1.1.1 and 8.2.2.1.1 testcase 4 so that the UE vendor can select whether the old HST propagation condition shall be used or a new HST1250 testcase showing it supports 500 km/h.

# References

[1] R4-168663 “ Way forward on unidirectional RRH arrangement”, Ericsson, Vodafone, Telecom Italia, 
NTT DoCoMo